3GPP TSG-RAN WG3 Meeting #104	R3-192976

# Reno, U.S.A, 13th – 17th May 2019

Agenda Item:	9.2

Source:	Ericsson

Title: 	NB-IoT Correction: inconsistent between tabular and ASN.1

Document for:	Discussion and Approval

# 1	Discussion

The CP relocation messages have inconsistence between the tabular and the ASN.1.

In tabular, it is defined the Message Type with Assigned criticality “ignore”.

#### 9.1.4.23        eNB CP RELOCATION INDICATION

This message is sent by the eNB to initiate the establishment of a UE-associated logical S1-connection, following the reception of re-establishment request as described in TS. 36.300 [14].

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| S-TMSI | M |  | 9.2.3.6 |  | YES | reject |
| E-UTRAN CGI | M |  | 9.2.1.38 |  | YES | ignore |
| TAI | M |  | 9.2.3.16 |  | YES | ignore |
| UL CP Security Information | M |  | 9.2.3.50 |  | YES | reject |

#### 9.1.4.24        MME CP RELOCATION INDICATION

This message is sent by the MME to inform the eNB that the UE is to be relocated as described in TS. 36.300 [14].

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |

But in the ASN.1 it is defined as “reject”.

eNBCPRelocationIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE      ENBCPRelocationIndication

PROCEDURE CODE          id-eNBCPRelocationIndication

CRITICALITY             reject

}

mMECPRelocationIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE      MMECPRelocationIndication

PROCEDURE CODE          id-MMECPRelocationIndication

CRITICALITY             reject

In order to make a correction without change the ASN.1, we propose to align the tabular with the ASN.1.

Proposal 1: RAN3 to agree to align the tabular with the ASN.1, so that no backwards compatible changes are introduced.

# 2	Proposal

Proposal 1: RAN3 to agree to align the tabular with the ASN.1, so that no backwards compatible changes are introduced.

The corrections are submitted for Rel 14 and Rel 15.