# 3GPP TSG-RAN WG3 #104	Tdoc R3-192987

Reno, Nevada, US, 13th – 17th May

Agenda Item:	09.3.15

Source:	Ericsson

Title:	gNB-CU-UP Graceful shutdown discussion

Document for:	Discussion on CR R3-R3-192988

# Introduction

In this contribution, we propose to enable graceful shutdown notification of the gNB-CU-UP using existing signalling capabilities and in particular by setting the gNB-CU-UP Capacity value in the gNB-CU-UP Capacity IE to “0”.

# Discussion

The operator may decide to take a gNB-CU-UP out of service e.g., because of maintenance in case of failures, software/hardware upgrades, or for updating the node configuration (e.g., adding new supported slices, PLMNs or served cells). Furthermore, in virtualized environments the gNB-CU-UPs can dynamically be instantiated/released based on traffic load. For gNB-CU-UPs that are about to be taken out of service, it would be beneficial to notify the gNB-CU-CP in advance. In this way, the gNB-CU-CP can avoid allocating new resources e.g., setting up new bearer contexts within these gNB-CU-UPs. This allows to shut down the gNB-CU-UP gracefully, without the risk of affecting connected UEs.

A possible solution to achieve this, is by using the gNB-CU-UP Capacity value in the gNB-CU-UP Capacity IE. By setting the gNB-CU-UP Capacity value of the gNB-CU-UP Capacity IE in the GNB-CU-UP CONFIGURATION UPDATE message to “0”, the gNB-CU-UP informs the gNB-CU-CP that it is going to be taken out of service soon. The gNB-CU-CP should then avoid setting up new bearer contexts in the gNB-CU-UP, while existing bearer contexts may be kept and handled according to normal procedures.

Note that a similar solution was defined by SA2 for graceful shutdown of an AMF (see TS 23.501 [1] and TS 23.502 [2]). As explained in these documents, a solution for graceful shutdown is not only very important for a planned removal of existing resources like a graceful AMF shutdown but is especially important when considering virtualized deployments. In this type of deployments, a Network Function instance can be deployed as fully distributed, fully redundant, and stateless, and correspondingly support for graceful addition or removal of NF instances, e.g. gNB-CU-UPs, for redundancy and scalability would be essential. The graceful shutdown method used by a gNB-CU-UP on the other hand cannot be the same as the one used for an AMF as in contrast to the AMF case the gNB-CU-UP is lacking access to the same level of information (TNLA weight factor).

Proposal 1: It is proposed that setting the gNB-CU-UP Capacity value in the GNB-CU-UP CONFIGURATION UPDATE message to “0” is used to indicate that the gNB-CU-UP is going to be taken out-of-service.

Proposal 2: RAN3 is respectfully asked to agree to this proposal as detailed in the associated CR in R3-192988.

# Conclusion

In this contribution, we discussed how to enable graceful shutdown of the gNB-CU-UP.

Proposal 1: It is proposed that setting the gNB-CU-UP Capacity value in the GNB-CU-UP CONFIGURATION UPDATE message to “0” is used to indicate that the gNB-CU-UP is going to be taken out-of-service.

Proposal 2: RAN3 is respectfully asked to agree to this proposal as detailed in the associated CR in R3-192988.

# References

- 3GPP TS 23.501: "System Architecture for the 5G System; Stage 2"

- 3GPP TS 23.502: " Procedures for the 5G System; Stage 2"