3GPP TSG-RAN WG3 Meeting #104	R3-192989

# Reno, U.S.A, 13th – 17th May 2019

Agenda Item:	9.3.16

Source:	Ericsson

Title: 	PDU Session NAS-PDU in PDU Session Resource Release Command

Document for:	Discussion and Approval

# 1	Discussion

In TS 24.501, the “Network-requested PDU session release procedure initiation”, the SMF should be able to send the 5GSM message PDU SESSION RELEASE COMMAND message to the UE.

This 5GSM message is per PDU Session.

Figure 6.3.3.2.1: Network-requested PDU session release procedure

The 5GSM message is defined as in below:

### 8.3.14 PDU session release command #### 8.3.14.1 Message definition The PDU SESSION RELEASE COMMAND message is sent by the SMF to the UE to indicate a release of a PDU session. See table 8.3.14.1.1. Message type: PDU SESSION RELEASE COMMAND Significance: dual Direction: network to UE Table 8.3.14.1.1: PDU SESSION RELEASE COMMAND message content IEI; Information Element; Type/Reference; Presence; Format; Length. E xtended protocol discriminator; Extended protocol discriminator 9.2; M; V; 1. PDU session ID; PDU session identity 9.4; M; V; 1. PTI; Procedure transaction identity 9.6; M; V; 1. PDU SESSION RELEASE COMMAND message identity; Message type 9.7; M; V; 1. 5GSM cause; 5GSM cause 9.11.4.2; M; V; 1. 37; Back-off timer value; GPRS timer 3 9.11.2.5; O; TLV; 3. 78; EAP message; EAP message 9.11.2.2; O; TLV-E; 7-1503. 7B; Extended protocol configuration options; Extended protocol configuration options 9.11.4.6; O; TLV-E; 4-65538.

In the current NGAP message, PDU SESSION RESOURCE RELEASE COMMAND, it is not possible to send the 5GSM to NG-RAN node, so that the NG-RAN node could pass to the UE.

Proposal 1: RAN3 to agree to introduce the PDU Session NAS-PDU in the PDU Session Resource Release Command

# 2	Proposal

Proposal 1: RAN3 to agree to introduce the PDU Session NAS-PDU in the PDU Session Resource Release Command.

The CR to implement the change is submitted.