# 3GPP TSG-RAN WG3 #104	Tdoc R3-192999

Reno, Nevada, US, 13th – 17th May

Agenda Item:	09.3.19

Source:	Ericsson

Title:	About the possibility that the full configuration IE is included in the UE Context Modification Response.

Document for:	Discussion, Decision

# Introduction

Currently the gNB-DU can inform the gNB-CU in the UE CONTEXT SETUP RESPONSE message when “full configuration” shall be applied. However, we claim that this information should also be included in the UE CONTEXT MODIFICATION RESPONSE message. In this contribution we present our arguments and make our proposal.

# Discussion

In the contribution R3-192888 we claim that there are cases when the gNB-CU needs to inform the gNB-DU that it needs to apply full configuration. For that we propose in R3-192846 to add a Full Configuration IE in the UE CONTEXT SETUP REQUEST message and in the UE CONTEXT MODIFICATION REQUEST message.

It is expected then that when sending Full Configuration IE in the UE CONTEXT SETUP REQUEST message the gNB-DU would also include the Full Configuration IE in the UE CONTEXT SETUP RESPONSE message.

We note though that there does not exist a Full Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message. Even without considering the proposed changes in R3-192846, this lack obviously makes the two procedures asymmetrical. Furthermore, it removes the possibility for the gNB-DU to trigger full configuration in cases of intra-gNB intra-DU handover.

Considering specifically intra-gNB intra-DU handover scenarios triggered via F1 UE Context Modification procedure, we believe that support for full configuration also in the UE Context Modification procedure instead of only when change of gNB-DU occurs, would be beneficial. That is because it could be useful in some cases to have a reconfiguration with synch at intra gNB-DU mobility. For example, if numerous configurations have been applied to the UE while the UE context is active at a gNB-DU, it is useful to have a procedure that ensures that UE and RAN are in synch with the full set of configuration parameters. Also, in cases the gNB-DU is implemented in a virtualised platform and intra gNB-DU mobility may mean to handover the UE to a different module of the virtual platform, such procedures could be useful.

Based on that we propose to add a Full Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message.

Proposal 1: 	Add Full Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message.

# Conclusion

In this contribution, we discussed the case where the gNB-DU needs to apply full configuration also at UE Context Modification procedures. For that the Full Configuration IE should also be included in the UE CONTEXT MODIFICATION RESPONSE message..

Proposal 1: 	Add Full Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message.

### A CR mirroring the proposal above is presented in R3-193000.