3GPP TSG-RAN3 Meeting #104	R3-193019

Reno, U.S., 13th – 17th May 2019

Agenda Item:	14.2

Source: 	Ericsson

Title:  	Discussion on general aspects of MT EDT in Msg4

Document for:	Discussion

# Introduction

MT EDT has been discussed in recent RAN3 and RAN2 meetings, where the focus is now on the two following options: DL data transmission after the preamble transmission (Msg2-based option [1]) and DL data transmission in Msg4 (Msg4-based option [2]). In RAN2#105bis, the following agreements were made:

- For both UP and CP solutions, an MT-EDT indication is needed in the S1 paging message to eNB. It is up to RAN3 to decide how such indication is provided to the eNB. [3]

- RAN2 assumes that MME initiates MT-EDT.

- It is up to eNB to use MT-EDT based on e.g., UE capability.

- RAN2 assumes that DL data information is needed from S-GW/SCEF to the MME to assist MME to initiate MT-EDT.

- For the UP solution, the DL data are ciphered and sent over DTCH.

- For both UP and CP solutions eNB sends MT EDT indication to the UE via paging

- For Msg-2 based solution (if agreed)

- A CF RACH resource is provided in the paging message to page the UE for MT-EDT. FFS whether/how security related concerns are addressed and how number of repetitions required and RNTI are provided.

- For Msg-4 based solution (if agreed)

- For UP solution, RRCConnectionResume is used in Msg4 in case UL transmission is expected in response

It is assumed that RAN2 will further discuss how to develop UP and CP solutions based on the two remaining options. This contribution discusses in detail remaining aspects for UP and CP solutions for DL data in Msg4. We also discuss the RAN3 requirements needed for these solutions as well as the open issues.

# Discussion

## 2.1		MT UP-EDT

Figure 1 shows the signaling flow of the proposed MT UP-EDT solution.

As mentioned in our paper [4] and CR in [5], the eNB needs to determine the initiation of MT-EDT based on DL data size received from MME in S1 paging message. Depending on the received information, the eNB can decide whether to initiate MT-EDT accordingly.

Figure 1: Example of signalling flow for MT UP-EDT

In [2] and in our previous contributions, we stressed the fact that DL data may arrive after the paging so that the conditions for MT-EDT are no longer met by the time of sending Msg4. Thus, in addition to UL indication about possible UL response, it is preferred that the eNB knows DL data size before building Msg4 so that it can determine whether the UE can be released or not. We think it would be beneficial if information about DL data size is included in the Modify bearer procedure, as in Figure 1 and Figure 2. In addition, as also discussed in [1], this aspect is applicable to also Msg2-based solutions.

- Is is beneficial if the MME/eNB can be aware of DL data size before DL data transmission with data to determine whether to send the UE back to RRC_IDLE.

Once MME is aware of DL data size, it can include this information or an indication whether the connection can be released or not in the S1-AP message to eNB. For example, for the UP solution, the DL  data size information can be sent in the UE CONTEXT RESUME message so that eNB can determine whether the UE can be released or should be kept in connected mode for further data transmissions. (see our proposed CR in [6]).

MME sends to eNB an MT UP-EDT DL data size IE via UE CONTEXT RESUME, so that eNB can quickly decide whether to send the UE to IDLE mode.

## 2.2	MT CP-EDT

Figure 2 shows the signaling flow of the proposed MT CP-EDT solution.

Figure 2: Example of signaling flow for MT CP-EDT

If the UE sends the RRCEarlyDataRequest in Msg3, the eNB can then send to MME the S1-AP Initial UE message to establish a connection between MME and S-GW. As mentioned above, it can be beneficial if the DL data size is included in Step 9 Modify Bearer before sending DL data to the MME. Based on the information about DL data size, the MME can then include the End indication (which was introduced for MO-EDT) in the S1-AP DL NAS message to facilitate the release. Depending on the received information, the eNB can hence decide whether to release the RRC connection.

“End indication” from MME to eNB, introduced for MO CP-EDT, is used to facilitate immediate release in MT CP-EDT.

RAN3 will likely receive an LS from RAN2 at the end of this meeting to introduce these changes. Therefore RAN3 should monitor RAN2 progress and align on any decision that has S1 impacts.

# Conclusion

In the previous sections we discussed general signaling requirement between eNB and Core Network to support efficient support of DL transmission of MT EDT and made the following observation:

Observation 1	Is is beneficial if the MME/eNB can be aware of DL data size before DL data transmission with data to determine whether to send the UE back to RRC_IDLE

Based on the discussion in this paper, we propose the following:

- MME sends to eNB an MT UP-EDT DL data size IE via UE CONTEXT RESUME, so that eNB can quickly decide whether to send the UE to IDLE mode

- “End indication” from MME to eNB, introduced for MO CP-EDT, is used to facilitate immediate release in MT CP-EDT

# Reference

- R2-1906930, “MT early data after preamble”, source Ericsson, RAN2#106

- R2-1906929, “MT early data in Msg4”, Ericsson, RAN2#106

- R2-1905276, “LS on Mobile-terminated Early Data Transmission”, RAN2#105bis

- R3-193018, “MT EDT indication in S1 paging”, Ericsson, RAN3#104

- R3-192868, “solution for MT EDT initiation in Paging”, Ericsson, RAN3#104

- R3-192869, “MT UP EDT indication in S1-AP”, Ericsson, RAN3#104