3GPP TSG-RAN WG3 Meeting #104	R3-193113

Reno, Nevada, US, 13 - 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0065 | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple TNLAs for Xn-C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-5-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no clarification for the addition and removal of multiple TNLA on Xn. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new Section on Multiple TNLA Addition and Removal for Xn Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR is backward compatible because this only add clarifications on Multiple TNLAs for Xn-C, and has no functional impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | no clarification for Multiple TNLA Addition and Removal procedures for Xn. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X, and X.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

# X	Overall procedures in NG-RAN Architecure

## x.1	Multiple TNLAs for Xn-C

In the following, the procedure for managing multiple TNLAs for Xn-C is described.

Figure x.1-1: Managing multiple TNLAs for Xn-C.

1.	The NG-RAN node1 establishes the first TNLA with the NG-RAN node2 using a configured TNL address.

NOTE: 	The NG-RAN node1 may use different source and/or destination IP end point(s) if the TNL establishment towards one IP end point fails. How the NG-RAN node1 gets the remote IP end point(s) and its own IP address are outside the scope of this specification.

2-3.	Once the TNLA has been established, the NG-RAN node1 initiates the Xn Setup procedure to exchange application level configuration data

4-6.	The NG-RAN node2 may add additional TNL Endpoint(s) to be used for Xn-C signalling between the NG-RAN node1 and the NG-RAN node2 pair using the NG-RAN node Configuration Update procedure. NG-RAN node Configuration Update procedure also allows the NG-RAN node2 to request the NG-RAN node1 to modify or release TNLA(s).

7-9.	The NG-RAN node1 may add additional TNL Endpoint(s) to be used for Xn-C signalling between the NG-RAN node1 and the NG-RAN node2 pair using the NG-RAN node Configuration Update procedure. NG-RAN node Configuration Update procedure also allows the NG-RAN node1 to request the NG-RAN node2 to modify or release TNLA(s).

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>