3GPP TSG-RAN WG3 Meeting #105	R3-193730

# Ljubljana, Slovenia, August 26th – 30th 2019

Agenda Item:	13.2.1.1

Source:	Ericsson

Title:	(TP for NR-IAB BL CR for TS 38.401): IAB OAM Connectivity

Document for:	Agreement

# Discussion

At the RAN3#104 meeting the discussion on IAB OAM connectivity options has resulted in the following agreement:

Specify in St2 optA, and that optB is allowed

Where the 2 methods of handling OAM signalling for the IAB-node:

- Option A: Using a normal PDU session or PDN connection,

- Option B: Using the backhaul (BH) IP layer.

Accordingly, this TP proposes capturing the above agreement into Stage-2 specification i.e. TS 38.401.

Proposal: Agree to the TP for NR-IAB BL CR for TS 38.401 on IAB OAM connectivity, given in the Annex.

# Annex: TP for NR-IAB BL CR for TS 38.401

-------------------------------------------Change 1-------------------------------------------

## 8.z	IAB-node Integration Procedure

A high-level flow chart for SA-based IAB integration is shown in the Figure 8.z-1:

Figure 8.z-1: The integration procedure for IAB-node.

- Phase 1: IAB-MT setup. In this phase, the MT functionality of the new IAB-node (e.g. the IAB-node 2 in Figure 8.z-1) connects to the network as a normal UE, by performing RRC connection setup procedure with IAB-donor-CU, authentication with the core network, IAB-node 2-related context management, IAB-node 2’s access traffic-related radio bearer configuration at the RAN side, and, optionally, OAM connectivity establishment.

Editor’s Note: The signalling flow for UE initial access procedure as shown in Figure 8.1-1/Figure 8.9.1-1 should be used as baseline for the setup of the MT functionality of IAB-node. Whether to specify further details, e.g. how to perform the IAB node indication, etc. is FFS.

- Phase 2-1: Backhaul RLC channel establishment. In this phase, at least the backhaul RLC channels for CP traffic e.g. carrying F1-C messages to and from the IAB-node, are established. If the OAM connectivity is supported via BH IP layer (possible by implementation), one or more backhaul RLC channels used for OAM traffic can also be established.

Editor’s Note: It’s FFS whether the backhaul RLC channels for carrying UEs’ traffic are set up in this phase also.

- Phase 2-2: Routing update. In this phase, the BAP layer is updated to support routing between the new IAB-node 2 and the IAB-donor-DU. This includes configuration of a BAP routing identifier for routing in downstream direction on the IAB-donor-DU and a BAP route identifier in upstream direction on MT functionality of IAB-node 2. The routing tables are updated for all ancestor IAB-nodes (e.g. IAB-node 1 in Figure 8.z-1) and the IAB-donor-DU with routing entries for the new BAP routing identifier. The DU functionality of the new IAB-node configures an IP address to establish IP connectivity to the operator’s network.

Editor’s Note: It is FFS on how to support routing configuration and update.

Editor’s Note: It is FFS if bearer mapping has to be updated on IAB-nodes and IAB-donor-DU.

Editor’s Note: Details about IP address allocation for the DU functionality of IAB node are FFS.

- Phase 3: IAB-DU part setup. In this phase, the DU functionality of the IAB-node 2 is configured. The DU functionality of the IAB-node 2 initiates the set up F1-C connection with the IAB-donor-CU. After the F1 is setup, the IAB node can now start serving the UEs.

Editor’s Note: F1-C setup includes establishment of TNL association to the IAB-donor-CU.

Editor’s Note: The F1 Startup procedure in clause 8.5 can be reused for the DU part of IAB-node setup.

Editor’s Note: The IAB-node DU can discover the IAB-donor-CU’s IP address in the same manner as a normal DU. Alternative options are FFS.

-------------------------------------------End of changes-------------------------------------------