3GPP TSG-RAN WG3 Meeting #105	     R3-193798

Ljubljana, Slovenia, 26 – 30 August, 2019

Title:	[DRAFT] Reply LS on radio resource management policy

Response to:	S5-194458

Release:	Release 16

Work Item:	eNRM, NR_SON_MDT

Source:	Nokia [to be RAN3]

To:	SA5

Cc:	RAN2

Contact Person:

#### Name:	Håkon Helmers

E-mail Address:	hakon.helmers@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thank SA5 for their LS on radio resource management policy. RAN3 has reviewed SA5’s proposals and would like to provide the following feedback.

SA5 write in their LS: “The RRM Policies defined in TS 28.541, are today used in gNB-CU (NRCellCU). This could work when gNB is deployed in a non-split deployment scenario.” RAN3 would like to draw the attention to the NG-RAN architecture description in TS 38.300 valid for non-split deployment scenario, in which there is no gNB-CU node.

For 2-split and 3-split scenarios, RAN3 would like to confirm SA5’s proposal based on the following understanding:

For  2-split scenario:

O&M configures quotas in the gNB-CU for:

- number of DRBs per network slice / per sharing operator. Granularity may be per gNB and/or per gNB-DU.

- number of RRC connected user (RRC active, RRC inactive) per network slice / per sharing operator. Granularity may be per gNB and/or per gNB-DU.

- PDCP resources (e.g. consumed bandwidth / bit rate) per network slice / per sharing operator. Granularity may be per gNB.

O&M configures quotas in the gNB-DU for:

- PRB consumption per network slice / per sharing operator. Granularity may be per gNB-DU and/or cell.

The gNB-CU will typically use the above mentioned gNB-CU parameters for admission control, while the gNB-DU will use the configured PRB information either for admission control or scheduling decision.

For  3-split scenario:

O&M configures quotas in the gNB-CU-CP for:

- number of DRBs per network slice / per sharing operator. Granularity may be per gNB-CU-CP and/or per gNB-CU-UP and/or per gNB-DU.

- number of RRC connected user (RRC active, RRC inactive) per network slice / per sharing operator. Granularity may be per gNB-CU-CP and/or per gNB-DU.

O&M configures quotas in the gNB-CU-UP for:

- PDCP resources (e.g. consumed bandwidth / bit rate) per network slice / per sharing operator. Granularity may be per gNB-CU-UP.

O&M configures quotas in the gNB-DU for:

- PRB consumption per network slice / per sharing operator. Granularity may be per gNB-DU and/or cell.

The main difference from the 2-split scenario is that PDCP resources are configured in the gNB-CU-UP, which will use this information for admission control.

2. Actions:

To SA5 : 	RAN3 kindly asks SA5 to take the above into account.

3. Dates of Next TSG-RAN WG3 Meetings:

3GPP TSG RAN WG3#105bis	14 – 18 October, 2019		Chongqing, China

3GPP TSG RAN WG3#106		18 – 22 November, 2019	Reno (NV), USA