3GPP TSG-RAN3 Meeting #105	R3-194091

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0078 | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.300 on RACH optimization solution for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-06-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to capture the stage 2 text for support of RACH optimization function in split gNB case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Stage 2 text description for RACH optimization function is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of RACH optimization can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.300 CR 00XX |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS 38.423 CR 00XX |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS 38.473 CR 00XX |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

7.X	RACH Optimisation Function

RACH optimization in the case of split gNB is supported by PRACH parameters exchange over F1 between gNB-CU and gNB-DU.

In the case of split gNB, the gNB-DU receives the UE’s RACH report from the gNB-CU which is reported by the UE via RRC signalling. The gNB-CU may forward the neighbour cell’s PRACH configurations receving from neighbour nodes to the gNB-DU.

End of the first change