3GPP TSG-RAN3 Meeting #105	R3-194113

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for NR_Mob_enh BL CR for TS 38.300): CHO procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN3#104 meeting, it has agreed to reuse existing HO Preparation procedure for CHO preparation, reuse existing HO cancel for canceling CHO from source and introduce the Handover Success procedure to inform the source NG-RAN node that the UE has successfully attached to the target NG-RAN node. The stage 2 message flows for conditional handover is still missing in TS38.300. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the stage 2 messag flows for CHO in TS38.300. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 conditional handover procedure in TS38.300 is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

#### 9.2.3.2a	Conditional Handover

Conditional Handover (CHO) is defined as having a configured condition that determines when/whether the corresponding handover command is executed. Upon receiving the CHO configuration, UE starts to evaluate the condition(s) and executes the HO command once the condition is met.

##### 9.2.3.2a.1	C-plane handling

The figure below depicts the basic conditional handover procedure:

# Figure 9.2.3.x.1-1: Intra-AMF/UPF CHO

0.	The UE context within the source gNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

1.	The source gNB configures the UE measurement procedures and the UE reports according to the measurement configuration.

2.	The source gNB makes CHO decision based on MeasurementReport and RRM information.

3.	The source gNB issues a Handover Request message which includes the CHO indidation to the candidate gNB passing a transparent RRC container with necessary information to prepare the handover at the candidate side. In CHO, the source gNB can issue multiple Handover Request messages to multiple candidate gNBs. And then, admission control may be performed by the multiple candidate gNBs.

4.	The candidate gNB prepares the conditional handover with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source gNB. In CHO, each candidate gNB can send the HANDOVER REQUEST ACKNOWLEDGE message to the source gNB.

FFS the detailed information in HANDOVER REQUEST ACKNOWLEDGE message.

5.	The source gNB triggers the Uu conditional handover by sending the CHO configurations to the UE.

FFS which message to include the CHO configurations and the detailed information of CHO configurations.

6.	The UE evaluates the CHO configurations and synchronises to the target cell which fulfils the corresponding CHO trigger condition.

7.	The UE performs RACH procedure with the target cell and completes the conditional handover procedure by sending one RRC message to target gNB. FFS whether the RRC message can be RRCReconfigurationComplete message.

8.	When conditional handover procedure is complete, the target gNB sends the Handover Success message to inform the source gNB that the UE has successfully attached to the target gNB.

9.	When reveiving the Handover Success message, the source gNB sends the SN STATUS TRANSFER message to the target gNB, and data forwarding starts.

10.	The target gNB sends a PATH SWITCH REQUEST message to AMF to trigger 5GC to switch the DL data path towards the target gNB and to establish an NG-C interface instance towards the target gNB.

11.	5GC switches the DL data path towards the target gNB. The UPF sends one or more "end marker" packets on the old path to the source gNB per PDU session/tunnel and then can release any U-plane/TNL resources towards the source gNB.

12.	The AMF confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

13.	Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message from the AMF, the target gNB sends the UE CONTEXT RELEASE to inform the source gNB about the success of the handover. The source gNB can then release radio and C-plane related resources associated to the UE context.

End Change