3GPP TSG-RAN WG3 Meeting #105 	R3-194217

# Ljubljana, Slovenia, August 26-30 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0130 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to DRB to Setup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CPUP_Split |  |  |  |  |  |  | Date: |  | 2019-08-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - PDCP SN Status Information IE is positioned incorrectly included outside of DRB information in tabular. - An instance of DRB To Setup Item IE incorrectly includes Flow Mapping Information IE, the correct IE should be QoS Flows Information To Be Setup IE. Both of these IEs are of the same type. Thus, this correction consists only of a name change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Fix indentation on PDCP SN Status Information IE - Rename Flow Mapping Information IE to QoS Flows Information to Be Setup IE in DRB To Setup Item IE Impact assessment towards the previous version of the specification (same release): This CR has an impact under protocol point of view. The impact can be considered isolated because the change is limited to PDCP SN Status Information IE and renaming of an IE within DRB To Setup Item IE. This CR is ASN.1 backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned ASN.1 and tabular. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

Beginning of Text Proposal to TS 38.463

#### 9.3.3.10	PDU Session Resource To Setup Modification List

This IE contains PDU session resource to setup related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Modification Item |  | 1..<maxnoofPDUSessionResource> |  |  |
| >PDU Session ID | M |  | 9.3.1.21 |  |
| >PDU Session Type | M |  | 9.3.1.22 |  |
| >S-NSSAI | M |  | 9.3.1.9 |  |
| >Security Indication | M |  | 9.3.1.23 |  |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 | This IE shall be present when Non-GBR QoS Flows are setting up. |
| >NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. |
| >Network Instance | O |  | 9.3.1.62 |  |
| >DRB To Setup List |  | 1 |  |  |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  |
| >>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry. |
| >>>QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  |
| >>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

<< unchanged sections omitted >>

#### 9.3.3.11	PDU Session Resource To Modify List

This IE contains PDU session resource to modify related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  |
| >PDU Session ID | M |  | 9.3.1.21 |  |
| >Security Indication | O |  | 9.3.1.23 |  |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  |
| >NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. |
| >PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information Response 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. |
| >Network Instance | O |  | 9.3.1.62 |  |
| >DRB To Setup List |  | 0..1 |  |  |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  |
| >>>Cell Group Information | M |  | 9.3.1.11 |  |
| >>> QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  |
| >>>DRB Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. |
| >DRB To Modify List |  | 0.. 1 |  |  |
| >>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |
| >>>SDAP Configuration | O |  | 9.3.1.39 |  |
| >>>PDCP Configuration | O |  | 9.3.1.38 |  |
| >>>DRB Data forwarding information Response | O |  | Data Forwarding Information Response 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. |
| >>>PDCP SN Status Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP SN Status in the response message. |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status to the target gNB-CU-UP. |
| >>>DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  |
| >>>Cell Group To Add | O |  | Cell Group Information 9.3.1.11 |  |
| >>>Cell Group To Modify | O |  | Cell Group Information 9.3.1.11 |  |
| >>>Cell Group To Remove | O |  | Cell Group Information 9.3.1.11 |  |
| >>>Flow Mapping Information | O |  | QoS Flow QoS Parameters List 9.3.1.25 | Overrides previous mapping information. |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. |
| >DRB To Remove List |  | 0.. 1 |  |  |
| >>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |
| >S-NSSAI | O |  | 9.3.1.9 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

<< unchanged sections omitted >>

End of Text Proposal to TS 38.463