3GPP TSG-RAN3 Meeting #105 	R3-194233

# Ljubljana, Slovenia,  26-30 Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0045 | rev | 2 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RRC Inactive transition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Similar to Step 5 that the CellGroupConfig could be retrieved from the old NG-RAN node, for periodic RNA update without UE context relocation, the last serving gNB responds to the gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. Therefore, the RRCRelease message may be generated by the old NG-RAN node instead of the local gNB-CU itself. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the gNB-CU may receive RRCRelease message from the old NG-RAN node in step 7. - Editorial - Aligning RRC message format Impact Analysis: Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change affects only RRC INACTIVE to RRC INACTIVE or IDLE procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not correctly captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is revised from R3-192556 and R3-191466 |  |  |  |  |  |  |  |  |

### 8.6.2	RRC inactive to other states

This section gives the RRC inactive to other RRC states transition given that gNB consists of gNB-CU and gNB-DU(s), as shown in Figure 8.6.2-1.

Figure 8.6.2-1: RRC inactive to other RRC states transition procedure

1.	If data is received from 5GC, the gNB-CU sends PAGING message to gNB-DU.

2.	The gNB-DU sends Paging message to UE.

NOTE: step 1 and 2 only exist in case of DL data arrival.

3.	UE sends RRCResumeRequest message either upon RAN-based paging, UL data arrival or RNA update.

4.	The gNB-DU includes RRCResumeRequest in a non-UE associated  INITIAL UL RRC MESSAGE TRANSFER message and transfer to the gNB-CU.

5.	For UE Inactive to UE Active transitions, excluding transitions due to signalling exchange only, the gNB-CU allocates gNB-CU UE F1AP ID and sends UE CONTEXT SETUP REQUEST message to gNB-DU, which may include SRB ID(s) and DRB ID(s) to be setup, CellGroupConfig stored in gNB-CU or retrieved from the old NG-RAN node may also be included.

6.	The gNB-DU responds with UE CONTEXT SETUP RESPONSE message, which contains RLC/MAC/PHY configuration of SRB and DRBs provided by the gNB-DU.

NOTE:	step 5 and 6 exist for inactive to active transitions, excluding transitions due to signalling exchange only. When gNB-CU successfully retrieves and verifies the UE context, it may decide to let the UE enter into RRC active mode. gNB-CU shall trigger UE context setup procedure between gNB-CU and gNB-DU, during which both SRB1, SRB2 and DRB(s) can be setup. For signalling exchange only transitions, gNB-CU does not trigger UE Context Setup procedure. For inactive to Idle transitions the gNB-CU does not trigger the UE Context Setup procedure.

7.	The gNB-CU generates RRCResume/RRCSetup/RRCReject/RRCRelease message or receives RRCRelease message from the old NG-RAN node towards UE. The RRC message is encapsulated in DL RRC MESSAGE TRANSFER message together with SRB ID.

8.	The gNB-DU forwards RRC message to UE either over SRB0 or SRB1 as indicated by the SRB ID.

NOTE:	in step 7, it is expected that gNB-CU takes appropriate action, e.g. generates RRC resume message for inactive to active state transition(for both cases of signaling exchange only, and UP data exchange), generates RRCSetup message for fallback to establish a new RRC connection, and generates or receives from the old NG-RAN node either RRCRelease message without suspend configuration for inactive to idle state transition, or RRCRelease message with suspend configuration to remain in inactive state. 
If step 5 and 6 are not performed, the gNB-DU deduces the SRB on which to deliver the RRC message in step 7 from the SRB ID, i.e. SRB ID “0” corresponds to SRB0, SRB ID “1” corresponds to SRB1.

9.	UE sends RRCResumeComplete/RRCSetupComplete message to the gNB-DU.

10.	The gNB-DU encapsulates RRC in UL RRC MESSAGE TRANSFER message and send to the gNB-CU.

NOTE:	step 9 and step 10 exist for inactive to active state transition (for both cases of signaling exchange only, and UP data exchange). UE generates RRCResumeComplete/RRCSetupComplete message for resume the existing RRC connection or fallback to a new RRC connection respectively.