3GPP TSG RAN WG3 Meeting #105                                     R3-194760

Ljubljana, Slovenia, 26th – 30th August 2019

Agenda Item	: 26.2

Source		: CMCC

Title			: Text proposal to TR 38.856 on local LMF architecture

Document for	: Approval

- Introduction

This contribution provides a TP to TR 38.856 on architectures to support local LMF in NG-RAN, based on the summary of offline discussion in [1].

2	References

- R3-194713, Summary of offline discussion for local LMC support in NG-RAN, CMCC

Text proposal for TR 38.856

5		Local LMF in NG-RAN

Editor note: Specification impact of Local LMF including location of the LMF, potential new interface(s) (if any), impact on existing protocols, and coordination with the LMF in the 5GC

5.1		Architecture

5.1.X 	Alternative 1

Editor’s note: An outline of the characteristics of the architecture should be provided, explaining rationale and differences vs other architectures

- Alternative 1: LMC is a functionality of NG-RAN node. In case of split gNB, the LMC is a function of the gNB-CU-CP

Figure 5.1-x  Alternative 1: UE Positioning Architecture applicable to NG-RAN, including LMC

5.1.Y 	Alternative 2

Editor’s note: An outline of the characteristics of the architecture should be provided, explaining rationale and differences vs other architectures

- Alternative 2: LMC is a logical node of gNB in split gNB architecture, the LMC is connected to gNB-CU

Figure 5.1-y Alternative 2: UE Positioning Architecture applicable to NG-RAN, including LMC

5.1.Z 	Alternative 3

Editor’s note: An outline of the characteristics of the architecture should be provided, explaining rationale and differences vs other architectures

Figure 5.1-z Alternative 3: UE Positioning Architecture applicable to NG-RAN, including LMC

- Alternative 3: LMC is a logical node of NG-RAN architecture. A LMC connects with a set of NG-RAN nodes through a new interface or extension of an existing interface.

5.1.Y Function supported by LMC [FFS]

Editor note: At this stage it is not clear if the LMC is able to support all the function of the LMF. The positioning method supported by the LMC is FFS.

The LMC need to support all the location request types defined for the LMF in clause 4.1a of TS 23.273 [4] and supported in Rel-15, i.e.:

-	Network Induced Location Request (NI-LR);

-	Mobile Terminated Location Request (MT-LR);

-	Mobile Originated Location Request (MO-LR);

Note:	According to TS 38.305, MO-LR location service is not supported in Rel-15. It is therefore out-of-scope of this study item.

-	Immediate Location Request; and

-	Deferred Location Request.

The LMC should be able to support location service requests from functions internal to the NG-RAN node, this type of location request can be called RAN Induced Location Request (RI-LR). The LMC is ideally suited to handle RI-LR since the necessary operations are internal to the RAN and can therefore be handled locally.

In order to manage the various location services for target UEs, the LMC can be expected to have capabilities similar to an LMF, e.g.:

-	receive location service requests for a target UE from the serving AMF

Editor note: it is FFS whether service request can come from an internal function of the serving NG-RAN node (e.g. RRM, MDT, etc.);

-	decide on the position method(s) to be used;

-	interact with other internal functions of the serving NG-RAN node (e.g. TMF) in order to e.g. obtain position measurements for the UE;

-	interact with a target UE in order to e.g. deliver assistance data or to obtain a location estimate;

-	interact with neighboring NG-RAN nodes in order to e.g. obtain position measurements for the UE or coordinate dynamic PRS configuration, handling positioning request for UE in neighboring node;

-	calculate or verify the final location of the UE and provide it to the requesting entity or requesting internal function of the serving NG-RAN node.

5.2		Impact on existing protocols and interfaces [FFS]

5.2.1	Location Service procedures involving LMC

Editor Note: flows are intended to show common operation for all architectures with LMC. Specific architectures may require further details

5.2.1.1	RAN Induced Location Request (RI-LR)

Figure 5.2.1.1-1 illustrates the steps applicable to the UE and NG-RAN node in case of RI-LR involving LMC.

In this scenario, the UE is assumed to be in connected mode prior to step 1. It is also assumed that the NG-RAN node is always authorized to use the LCS service to obtain the target UE location estimation, thus no privacy verification and LCS service authorization are required for requests initiated by the NG-RAN (see subclause 6.11.1 of TR 23.731 [1]).

Figure X.2.1.1-1: RI-LR involving LMC

The steps of Figure X.2.1.1-1 are as follows:

1.	Some function in the NG-RAN node (e.g. RRM, MDT) requests some location service (e.g. positioning) for a target UE to the LMC using signaling internal to the NG-RAN node.

EN:	Enhancements to F1AP, if needed (e.g. in case the requesting function is in the DU), are FFS.

2.	The NG-RAN node (LMC) sends an LCS Indication message to the AMF, indicating that UE positioning is being performed locally (LCS session start). The LCS Indication message includes information about the “local” LCS Request that may be useful for the AMF (location management information), e.g. the same or subset of information contained in a “normal” location service request (e.g. MT-LR) such as Location Quality of Service information.

3.	The AMF stores the relevant location management information in the UE location context, to be used for coordination of concurrent LCS requests, e.g. if the AMF receives an MT-LR while there is an active location session at the LMC for the same UE.

EN:	Coordination with the 5GC, if needed, is FFS.

4a.	The LMC instigates location procedures internal to the NG-RAN node – e.g. to obtain positioning measurements or assistance data. The LMC may also instigate location procedures with neighbor NG-RAN nodes – e.g. to obtain assistance data such as PRS configuration of TPs served by the neighbor NG-RAN node(s).

EN:	Enhancements to XnAP, if needed, are FFS.

4b.	In addition to step 4a or instead of step 4a, for downlink positioning the LMC instigates location procedures with the UE – e.g. to obtain a location estimate or positioning measurements or to transfer location assistance data to the UE.

5.	The LMC responds to the requesting function in the NG-RAN node and includes any needed results – e.g. success or failure indication and, if requested and obtained, a location estimate for the UE.

6.	If the response in step 5 ends the local LCS session, the NG-RAN node (LMC) sends an LCS Indication message to the AMF indicating that positioning is no longer being performed locally for the target UE (LCS session stop).

5.2.1.2	5GC Mobile Terminated Location Request (5GC-MT-LR)

Figure X.2.1.2-1 illustrates the steps applicable to the UE and NG-RAN node in case of 5GC-MT-LR involving LMC. In this scenario, the UE is assumed to be in connected mode prior to step 1.

Figure X.2.1.2-1: 5GC-MT-LR involving LMC

The steps of Figure X.2.1.2-1 are as follows:

1.	Some LCS entity in the 5GC (e.g. GMLC) requests some location service (e.g. positioning) for a target UE to the serving AMF (see step 4 of Figure 6.1.1-1 in TS.23.273 [4]).

2.	The AMF selects the serving NG-RAN node (LMC) to perform the positioning, based on e.g. knowledge of the NG-RAN node’s location management capabilities, Requested Quality of Service information, etc.

Note 1:	The details of whether to select LMC are SA2 scope, including whether it is part of the LMF Selection function or a new function of the AMF.

3.	The AMF sends a location service request to the NG-RAN node (LMC) via NGAP, instructing it to perform the UE positioning.

4.	Same as steps 4a/4b of the RI-LR procedure in clause X.Y2.1.

5.	The NG-RAN node (LMC) provides a location service response to the AMF via NGAP and includes any needed results – e.g. success or failure indication and, if requested and obtained, a location estimate for the UE.

6.	The AMF returns a location service response to the 5GC entity of step 1 and includes any needed results – e.g. a location estimate for the UE (see step 10 of Figure 6.1.1-1 in TS.23.273 [4]).

5.3		Coordination and coexistence with LMF in the 5GC

Editor Note: Function split between LMC in NG-RAN and LMF in 5GC is FFS

Editor Note: Whether functions supported by LMC can be different per architecture alternative is FFS.

Editor Note: How/who to trigger location request to LMC and whether the location request can come from internal function of NG-RAN node is FFS