3GPP TSG-RAN3 Meeting #105bis	R3-194905

Chongqing, China, 14-18 October 2019

3GPP TSG RAN WG1 #98		R1-1909550

Prague, CZ, August 26th – 30th, 2019

Title:	Reply LS on PDCCH monitoring for NR-DC

Response to:	R1-1908005/ R2-1908487

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG1

To:	RAN WG2

CC:	RAN WG3

Contact Person:

#### Name:		Aris Papasakellariou

- E-mail Address:	aris.papas@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 would like to thank RAN2 for introducing the configuration, capability, and inter-node parameters for supporting PDCCH monitoring in NR-DC.

RAN1 would like to confirm that all associated constraints have been captured in Subclause 10.1 of TS38.213 v15.6.0 except for the following (see also R1-1907956/ R2-1908625).

- If the UE does not report pdcch-BlindDetectionCA, and if X is the maximum number of CCs supported by the UE across all NR-DC band combinations then there is at least one parameter pair (X1, X2) such that X1 + X2 = X and the UE supports at least one NR-DC band combination with X1 CCs in MCG and X2 CCs in SCG and for which X1 <= pdcch-BlindDetectionMCG-UE and X2 <= pdcch-BlindDetectionSCG-UE.

One example for the above constraint is that, for a UE reporting (pdcch-BlindDetectionMCG-UE, pdcch-BlindDetectionSCG-UE) as (1, 3) and X=4, the UE shall support at least a band combination of [1CC, 3CC], instead of supporting only [2CC, 2CC] for all band combinations.

2. Actions to RAN WG2

RAN1 respectfully requests RAN2 to capture the above constraint in the RAN2 specifications for NR-DC (including for late drop UEs).

3. Dates of Next RAN WG1 Meetings:

3GPP RAN1#98bis		14 – 20 October 2019	Chongqing, P.R. China

3GPP RAN1#99		18 – 22 November 2019	Reno, NV, USA