3GPP TSG-RAN WG3 #105Bis							                                               R3-195045

Chongqing, China, 14th –18th October 2019

Agenda item:			17.3.2

Source:					ZTE

Title:	 (TP for NR_IIOT BL CR for TS 38.473): TSC Traffic Characteristics related modification

Document for:		Approval

- Introduction

As specified in TS 23.501, the knowledge of TSN traffic pattern is useful for the gNB to allow it to more efficiently schedule periodic deterministic traffic flows either via Configured Grants, Semi-Persistent Scheduling or with dynamic grants. In the BL CR for R3-194650, TSC Traffic Characteristics IE needs further study, and the Maximum Data Burst Volume IE cannot differentiate uplink/downlink.

To support this, TSC Traffic Characteristics and Maximum Data Burst Volume should be provided from gNB-CU to gNB-DU.

Detailed analysis can be seen in the related discussion paper (R3-195042).

- Text Proposal (TS38.473 15.7.0)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.1.45	QoS Flow Level QoS Parameters

This IE defines the QoS to be applied to a QoS flow or to a DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  | - |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.49 |  | - |  |
| >Dynamic 5QI |  |  |  |  | - |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.47 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.48 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.46 | This IE shall be present for GBR QoS Flows only and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | Details in TS 23.501 [21]. This IE applies to non-GBR flows only and is ignored otherwise. | - |  |
| PDU Session ID | O |  | INTEGER (0 ..255) | As specified in TS 23.501 [21]. | YES | ignore |
| UL PDU Session Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.22 | The PDU session Aggregate Maximum Bit Rate Uplink which is associated with the involved PDU session. | YES | ignore |
| TSC Traffic Characteristics | O |  | 9.3.1.y | This IE provides the traffic characteristics of TSC QoS flows. This IE shall be present for TSC traffic only. | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.1.y	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [21]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | FFS | Periodicity as specified in TS 23.501 [21]. |
| Burst Arrival Time | O |  | FFS | Burst Arrival Time as specified in TS 23.501 [21]. |
| Maximum Data Burst Volume | C-ifBurstArrivalTime |  | 9.3.1.54 | This IE is specified in TS 23.501 [21]. If this IE is present here, it shall be ignored in both subclause 9.3.1.47 Dynamic 5QI Descriptor IE and subclause 9.3.1.49 Non Dynamic 5QI Descriptor IE if configured. |

| Condition | Explanation |
| --- | --- |
| C-ifBurstArrivalTime | This IE can be present if the Burst Arrival Time IE is present. Otherwise, it is absent. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>