3GPP TSG-RAN3 Meeting #105bis	R3-195138

Chongqing, China, 14-18 October 2019

Title: 	[TP to BL CR#0173 “Introduction of Control Plane CIoT 5GS Optimisation for NB-IOT and eMTC” for TS 38.413] Removing of Editor note

Source: 	Huawei

Agenda item:	14.3

Document for:	other

- Introduction

In R3-194976/R3-194809 there is an Editor’s Note about Connection Establishment Indication procedure:

Editor’s Note: it should be confirmed whether there is a need for this procedure to be used after a UE-associated logical NG-connection has been established

There was quite a long controversial discussion on the same question in EPS, and current design in S1AP is the outcome of a compromise, as similar function and status exists in NGAP, it is propose to define CEI procedure in NGAP similar to S1AP.

This paper provides the Text Proposal to remove the Editor’s note for CEI procedure.

- Text Proposal

---Start of the Change----

8.3.a	Connection Establishment Indication

8.3.a.1	General

The purpose of the Connection Establishment Indication procedure is to enable the AMF to complete the establishment of the UE-associated logical NG-connection, and/or trigger the NG-RAN node to obtain and report UE Radio Capability. The procedure uses UE-associated signalling. This procedure applies only if the NG-RAN node is an ng-eNB.

---End of the Change----

- Reference

- R3-194976/R3-194809, Introduction of Control Plane CIoT 5GS Optimisation for NB-IOT and eMTC, NGAP CR#173, Qualcomm Incorporated