3GPP TSG-RAN3 Meeting #105bis	R3-195139

Chongqing, China, 14-18 October 2019

Title: 	[TP to BL CR#0156 “Introduction of NB-IoT related NG-AP procedures” for TS 38.413] NB-IoT UE Priority

Source: 	Huawei

Agenda item:	14.3

Document for:	other

- Introduction

In R3-194978/R3-194811 [1], there is an FFS on the Integer range of NB-IoT UE Priority, and an Editor’s Note:

Editor’s Note: whether the NB-IoT UE Priority IE could be re-use for other functionality is FFS e.g. complementary to the Data Size.

In this contribution, we analyses this leftover issue and provide the corresponding Text Proposal.

- Discussion

As the NB-IoT UE Priority is introduced in 5GS to replace the inter UE QoS parameter used in EPS, considering that as inter UE QoS parameter includes QCI which is defined as INTEGER (0..255), to support similar granularity of priority, the current NB-IoT UE Priority IE is defined as INTEGER (0..255, …) in the baseline CR. it is proposed to remove the FFS on the value range, and only updated if any further information received from other groups.

Proposal 1: define the NB-IoT UE Priority IE as INTEGER (0..255, …), remove the corresponding FFS.

On the other hand, Editor’s Note was added based on a comment received in last meeting, “whether the NB-IoT UE Priority IE could be re-use for other functionality is FFS e.g. complementary to the Data Size.”, as we know that the data size is agreed to be included in Paging message to support MT-EDT, the NB-IoT UE Priority is used as inter UE QoS, therefore we do not see the explicit relationship between these two IEs.

Proposal 2: remove the Editor’s Note on NB-IoT UE Priority IE.

- Text Proposal

----Start of the Change----

9.3.1.a	NB-IoT UE Priority

This IE provides the NB-IoT UE Priority and to be used by the NG-RAN to prioritise between UEs accessing via NB-IoT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NB-IoT UE Priority | M |  | INTEGER (0..255, …) | Lower value indicates higher priority. |

----End of the Change----

- Reference

- R3-194978/R3-194811 Introduction of NB-IoT related NG-AP procedures, NGAP CR#156, Huawei