3GPP TSG-RAN3 Meeting #105bis	R3-195144

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1378 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NPRACH configuration exchange |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2019-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to support the RACH optimization, it is needed to exchange the NPRACH Configuration between eNBs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Include the NPRACH Configuration in Served Cell Information IE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support RACH optimization for NB-IOT Cells |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.2, 8.3.5.2, 9.2.8, 9.2.xxx, 9.2.yyy |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------Start of the First Change------

8.3.3	X2 Setup

8.3.3.1	General

The purpose of the X2 Setup procedure is to exchange application level configuration data needed for two eNBs to interoperate correctly over the X2 interface. This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also resets the X2 interface like a Reset procedure would do.

The procedure uses non UE-associated signalling.

8.3.3.2	Successful Operation

Figure 8.3.3.2-1: X2 Setup, successful operation

An eNB1 initiates the procedure by sending the X2 SETUP REQUEST message to a candidate eNB2. The candidate eNB2 replies with the X2 SETUP RESPONSE message. The initiating eNB1 shall transfer the complete list of its served cells and, if available, a list of supported GU Group Ids to the candidate eNB2. The candidate eNB2 shall reply with the complete list of its served cells and shall include, if available, a list of supported GU Group Ids in the reply.

If a cell is switched off for energy savings reasons, it should be activated before initiating or responding to the X2 Setup procedure and shall still be included in the list of served cells.

The initiating eNB1 may include the Neighbour Information IE in the X2 SETUP REQUEST message. The candidate eNB2 may also include the Neighbour Information IE in the X2 SETUP RESPONSE message. The Neighbour Information IE shall only include E-UTRAN cells that are direct neighbours of cells in the reporting eNB. A direct neighbour of one cell of a given eNB may be any cell belonging to an eNB that is a neighbour of that given eNB cell e.g. even if the cell has not been reported by a UE. The initiating eNB1 may include the TAC IE with the Neighbour Information IE in the X2 SETUP REQUEST message. The candidate eNB2 may also include the TAC IE with the Neighbour Information IE in the X2 SETUP RESPONSE message. The eNB receiving the IE may use it according to TS 36.300 [15].

The initiating eNB1 may include the Number of Antenna Ports IE in the X2 SETUP REQUEST message. The candidate eNB2 may also include the Number of Antenna Ports IE in the X2 SETUP RESPONSE message. The eNB receiving the IE may use it according to TS 36.331 [9].

The initiating eNB1 may include the PRACH Configuration IE in the X2 SETUP REQUEST message. The candidate eNB2 may also include the PRACH Configuration IE in the X2 SETUP RESPONSE message. The eNB receiving the IE may use this information for RACH optimisation.

The initiating eNB1 may include the MBSFN Subframe Info IE in the X2 SETUP REQUEST message. The candidate eNB2 may also include the MBSFN Subframe Info IE in the X2 SETUP RESPONSE message. The eNB receiving the IE may use it according to TS 36.331 [9].

For each CSG cell or hybrid cell served by the initiating eNB1 the X2 SETUP REQUEST message shall contain the CSG ID IE. For each CSG cell or hybrid cell served by the candidate eNB2 the X2 SETUP RESPONSE message shall contain the CSG ID IE. The eNB receiving the IE shall take this information into account when further deciding whether X2 handover between the source cell and the target cell may be performed.

The initiating eNB1 may include the MBMS Service Area Identity List IE in the X2 SETUP REQUEST message. The candidate eNB2 may also include the MBMS Service Area Identity List IE in the X2 SETUP RESPONSE message. The eNB receiving the IE may use it according to TS 36.300 [15].

For each cell served by the initiating eNB1 the X2 SETUP REQUEST message may contain the MultibandInfoList IE and may also contain the FreqBandIndicatorPriority IE. For each cell served by the candidate eNB2 the X2 SETUP RESPONSE message may contain the MultibandInfoList IE and may also contain the FreqBandIndicatorPriority IE. The eNB receiving the MultibandInfoList IE shall, if supported, take this information into account when further deciding whether subsequent mobility actions between the source cell and the target cell may be performed, and use this IE and the FreqBandIndicatorPriority IE, if received, as specified in TS 36.331 [9].

The initiating eNB1 may include the LHN ID IE in the X2 SETUP REQUEST message. The candidate eNB2 may also include LHN ID IE in the X2 SETUP RESPONSE message. The eNB receiving the IE may use it according to TS 36.300 [15].

The initiating eNB1 may include the BandwidthReducedSI IE in the X2 SETUP REQUEST message. The candidate eNB2 may also include BandwidthReducedSI IE in the X2 SETUP RESPONSE message. The eNB receiving the IE may use it to determine a suitable target in case of subsequent outgoing mobility involving BL UEs or UEs requiring CE.

The initiating eNB1 may include the NPRACH Configuration IE in the X2 SETUP REQUEST message. The candidate eNB2 may also include the NPRACH Configuration IE in the X2 SETUP RESPONSE message. The eNB receiving the IE may use this information for RACH optimisation.

------Start of the Next Change------

8.3.5	eNB Configuration Update

8.3.5.1	General

The purpose of the eNB Configuration Update procedure is to update application level configuration data needed for two eNBs to interoperate correctly over the X2 interface.

The procedure uses non UE-associated signalling.

8.3.5.2	Successful Operation

Figure 8.3.5.2-1: eNB Configuration Update, successful operation

An eNB1 initiates the procedure by sending an ENB CONFIGURATION UPDATE message to a peer eNB2 . Such message shall include an appropriate set of up-to-date configuration data, including, but not limited to, the complete lists of added, modified and deleted served cells, that eNB1 has just taken into operational use.

Upon reception of an ENB CONFIGURATION UPDATE message, eNB2 shall update the information for eNB1 as follows:

Update of Served Cell Information:

-	If Served Cells To Add IE is contained in the ENB CONFIGURATION UPDATE message, eNB2 shall add cell information according to the information in the Served Cell Information IE.

-	If Number of Antenna Ports IE is contained in the Served Cell Information IE in the ENB CONFIGURATION UPDATE message, eNB2 may use this information according to TS 36.331 [9].

-	If the PRACH Configuration IE is contained in the Served Cell Information IE in the ENB CONFIGURATION UPDATE message, the eNB receiving the IE may use this information for RACH optimisation.

-	If Served Cells To Modify IE is contained in the ENB CONFIGURATION UPDATE message, eNB2 shall modify information of cell indicated by Old ECGI IE according to the information in the Served Cell Information IE.

-	If MBSFN Subframe Info IE is contained in the Served Cell Information IE in the ENB CONFIGURATION UPDATE message, eNB2 may use this information according to TS 36.331 [9]. If a MBSFN subframe indicated in the MBSFN Subframe Info IE coincides with an ABS, the eNB2 shall consider that the subframe is designated as ABS by the sending eNB.

-	If BandwidthReducedSI IE is contained in the Served Cell Information IE in the ENB CONFIGURATION UPDATE message, eNB2 may use this information to determine a suitable target in case of subsequent outgoing mobility involving BL UEs or UEs requiring CE.

When either served cell information or neighbour information of an existing served cell in eNB1 need to be updated, the whole list of neighbouring cells, if any, shall be contained in the Neighbour Information IE.

If the Deactivation Indication IE is contained in Served Cells To Modify IE, it indicates that the concerned cell was switched off to lower energy consumption.

The eNB2 shall overwrite the served cell information and the whole list of neighbour cell information for the affected served cell.

-	If Served Cells To Delete IE is contained in the ENB CONFIGURATION UPDATE message, eNB2 shall delete information of cell indicated by Old ECGI IE.

-	If MBMS Service Area Identity List IE is contained in the Served Cell Information IE in the ENB CONFIGURATION UPDATE message, the eNB receiving the IE may use it according to TS 36.300 [15].

When the MBMS Service Area Identities of a cell in eNB1 need to be updated, the whole list of MBMS Service Area Identities of the affected cell shall be contained in the Served Cell Information IE.

-	If the NPRACH Configuration IE is contained in the Served Cell Information IE in the ENB CONFIGURATION UPDATE message, the eNB receiving the IE may use this information for RACH optimisation.

Update of GU Group Id List:

-	If GU Group Id To Add List IE is contained in the ENB CONFIGURATION UPDATE message, eNB2 shall add the GU Group Id to its GU Group Id List.

-	If GU Group Id To Delete List IE is contained in the ENB CONFIGURATION UPDATE message, eNB2 shall remove the GU Group Id from its GU Group Id List.

------Start of the Next Change------

9.2.8	Served Cell Information

This IE contains cell configuration information of a cell that a neighbour eNB may need for the X2 AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PCI | M |  | INTEGER (0..503, …) | Physical Cell ID | – | – |
| Cell ID | M |  | ECGI 9.2.14 |  | – | – |
| TAC | M |  | OCTET STRING(2) | Tracking Area Code | – | – |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs | – | – |
| >PLMN Identity | M |  | 9.2.4 |  | – | – |
| CHOICE EUTRA-Mode-Info | M |  |  |  | – | – |
| >FDD |  |  |  |  |  |  |
| >>FDD Info |  | 1 |  |  | – | – |
| >>>UL EARFCN | M |  | EARFCN 9.2.26 | Corresponds to NUL in TS 36.104 [16] for E-UTRA operating bands for which it is defined; ignored for E-UTRA operating bands for which NUL is not defined | – | – |
| >>>DL EARFCN | M |  | EARFCN 9.2.26 | Corresponds to NDL in TS 36.104 [16] | – | – |
| >>>UL Transmission Bandwidth | M |  | Transmission Bandwidth 9.2.27 | Same as DL Transmission Bandwidth in this release; ignored in case UL EARFCN value is ignored | – | – |
| >>>DL Transmission Bandwidth | M |  | Transmission Bandwidth 9.2.27 |  | – | – |
| >>>UL EARFCN Extension | O |  | EARFCN Extension 9.2.65 | If this IE is present, the value signalled in the UL EARFCN IE is ignored. | YES | reject |
| >>>DL EARFCN Extension | O |  | EARFCN Extension 9.2.65 | If this IE is present, the value signalled in the DL EARFCN IE is ignored. | YES | reject |
| >>>Offset of NB-IoT Channel Number to DL EARFCN | O |  | Offset of NB-IoT Channel Number to EARFCN 9.2.94 | Corresponds to MDL in TS 36.104 [16] | YES | reject |
| >>>Offset of NB-IoT Channel Number to UL EARFCN | O |  | Offset of NB-IoT Channel Number to EARFCN 9.2.94 | Corresponds to MUL in TS 36.104 [16] | YES | reject |
| >>>NRS-NSSS-PowerOffset | O |  | ENUMERATED (-3, 0, 3, …) | NRS to NSSS power ratio, as defined in TS6.213 [11]. | YES | Ignore |
| >>>NSSS-NumOccasionDifferentPrecoder | O |  | ENUMERATED (2, 4, 8, …) | The number of consecutive NSSS occasions that use different precoders for NSSS transmission, as defined in TS6.213 [11]. | YES | ignore |
| >TDD |  |  |  |  | – | – |
| >>TDD Info |  | 1 |  |  | – | – |
| >>>EARFCN | M |  | 9.2.26 | Corresponds to NDL/NUL in TS 36.104 [16] | – | – |
| >>>Transmission Bandwidth | M |  | Transmission Bandwidth 9.2.27 |  | – | – |
| >>>Subframe Assignment | M |  | ENUMERATED(sa0, sa1, sa2, sa3, sa4, sa5, sa6,…) | Uplink-downlink subframe configuration information defined in TS 36.211 [10]. In NB-IOT, sa0 and sa6 are not applicable. | – | – |
| >>>Special Subframe Info |  | 1 |  | Special subframe configuration information defined in TS 36.211 [10] | – | – |
| >>>>Special Subframe Patterns | M |  | ENUMERATED(ssp0, ssp1, ssp2, ssp3, ssp4, ssp5, ssp6, ssp7, ssp8, …) |  | – | – |
| >>>>Cyclic Prefix DL | M |  | ENUMERATED(Normal, Extended,…) |  | – | – |
| >>>>Cyclic Prefix UL | M |  | ENUMERATED(Normal, Extended,…) |  | – | – |
| >>>Additional Special Subframe Info | O |  |  | Special subframe configuration information defined in TS 36.211 [10]. Only for newly defined configuration of special subframe from Release 11. | YES | ignore |
| >>>>Additional Special Subframe Patterns | M |  | ENUMERATED(ssp0, ssp1, ssp2, ssp3, ssp4, ssp5, ssp6, ssp7, ssp8, ssp9, …) |  | – | – |
| >>>>Cyclic Prefix DL | M |  | ENUMERATED(Normal, Extended,…) |  | – | – |
| >>>>Cyclic Prefix UL | M |  | ENUMERATED(Normal, Extended,…) |  | – | – |
| >>>EARFCN Extension | O |  | 9.2.65 | If this IE is present, the value signalled in the EARFCN IE is ignored. | YES | reject |
| >>>Additional Special Subframe Extension Info | O |  |  | Special subframe configuration information defined in TS 36.211 [10]. Only for newly defined configuration of special subframe from Release 14. | YES | ignore |
| >>>>Additional Special Subframe Patterns Extension | M |  | ENUMERATED(ssp10, …) |  | – | – |
| >>>>Cyclic Prefix DL | M |  | ENUMERATED(Normal, Extended,…) |  | – | – |
| >>>>Cyclic Prefix UL | M |  | ENUMERATED(Normal, Extended,…) |  | – | – |
| Number of Antenna Ports | O |  | 9.2.43 |  | YES | ignore |
| PRACH Configuration | O |  | PRACH Configuration 9.2.50 |  | YES | ignore |
| MBSFN Subframe Info |  | 0..<maxnoofMBSFN> |  | MBSFN subframe defined in TS 36.331 [9] | GLOBAL | ignore |
| >Radioframe Allocation Period | M |  | ENUMERATED(n1, n2, n4, n8, n16, n32, …) |  | – | – |
| >Radioframe Allocation Offset | M |  | INTEGER (0..7, ...) |  | – | – |
| >Subframe Allocation | M |  | 9.2.51 |  | – | – |
| CSG ID | O |  | 9.2.53 |  | YES | ignore |
| MBMS Service Area Identity List |  | 0..<maxnoofMBMSServiceAreaIdentities > |  | Supported MBMS Service Area Identities in the cell | GLOBAL | ignore |
| >MBMS Service Area Identity |  |  | OCTET STRING(2) | MBMS Service Area Identities as defined in TS 23.003 [29] |  |  |
| MultibandInfoList | O |  | 9.2.60 |  | YES | ignore |
| FreqBandIndicatorPriority | O |  | ENUMERATED (not-broadcasted, broadcasted, ...) | This IE indicates that the eNodeB supports FreqBandIndicationPriority, and whether FreqBandIndicatorPriority is broadcasted in SIB 1 (see TS 36.331 [9]) | YES | ignore |
| BandwidthReducedSI | O |  | ENUMERATED (scheduled, ...) | This IE indicates that the SystemInformationBlockType1-BR is scheduled in the cell (see TS 36.331 [9]) | YES | ignore |
| Protected E-UTRA Resource Indication | O |  | 9.2.125 | This IE indicates which E-UTRA control/reference signal resources are protected and are not subject to E-UTRA - NR Cell Resource Coordination. | YES | ignore |
| NPRACH Configuration | O |  | NPRACH Configuration 9.2.xxx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofMBSFN | Maximum no. of MBSFN frame allocation with different offset. Value is 8. |
| maxnoofMBMSServiceAreaIdentities | Maximum no. of MBMS Service Area Identities. Value is 256. |

------Start of the Next Change------

9.2.xxx	NPRACH Configuration

This IE indicates the NPRACH Configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Anchor Carrier NPRACH Configuration | M |  | NPRACH Parameters List 9.2.yyy |  | YES | ignore |
| Non-Anchor-Carrier NPRACH Configuration List |  | 0..<maxnoofNonAnchorCarriers> |  |  | GLOBAL | ignore |
| > Non Anchor Carrier NPRACH Configuration | M |  | NPRACH Parameters List 9.2.yyy |  | - | - |
| > CHOICE Non Anchor Carrier Info |  |  |  |  |  |  |
| >>FDD |  |  |  |  |  |  |
| >>> Non Anchor Carrier UL EARFCN | M |  | EARFCN 9.2.26 |  | - | - |
| >>> Non Anchor Carrier UL EARFCN Extension | M |  | EARFCN Extension 9.2.65 |  | - | - |
| >>> Offset of NB-IoT Channel Number to UL EARFCN | M |  | Offset of NB-IoT Channel Number to EARFCN 9.2.94 | Corresponds to MUL in TS 36.104 [16] | - | - |
| >>TDD |  |  |  |  |  |  |
| >>> Non Anchor Carrier DL EARFCN | M |  | EARFCN 9.2.26 |  | - | - |
| >>> Non Anchor Carrier DL EARFCN Extension | M |  | EARFCN Extension 9.2.65 |  | - | - |
| >>> Offset of NB-IoT Channel Number to DL EARFCN | M |  | Offset of NB-IoT Channel Number to EARFCN 9.2.94 | Corresponds to MUL in TS 36.104 [16] | - | - |
| >>> NB-IoT UL DL Alignment Offset |  |  | NB-IoT UL DL Alignment Offset 9.2.bbb //to be introduced in CR### | Corresponds to the TDD-UL-DL-AlignmentOffset-NB in TS 36.331 [9]. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofNonAnchorCarriers | Maximum no. of Non Anchor Carriers of an anchor carrier. Value is 15. |

9.2.yyy	NPRACH Parameters List

This IE indicates the NPRACH parameters for the NPRACH Resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NPRACH Parameters Item |  | 1 .. <maxnoofNPRACH-Resources> |  |  |  |  |
| >NPRACH Periodicity | M |  | ENUMERATED (ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560, ms5120, ms10240, …) | Periodicity of a NPRACH resource. Unit in millisecond. See TS 36.331 [9]. | - | - |
| >NPRACH Start Time | M |  | ENUMERATED (ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024, ms10, ms20, ms40, ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, …) | Start time of the NPRACH resource in one period. Unit in millisecond. See TS 36.331 [9]. | - | - |
| >NPRACH Subcarrier Offset | M |  | ENUMERATED (n0, n12, n24, n36, n2, n18, n34, n72, n108, n6, n54, n102, n42, n78, n90, n12, n24, n48, n84, n60, n18, …) | Frequency location of the NPRACH resource. In number of subcarriers, offset from sub-carrier 0. See TS 36.331 [9]. | - | - |
| >NPRACH Num Subcarriers | M |  | ENUMERATED (n12, n24, n36, n48, n72, n108, n144, …) | Number of sub-carriers in a NPRACH resource. In number of subcarriers. See TS 36.331 [9]. | - | - |
| >Num Repetitions Per Preamble Attempt | M |  | ENUMERATED (n1, n2, n4, n8, n16, n32, n64, n128, n256, n512, n1024, …) | Number of NPRACH repetitions per attempt for each NPRACH resource, See TS 36.331 [9]. | - | - |
| >NPRACH Num CBRA Start Subcarriers | M |  | ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24, n30, n32, n33, n34, n35, n36, n40, n44, n46, n48, n60, n66, n69, n72, n96, n102, n105, n108, n120, n132, n138, n144, …} | The number of start subcarriers from which a UE can randomly select a start subcarrier, See TS 36.331 [9]. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofNPRACH-Resources | Maximum no. of NPRACH-Resources. Value is 3. |

------Start of the Next Change------

Asn.1 to be added.

------End of the Changes------