3GPP TSG-RAN WG3 Meeting #105bis	R3-195285

Chongqing, China, 14th -18th  October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR |  | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for SON BL CR on 36.413)Support of Inter-system unnecessary HO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-09-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently,there is no support of Inter-system unnecessary HO |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Source eNB to Target eNB Transparent Container, introduce Inter-system mobility measurement configuration IE to support inter-system unnecessary HO. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the SON function. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | inter-system unnecessary handove could not be suppored in REL-16 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.7,9.2.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.300 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----start of the change----

9.2.1.7	Source eNB to Target eNB Transparent Container

The Source eNB to target eNB Transparent Container IE is an information element that is produced by the source eNB and is transmitted to the target eNB. For inter-system handovers to E-UTRAN, the IE is transmitted from the external handover source to the target eNB.

This IE is transparent to the EPC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING | Includes the RRC Handover Preparation Information message as defined in subclause 10.2.2 of TS 36.331 [16]. | - |  |
| E-RABs Information List |  | 0..1 |  |  | - |  |
| >E-RABs Information Item |  | 1 .. <maxnoof E-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>DL Forwarding | O |  | 9.2.3.14 |  | - |  |
| Target Cell ID | M |  | E-UTRAN CGI 9.2.1.38 |  | - |  |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | - |  |
| UE History Information | M |  | 9.2.1.42 |  | - |  |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the external handover source provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | VisitedCellInfoList contained in the UEInformationResponse message (TS 36.331 [16]) | YES | ignore |
| Inter-system measurement configuration | O |  | 9.2.1.X |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

-----Next change----

9.2.1.X	Inter-system Measurement Configuration

The Inter-system Measurement Configuration IE contains information for instructing the incoming UE to continue measuring the cells of the source system after a successful inter-system handover. It is used by the source system to specify the minimum radio quality and the period of time measurements should last for triggering a HO report for unnecessary HO to another system. A subset of source system frequencies to measure and its measurement time configuration together with beam level RSRP and number of beams which is used to derive the cell quality for RRM may be specified. The Inter-system Measurement Configuration IE shall contain at least one of the RSRP or RSRQ thresholds. If only one of the thresholds is present, the node in the target system will use the present threshold to compare against the measurement results received from the UE. HO Report should be sent if there is either a single source system cell whose measurement results exceed the threshold for the whole measurement duration, or a group of source system cells together provide such coverage. The cells that exceed the threshold in the first UE measurement report are included in the HO Report. If both thresholds are present, the received radio measurements must exceed both the RSRP and the RSRQ thresholds in order to satisfy the indicated radio conditions.

When the HO Report is sent from E-UTRAN node at the end of the configured measurement duration, it shall set the HO Report Type IE to "Unnecessary HO to another system"..

No HO Report shall be sent in case no NG-RAN node cell could be included, or if the indicated period of time is interrupted by an inter-system handover to a system different than 5GS or by an intra-EPS handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Inter-system Measurement Parameters | M |  |  |  | - | - |
| >Measurement Duration | M |  | INTEGER (1..100) | The period of time following the successful IRAT handover, during which the target RAT instructs the UE to measure cells of the source RAT. Unit: [second]. | YES | ignore |
| >E-UTRA frequencies |  | 0 to <maxnoofEUTRAFreqs> |  | If present, designates the specific E-UTRAN frequencies which the target RAT may instruct the UE to measure. |  |  |
| >>E-ARFCN | M |  | INTEGER (0..65535) | EARFCN of the downlink carrier frequency TS 36.101 [58]. | YES | ignore |
| >>Measurement Bandwidth | O |  | ENUMERATED(6,15,25,50,75,100) | Measurement bandwidth of the carrier frequency as defined in TS 25.331 [10]. | YES | ignore |
| >>E-ARFCN-Extended | O |  | INTEGER (65536..262143, …) | EARFCN of the downlink carrier frequency TS 36.101 [58]. If this IE is present, the value signalled in the IE “E-ARFCN” is ignored. | YES | reject |
| >>ThresholdEUTRA | M |  | OCTET STRING | ThresholdEUTRA as defined in 36.331, Threshold to be used in EUTRA measurement report triggering condition | YES | ignore |
| >>triggerQuantity | M |  | ENUMERATED {rsrp, rsrq} |  | YES | ignore |
| >NR frequencies |  | 0 to <maxnoofNRFreqs> |  | If present, designates the specific NR frequencies which the target RAT may instruct the UE to measure. |  |  |
| >>NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) | RF Reference Frequency as defined in TS 38.104 [24], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. | YES | ignore |
| >> Meas Timing Configuration | M |  | OCTET STRING | Contains the MTC-SSB-NR-15 as defined in TS 36.331 [10]. | YES | ignore |
| >> SubcarrierSpacingSSB | M |  | OCTET STRING | SubcarrierSpacingSSB as defind in 36.331.Indicate the subcarrier spacing of the SSB. | YES | ignore |
| >>SSB-ToMeasure | M |  | OCTET STRING | SSB-ToMeasure as defined in 36.331. The set of SS blocks to be measured within the SMTC measurement duration | YES | ignore |
| >> MaxRS-IndexCellQualNR | M |  | INTEGER (1..maxRS-IndexCellQual-r15) | Indicates the maximum number of RS indices to be considered/ averaged to derive the cell quality for RRM.Also defined in 36.331 | YES | ignore |
| >> ThresholdRSNR | M |  | OCTET STRING | ThresholdRSNR as defined in 36.331，List of thresholds for consolidation of L1 measurements per RS index | YES | ignore |
| >>ThresholdNR | M |  | OCTET STRING | ThresholdNR as defined in 36.331, Threshold to be used in NR measurement report triggering condition | YES | ignore |
| >>ReportQuantityNR | M |  | OCTET STRING | ReportQuantityNR as defined in 36.331. | YES | ignore |
| RSRQ Type | O |  |  |  |  |  |
| >All Symbols | M |  | BOOLEAN | TRUE indicates use of all OFDM symbols when performing RSRQ measurements. | YES | ignore |
| >Wide band | M |  | BOOLEAN | TRUE indicates use of a wider bandwidth when performing RSRQ measurements. | YES | ignore |
| RSRQ Extension | O |  | INTEGER (-30..46, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofEUTRAFreqs | Maximum no of EUTRA centre frequencies to measure |
| maxnoofNRFreqs | Maximum no of NR centre frequencies to measure |
| maxRS-IndexCellQual-r15 | Maximum no of RS indices to be considered/ averaged to derive the cell quality for RRM. Its value is16 |

ANS.1 to be added

-----End of change----