3GPP TSG-RAN WG3 Meeting #105-Bis	R3-195287

# Chongqing, china, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR |  | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for SON BL CR on 38.413)Support of Inter-system unnecessary HO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-09-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently,there is no support of transfering HO report via NG interface which is a popular scenario of Inter-system Unnecessary HO |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce HO report transfer between NG-RAN nodes via AMF to support Inter-system Unnecessary HO . Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the SON function. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-system Unnecessary HO could not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2.2，9.3.3.X2，9.3.3.X3，9.3.3.X4，9.3.3.X5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.300 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----First change----

9.2.7.1	UPLINK RAN CONFIGURATION TRANSFER

This message is sent by the NG-RAN node in order to transfer RAN configuration information.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| SON Configuration Transfer | O |  | 9.3.3.6 |  | YES | ignore |
| EN-DC SON Configuration Transfer | O |  | OCTET STRING | Contains the EN-DC SON Configuration Transfer IE as defined in TS 36.413 [16]. | YES | ignore |
| Inter-system SON Configuration Transfer | O |  | 9.3.3.X1 |  | YES | ignore |

9.2.7.2	DOWNLINK RAN CONFIGURATION TRANSFER

This message is sent by the AMF in order to transfer RAN configuration information.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| SON Configuration Transfer | O |  | 9.3.3.6 |  | YES | ignore |
| EN-DC SON Configuration Transfer | O |  | OCTET STRING | Contains the EN-DC SON Configuration Transfer IE as defined in TS 36.413 [16]. | YES | ignore |
| Inter-system SON Configuration Transfer | O |  | 9.3.3.X1 |  | YES | ignore |

9.3.3.X1 Inter-system SON Configuration Transfer

This IE contains the configuration information, used by e.g., SON functionality, transmitted between an NG-RAN node and an eNB and additionally includes the node identifier of the destination of this configuration information and the node identifier of the source of this information.

Editors’s note: the details in the IE below are all FFS

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-system SON Configuration Transfer |  |  |  |  |
| >CHOICE Transfer Type | M |  |  |  |
| >>from E-TURAN to NR |  |  |  |  |
| >>>Source eNB-ID |  | 1 |  |  |
| >>>>Global eNB ID | M |  | FFS |  |
| >>>>Selected TAI | M |  | FFS |  |
| >>>Target gNB-ID |  | 1 |  |  |
| >>>>Global RAN node ID | M |  | 9.3.1.5 |  |
| >>>>Selected TAI | M |  | FFS |  |
| >>from NR to E-TURAN |  |  |  |  |
| >>>Source gNB-ID |  | 1 |  |  |
| >>>>Global RAN node ID | M |  | 9.3.1.5 |  |
| >>>>Selected TAI | M |  | FFS |  |
| >>>Target eNB-ID |  | 1 |  |  |
| >>>>Global eNB ID | M |  | FFS |  |
| >>>>Selected TAI | M |  | FFS |  |
| >Inter-system SON Information | M |  | 9.3.3.X2 |  |

9.3.3.X2	Inter-system SON Information

This IE identifies the nature of the configuration information transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information | M |  |  |  |
| >Inter-system SON Information Report |  |  |  |  |
| >>Inter-system SON Information Report | M |  | 9.3.3.X3 |  |

9.3.3.X3 Inter-system SON Information Report

This IE contains the configuration information to be transferred between inter-system nodes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Report | M |  |  |  |
| > Inter-system HO Report Information |  |  |  |  |
| >> Inter-system HO Report Information | M |  | Inter-system HO Report Information 9.2.3.X4 |  |

9.2.3.X4 Inter-system HO Report Information

This IE contains the HO report information to be transferred between inter-system nodes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Handover Report Type | M |  | ENUMERATED (Inter-system HO too early, unnecessary HO from 5GS to EPS …) |  |
| Source cell ID | M |  | CELL ID 9.2.3.X5 | source cell for handover procedure |
| Target cell ID | M |  | CELL ID 9.2.3.X5 | target cell for handover procedure |
| Candidate Cell List | O | 1 .. <maxnoofCandidateCells> |  |  |
| > Candidate Cell ID | M |  | CELL ID 9.2.3.X5 | This IE contains an E-UTRAN CGI or NR CGI. |
| Candidate E-UTRAN PCI List | O | 1 .. <maxnoofCandidateCells> |  |  |
| >Candidate E-UTRAN PCI | M |  | INTEGER (0..503) | This IE includes the Primary Cell Identifier of detected cells not included in the Candidate Cell List IE and for which an E-UTRAN CGI could not be derived. |
| Candidate NR PCI List | O | 1 .. <maxnoofCandidateCells> |  |  |
| >Candidate NR PCI | M |  | INTEGER (0..1007) | This IE includes the Primary Cell Identifier of detected cells not included in the Candidate Cell List IE and for which an NG CGI could not be derived. |
| >Candidate NR ARFCN | M |  | INTEGER (0.. maxNARFCN) | RF Reference Frequency as defined in TS 38.104 [24], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of candidate cells. |
| maxNARFCN | Maximum value of NR carrier frequency, defined in TS 38.331 |

9.2.3.X5 CELL ID

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Cell ID | M |  |  |  |
| >E-UTRAN |  |  |  |  |
| >>Cell Identifier | M |  | E-UTRA CGI 9.3.1.9 |  |
| >NR |  |  |  |  |
| >>Cell Identifier | M |  | NR CGI 9.3.1.7 |  |

-----End of change----