3GPP TSG-RAN WG3 Meeting #105bis	R3-195288

# Chongqing, China, 14th -18th  October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 179 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for SON BL CR on 38.423)Support of Inter-system pingpong |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current baseline CR,inter-system pingpong could not be supported completely. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - add another value inter-system ping-pong in Ho Report Type IE. - Change the presence of Failure cell NR CGI IE - Add target NR cell NR CGI IE - Add description on inter-system ping-pong Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the SON function. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-system ping-pong could not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.Y,9.1.3.Y,9.2.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR .. 38.300 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----First change----

8.4.y	Handover Report

8.4.y.1	General

The purpose of the Handover Report procedure is to transfer mobility related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

8.4.y.2	Successful Operation

Figure 8.4.y.2-1: Handover Report, successful operation

An NG-RAN node initiates the procedure by sending an HANDOVER REPORT message to another NG-RAN node. By sending the message NG-RAN node1 indicates to NG-RAN node2 that a mobility-related problem was detected.

If the Handover Report Type IE is set to "HO too early" or "HO to wrong cell", then the NG-RAN node1 indicates to NG-RAN node2 that, following a successful handover from a cell of NG-RAN node2 to a cell of NG-RAN node1, a radio link failure occurred and the UE attempted RRC Re-establishment either at the original cell of NG-RAN node2 (Handover Too Early), or at another cell (Handover to Wrong Cell). The detection of Handover Too Early and Handover to Wrong Cell events is made according to TS 38.300 [9].

If the UE-related information is available in NG-RAN node1, the NG-RAN node1 should include in HANDOVER REPORT message:

-	the Mobility Information IE, if the Mobility Information IE was sent for this handover from NG-RAN node2;

-	the Source cell C-RNTI IE.

If received, the NG-RAN node2 uses the above information according to TS 38.300 [15].

If the UE RLF Report received from the NG-RAN node sending the RLF INDICATION message, as described in TS 38.300 [15], is available, the NG-RAN node1 may also include it in the HANDOVER REPORT as UE RLF Report Container IE.

If the Handover Report Type IE is set to "Inter-system ping-pong", then the NG-RAN node1 indicates to NG-RAN node2 that a completed handover from a cell of NG-RAN node2 to a cell in other system might have resulted in an inter-system ping-pong and the UE was successfully handed over to a cell of NG-RAN node1 (indicated with Target cell ID2 IE).

The report contains the source and target cells, and cause of the handover. If the Handover Report Type IE is set to "HO to wrong cell", then the Re-establishment cell ECGI IE shall be included in the HANDOVER REPORT message. If the Handover Report Type IE is set to "Inter-system ping-pong", then the Target cell NG CGI IE and Target cell in E-UTRAN  IEshall be included in the HANDOVER REPORT message.

8.4.y.3	Unsuccessful Operation

Not applicable.

8.4.y.4	Abnormal Conditions

Void.

-----Next change----

#### 9.1.3.y	HANDOVER REPORT

Editor’s note: The content of this section is FFS

This message is sent by the NG-RAN node1 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system Pingpong…) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2 | YES | ignore |
| Source cell NR CGI | M |  | NR CGI 9.2.2.7 | NR CGI of source cell for handover procedure (in NG-RAN node 2) | YES | ignore |
| Failure cell NR CGI | C- ifHandoverReportType HoToWrongCellaAnd HoTooearly |  | NR CGI 9.2.2.7 | NR CGI of target cell for handover procedure (in NG-RAN node 1) | YES | ignore |
| Target cell NG CGI | C- ifHandoverReportType Intersystempingpong |  | NR CGI 9.2.2.7 | NR CGI of tartget cell for the first handover in case of inter-system pingpong | YES | ignore |
| Re-establishment cell NR CGI | C- ifHandoverReportType HoToWrongCell |  | NR CGI 9.2.2.7 | NR CGI of cell where UE attempted re-establishment | YES | ignore |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | OCTET STRING | Encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in in TS 36.413 [31] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message from NG-RAN node 2. | YES | ignore |
| UE RLF Report Container | O |  | OCTET STRING | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifHandoverReportType Intersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |
| ifHandoverReportType HoToWrongCellAndHoTooEarly | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" or “HO too early” |

9.2.2.X NG-RAN node CELL ID

This IE is used to globally identify an NG cell (see TS 38.300 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Cell ID | M |  |  |  |
| >E-UTRAN |  |  |  |  |
| >>Cell Identifier | M |  | E-UTRA CGI 9.2.2.8 |  |
| >NR |  |  |  |  |
| >>Cell Identifier | M |  | NR CGI 9.2.2.7 |  |

-----End of change----