3GPP TSG-RAN WG3 Meeting #105bis	R3-195290

# Chongqing, China, 14th -18th  October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR |  | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for SON BL CR on 36.423)Support of Optimization on SN change failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-09-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To support of SN change failure in case of EN-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1)Add SN CHANGE Report procedures in Xn interface This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the SON function. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MRO for SN change failure case could not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.X,9.1.4.X,9.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... 38.300 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----First change----

8.7.X SgNB change Report

8.7.X.1	General

The purpose of the SgNB change Report procedure is to transfer SgNB change failure related information from eNB to en-gNB.

The procedure uses non UE-associated signalling.

8.7.X.2	Successful Operation

Figure 8.7.X.2-1: SgNB Change Report, successful operation

A MeNB initiates the procedure by sending an SgNB CHANGE REPORT message to SgNB node. By sending the message MeNB indicates to SgNB that a SgNB change failure related problem was detected.

If the SN change Report Type IE is set to "SN change too early", then the MeNB indicates to SgNB node that, following a successful SgNB change from a source SgNB to a target SgNB, a SCG failure occurred. Else if the SN change Report Type IE is set to "SN change too late", then the MeNB indicates to SgNB that, following a before SgNB change trigger from a source SgNB to a target SgNB, a SCG failure occurred. Else if the SN change Report Type IE is set to " SN change to wrong cell ", then the MeNB indicates to SgNB that, following a successful SgNB change from a source SgNB node to a target SgNB, a SCG failure occurred.

8.7.X.3 Abnormal Conditions

void

-----Next change----

9.1.4.X SGNB CHANGE REPORT

This message is sent by the MeNB to report a SgNB change failure event to SgNB node.

Direction: MeNB  SgNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| SN change Report Type | M |  | ENUMERATED (SN change too early, SN change too late,SN change to wrong cell, …, ) |  | YES | ignore |
| Source SN cell CGI | M |  | NG-RAN CGI 9.2.111 | CGI of source SgNB for SN change procedure | YES | ignore |
| Failure SN cell CGI | M |  | NG-RAN CGI 9.2.111 | CGI of target SgNB node for SN change procedure | YES | ignore |
| Selected SN cell CGI | C- ifSNchangereportType HoToWrongCell |  | NG-RAN node CELL ID 9.2.2.X | CGI of cell which is the proper target Pscell after SN change failure |  |  |
| UE SCGfailureinformation Report Container | O |  | OCTET STRING | The UE SCGfailureinformation Report Container IE received from UE the SCG failure information message. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifSN changeReportType HoToWrongCell | This IE shall be present if the SN change Report Type IE is set to the value "SN change to wrong cell" |

9.2.X NG-RAN node CELL ID

This IE is used to globally identify an NG cell (see TS 38.300 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Cell ID | M |  |  |  |
| >E-UTRAN |  |  |  |  |
| >>Cell Identifier | M |  | ECGI 9.2.14 |  |
| >NR |  |  |  |  |
| >>Cell Identifier | M |  | NR CGI 9.2.111 |  |

-----End of change----