3GPP TSG-RAN WG3 Meeting #105bis	R3-195296

# Chongqing, China, 14th – 18th October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | <CR#> | rev | <Rev#> | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for SON BL CR on 38.300) Support on MLB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The functionality of Mobility Load Balancing is agreed to be introduced for R16 SON. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the MLB. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The desciption of support of MLB is added as a new section in TS38.300. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mobility Load Balancing functionality can not be suppored in R16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15,15.X,15.X.1,15.X.2,15.X.3,15.X.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS many CR N/A |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

////////////////////////////////////////////////////////////////////////skip irrelevant text////////////////////////////////////////////////////////////////////////

15.X	Support for Mobility Load Balancing

15.X.1	General

The objective of load balancing is to distribute cell load evenly among cells or to transfer part of the traffic from congested cells. This is done by the means of self-optimisation of mobility parameters or handover actions.

Self-optimisation of the intra-system mobility parameters to the current load in the cell and in the adjacent cells can improve the system capacity compared to static/non-optimised cell reselection/handover parameters. Such optimisation can also minimize human intervention in the network management and optimization tasks.

Support for mobility load balancing consists of one or more of following functions:

-	Load reporting;

-	Load balancing action based on handovers;

-	Adapting handover and/or reselection configuration.

Triggering of each of these functions is optional and depends on implementation. Functional architecture is presented in Figure 15.X.1-1.

Figure 15.X.1-1: Functional architecture of SON load balancing

15.X.2	Load reporting

The load reporting function is executed by exchanging cell- or band-specific load information between neighbour NG-RAN nodes over the Xn interface (intra-system scenario) or reporting cell or band specific load information over the X2 interface (EN-DC scenario, from SgNB to MeNB) , F1 interface (from gNB-DU to gNB-CU) or E1 interface (from gNB-CU-UP to gNB-CU-CP).

In addition to cell- or band-specific of a given cell which the reporter controls, cell- or band-specific load information of neighbouring cells can also be exchanged in order to facilitate MLB in MR-DC scenarios. For EN-DC, such information is delivered over X2 interface between the eNBs which are capable to be configured as MeNBs for UEs. For MR-DC with 5GC, such information is delivered over Xn interface between the NG-RAN nodes which are capable to be configured as M-NG-RAN nodes for UEs.

15.X.2.1	Load reporting for intra-system scenario

The load information consists of:

-	radio resource usage (DL/UL/SUL GBR PRB usage, DL/UL/SUL non-GBR PRB usage, DL/UL/SUL total PRB usage);

-	TNL load indicator (UL/DL /SUL TNL load: low, mid, high, overload);

-	Cell Capacity Class value (UL/DL relative capacity indicator);

15.X.3	Load balancing action based on handovers

The source cell may initiate handover due to load (see sub-clauses 9.2.3). The target cell performs admission control for the load balancing handovers. A handover preparation related to a mobility load balancing action shall be distinguishable from other handovers, so that the target cell is able to apply appropriate admission control.

15.X.4	Adapting handover and/or reselection configuration

This function enables requesting of a change of handover and/or reselection parameters at target cell. The source cell that initialized the load balancing estimates if it is needed to change mobility configuration in the source and/or target cell. If the amendment is needed, the source cell initializes mobility negotiation procedure toward the target cell.

The source cell informs the target cell about the new mobility setting and provides cause for the change (e.g. load balancing related request). The proposed change is expressed by the means of the difference (delta) between the current and the new values of the handover trigger. The handover trigger is the cell specific offset that corresponds to the threshold at which a cell initialises the handover preparation procedure. Cell reselection configuration may be amended to reflect changes in the HO setting. The target cell responds to the information from the source cell. The allowed delta range for HO trigger parameter may be carried in the failure response message. The source cell should consider the responses before executing the planned change of its mobility setting.

All automatic changes on the HO and/or reselection parameters must be within the range allowed by OAM.

////////////////////////////////////////////////////////////////////////end////////////////////////////////////////////////////////////////////////