3GPP TSG-RAN3 Meeting #105bis	R3-195347

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR |  | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of transport layer address on S1-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Last RAN3# meeting agreed to support the network deployments with IPv6 on S1-U in en-gNB while IPv4 on S1-MME in the eNB in R3-194369. - The MME may include two transport layer addresses of different versions in the Transport Layer Address IE to enable that an en-gNB can select either IPv4 or IPv6 for a bearer. It is not proper to describe the IE of stage 3 specifcation in the stage 2 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the stage 3 IE (Transport Layer Address IE) from stage 2 specification. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the stage2 descriptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 descriptions are included in the stage 2 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Change begins>>>>>>>>>>>>>>>>>>>>

4.1	Functional Split

The eNB hosts the following functions:

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and sidelink (scheduling);

-	IP header compression, uplink data decompression and encryption of user data stream;

-	Selection of an MME at UE attachment when no routing to an MME can be determined from the information provided by the UE;

-	Routing of User Plane data towards Serving Gateway;

-	Scheduling and transmission of paging messages (originated from the MME);

-	Scheduling and transmission of broadcast information (originated from the MME or O&M);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Scheduling and transmission of PWS (which includes ETWS and CMAS) messages (originated from the MME);

-	CSG handling;

-	Transport level packet marking in the uplink;

-	S-GW relocation without UE mobility, as defined in TS 23.401 [17];

-	SIPTO@LN handling;

-	Maintaining security and radio configuration for User Plane CIoT EPS optimizations, as defined in TS 24.301 [20];

-	Optionally registering with the X2 GW (if used).

The DeNB hosts the following functions in addition to the eNB functions:

-	S1/X2 proxy functionality for supporting RNs;

-	S11 termination and S-GW/P-GW functionality for supporting RNs.

The MME hosts the following functions (see TS 23.401 [17]):

-	NAS signalling;

-	NAS signalling security;

-	AS Security control;

-	Selection of CIoT EPS optimizations (e.g., Control Plane CIoT EPS optimization, as defined in TS 24.301 [20]);

-	Inter CN node signalling for mobility between 3GPP access networks;

-	Idle mode UE Reachability (including control and execution of paging retransmission);

-	Tracking Area list management (for UE in idle and active mode);

-	PDN GW and Serving GW selection;

-	MME selection for handovers with MME change;

-	SGSN selection for handovers to 2G or 3G 3GPP access networks;

-	Roaming;

-	Authentication;

-	Bearer management functions including dedicated bearer establishment. The MME may include two transport layer addresses of different versions to enable that an en-gNB can select either IPv4 or IPv6 for a bearer;

-	Support for PWS (which includes ETWS and CMAS) message transmission;

-	Optionally performing paging optimisation;

-	S-GW relocation without UE mobility, as defined in TS 23.401 [17].

NOTE 1:	The MME should not filter the PAGING message based on the CSG IDs towards macro eNBs

<Unchanged Text Omitted>

<<<<<<<<<<<<<<<<<<<< Change ends>>>>>>>>>>>>>>>>>>>>