3GPP TSG-RAN WG3 Meeting #105bis                   　    　　　　　　	     R3-195406

Chongqing, China, 14 – 18 Oct 2019

Source: 	KDDI Corporation

Title:	Discussion on NR restriction problem

Tdoc Type:	Discussion

Agenda Item: 9.3.4. Others

Document for: Discussion and decision

Release:	Rel-15

Introduction

In this contribution, we raise an issue regarding NR restriction during X2 handover, and propose to start the related RAN3 discussion to develop a solution for the issue.

Discussion

NR restriction issue during X2 handover

During an X2 handover for an EN-DC capable UE without 5G NR subscription, we found an issue when the source eNB is a Rel-14 eNB whereas the target eNB is an EN-DC support Rel-15 eNB. It results in that the target eNB initiates SgNB addition for the EN-DC capable UE even though the UE hasn’t a NR subscription.

In order to state the issue clearly, firstly the success case is shown in the left Figure below in which both source and target are Rel-15 eNBs. The Rel-15 source eNB indicates both EN-DC capable and NR restriction information including in the handover restriction list to the target Rel-15 eNB. Then the target Rel-15 eNB is able to avoid to initiate SgNB addition for the UE.

However in the failure case, shown in the right Figure, the source eNB is assumed to be a Rel-14 eNB. This Rel-14 eNB only indicates EN-DC capable information and ignores NR restriction information including in the handover restriction list to the target Rel-15 eNB. As a result, the target Rel-15 eNB initiates SgNB addition for the UE even though it hasn’t a NR subscription.

Figure: Success and Failure cases for X2 handover (an EN-DC capable UE without 5G NR subscription)

The above issue may be resolved by updating the source eNB to Rel-15. However, from operators’ point of view, there is still a risk that different releases eNBs exist in the network during the updating period which up to individual operator.

Proposal1:

RAN3 acknowledge the above X2 handover issue and start the discussion to develop a 3gpp standard solution

Proposal2:

RAN3 discuss solutions including the following candidates. Any other proposals will be discussed as well.

- Option1: Develop a new S1 procedure which enables the target Rel-15 eNBs to get NR restriction information from MME

- Option2: Add the NR restriction information to PATH SWITCH REQUEST ACKNOWLEDGE so that the target Rel-15 eNBs is able to get NR restriction information from MME

Conclusion

Proposal1:

RAN3 acknowledge the above X2 handover issue and start the discussion to develop a 3gpp standard solution

Proposal2:

RAN3 discuss solutions including the following candidates. Any other proposals will be discussed as well.

- Option1: Develop a new S1 procedure which enables the target Rel-15 eNBs to get NR restriction information from MME

- Option2: Add the NR restriction information to PATH SWITCH REQUEST ACKNOWLEDGE so that the target Rel-15 eNBs is able to get NR restriction information from MME