3GPP TSG-RAN3 Meeting #105bis	R3-195429

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR |  | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for SON BL CR for TS 36.423) MLB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the RAN3#105, RAN3 has the following agreements: - Add RESOURCE STATUS REQUEST/RESPONSE/UPDATE/FAILURE procedures for Xn(for EN-DC) RAN3 need add the IEs in this messages in order to support the MLB |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding load sharing and load balancing optimization function description and IEs in the RESOURCE STATUS REQUEST/RESPONSE/UPDATE/FAILURE procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of load sharing and load balancing optimization can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.x,8.7.y,9.1.4.x1, 9.1.4.x2, 9.1.4.x3, 9.1.4.x4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This TP is changed based on the BL CR R3-194775.The new changes are marked using “Huawei2” |  |  |  |  |  |  |  |  |

Start of the first change

### 8.7.x	EN-DC Resource Status Reporting Initiation

Editor’s note: The content of this section is FFS

#### 8.7.x.1	General

This procedure is used by the MeNB to request the reporting of load measurements to the en-gNB .

The procedure uses non UE-associated signalling.

#### 8.7.x.2	Successful Operation

Figure 8.7.x.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from eNB to en-gNB to start a measurment, stop a measurements, add cells to report for a measurement or remove certain cells from a measurement.

Upon receipt, en-gNB:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	if supported, stop cell measurements and terminate the reporting for cells indicated in the Cell To Report IE list, in case the Registration Request IE is set to "partial stop"; or

-	if supported, add cells indicated in the Cell To Report IE list to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add".

If the en-gNB received a RESOURCE STATUS REQUEST message, which includes the Registration Request IE set to "stop", the Cell To Report IE list shall be ignored.

If the Registration Request IE is set to "start" then the Report Characteristics IE shall be included in RESOURCE STATUS REQUEST message. The en-gNB shall ignore the Report Characteristics IE, if the Registration Request IE is not set to "start".

The Report Characteristics IE indicates the type of objects en-gNB shall perform measurements on. For each cell, the en-gNB shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the TNL Load Indicator IE, if the second bit, "TNL Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Hardware Load Indicator IE, if the third bit, "HW Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the fourth bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE;

If the Reporting Periodicity IE is included in the RESOURCE STATUS REQUEST message, en-gNB shall use its value as the time interval between two subsequent RESOURCE STATUS UPDATE messages that include the Radio Resource Status IE, TNL Load Indicator IE, Hardware Load Indicator IE, Composite Available Capacity Group IE.

If the en-gNB is capable to provide all requested resource status information, it shall initiate the measurement as requested by  the eNB, and respond with the EN-DC RESOURCE STATUS RESPONSE message.

If en-gNB is capable to provide some but not all of the requested resource status information and the Partial Success Indicator IE is present in the RESOURCE STATUS REQUEST message, it shall initiate the measurement for the admitted measurement objects and include the Measurement Initiation Result IE in the RESOURCE STATUS RESPONSE message.

#### 8.7.x.3	Unsuccessful Operation

Figure 8.2.x.3-1: Resource Status Reporting Initiation, unsuccessful operation

If none of the requested measurements can be initiated, gNB-DU shall send a RESOURCE STATUS FAILURE message.

#### 8.7.x.4	Abnormal Conditions

FFS

### 8.7.y	EN-DC Resource Status Reporting

Editor’s note: The content of this section is FFS

#### 8.7.y.1	General

This procedure is initiated by en-gNB to report the result of measurements admitted by en-gNB following a successful EN-DC Resource Status Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.7.y.2	Successful Operation

Figure 8.7.y.2-1: Resource Status Reporting, successful operation

The en-gNB shall report the results of the admitted measurements in EN-DC RESOURCE STATUS UPDATE message. The admitted measurements are the measurements that were successfully initiated during the preceding EN-DC Resource Status Reporting Initiation procedure.

If the eNB receives the RESOURCE STATUS UPDATE message, which includes the Cell Reporting Indicator IE set to "stop request" in one or more items of the Cell Measurement Result IE, the en-gNB should initialise the Resource Status Reporting Initiation procedure to remove all or some of the corresponding cells from the measurement.

#### 8.7.y.3	Unsuccessful Operation

Not applicable.

#### 8.7.y.4	Abnormal Conditions

Void

////////////////////////////////////////////////////////////unchanged omitted///////////////////////////////////////////////////////////////////////

#### 9.1.4.x1	EN-DC RESOURCE STATUS REQUEST

Editor’s note: The content of this section is FFS

This message is sent by an eNB to neighbouring en-gNB to initiate the requested measurement according to the parameters given in the message.

Direction: eNB  en-gNB.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.2.3.1 |  |  |  | YES |  | reject |  |
| MeNB Measurement ID |  | M |  |  |  | INTEGER (1..4095,...) |  | Allocated by MeNB |  | YES |  | reject |  |
| en-gNB Measurement ID |  | C-ifRegistrationRequestStopororAddorPartialStop |  |  |  | INTEGER (1..4095,...) |  | Allocated by en-gNB |  | YES |  | ignore |  |
| Registration Request |  | M |  |  |  | ENUMERATED(start, stop,add, partial stop, …) |  | Type of request for which the resource status is required. |  | YES |  | reject |  |
| Report Characteristics |  | O |  |  |  | BITSTRING (SIZE(32)) |  | Each position in the bitmap indicates measurement object the en-gNB is requested to report. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, this bit should be set to 1 if at least one of the First, Second or Third bits is set to 1 . Other bits shall be ignored by the en-gNB. |  | YES |  | reject |  |
| Cell To Report |  | 1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellinen-gNB > |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.2.111 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Load Ind Periodic, HW Load Ind Periodic, Composite Available Capacity Periodic. | YES | ignore |
| Partial Success Indicator | O |  | ENUMERATED(partial success allowed, ...) | Included if partial success is allowed | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAddorPartialStop | This IE shall be present if the Registration Request IE is set to the value "stop", “partial stop” or "add". |

#### 9.1.4.x2	EN-DC RESOURCE STATUS RESPONSE

Editor’s note: The content of this section is FFS

This message is sent by the en-gNB to indicate that the requested measurement, for all or for a subset of the measurement objects included in the measurement is successfully initiated.

Direction: en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| MeNB Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by MeNB | YES | reject |
| en-gNB Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by en-gNB | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Measurement Initiation Result |  | 0..1 |  | List of all cells in which measurement objects were requested, included when indicating partial success | YES | ignore |
| >Measurement Initiation Result Item |  | 1 .. <maxCellinen-gNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.2.111 |  | – |  |
| >>Measurement Failure Cause List |  | 0..1 |  | Indicates that en-gNB could not initiate the measurement for at least one of the requested measurement objects in the cell | – |  |
| >>>Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>>>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object that failed to be initiated in the en-gNB. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic. Other bits shall be ignored by the MeNB. | – |  |
| >>>>Cause | M |  | 9.2.6 | Failure cause for measurement objects for which the measurement cannot be initiated | – |  |

#### 9.1.4.x3	EN-DC RESOURCE STATUS FAILURE

Editor’s note: The content of this section is FFS

This message is sent by the en-gNB to indicate that for none of the requested measurement objects the measurement can be initiated.

Direction: en-gNB  eNB.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.2.3.1 |  | YES | reject |
| MeNB Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by MeNB | YES | reject |
| en-gNB Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by en-gNB | YES | reject |
| Cause | M |  |  | 9.2.6 | Ignored by the receiver when the Complete Failure Cause Information IE is included | YES | ignore |
| Criticality Diagnostics |  | O |  | 9.2.7 |  | YES | ignore |
| Complete Failure Cause Information |  |  | 0..1 |  | Complete list of failure causes for all requested cells | YES | ignore |
| >Complete Failure Cause Information Item |  |  | 1 .. <maxCellinen-gNB> |  |  | EACH | ignore |
| >>Cell ID |  | M |  | NR CGI 9.2.111 |  | – |  |
| >>Measurement Failure Cause List |  |  | 1 |  |  | – |  |
| >>>Measurement Failure Cause Item |  |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>>>Measurement Failed Report Characteristics |  | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object that failed to be initiated in the en-gNB. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic. Other bits shall be ignored by the MeNB. | – |  |
| >>>>Cause |  | M |  | 9.2.6 | Failure cause for measurements that cannot be initiated | – |  |

#### 9.1.4.x4	EN-DC RESOURCE STATUS UPDATE

Editor’s note: The content of this section is FFS

This message is sent by en-gNB to neighbouring MeNB to report the results of the requested measurements.

Direction: en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| MeNB Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by MeNB | YES | reject |
| en-gNB Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by en-gNB | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Ite`m |  | 1 .. <maxCellinen-gNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.2.111 |  |  |  |
| >>Hardware Load Indicator | O |  | 9.2.34 |  |  |  |
| >>TNL Load Indicator | O |  | 9.2.35 |  |  |  |
| >>Radio Resource Status | O |  | 9.2.37 |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.44 |  | YES | ignore |
| >>Cell Reporting Indicator | O |  | ENUMERATED(stop request, ...) |  | YES | ignore |