3GPP TSG-RAN3 Meeting #105bis	R3-195436

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for SON BL CR for TS 38.300): Inter-system load reporting in MLB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of Inter-system and Inter-RAT load reporting in MLB |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NR related description on Inter-RAT SON Information. This CR is based on the agreed BL CR R3-194776 which added some MLB description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of Inter-system load reporting in MLB function between NR and LTE is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.300 CR TS 36.413 CR 1710 TS 36.423 CR 1373 TS 38.423 CR 0221 TS 38.463 CR 0142 TS 38.473 CR 0441 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

## 15.X	Self-optimisation

### 15.X.1	Support for Mobility Load Balancing

Editor’s note: This section captures the stage 2 descriptions for Mobility Load Balancing

#### 15.X.1.1	General

Editor’s note: the content of this section is FFS.

The objective of mobility load balancing is to distribute cell load evenly among cells or to transfer part of the traffic from congested cell, or to offload users from one cell or carrier or RAT to achieve network energy saving, This can be done by means of optimization of cell reselection/handover parameters and handover actions. The automation of such optimisation can provide high quality user experience, while simultaneously improving the system capacity and also to minimize human intervention in the network management and optimization tasks.

Both intra-RAT and intra-system inter-RAT load balancing scenarios should be supported.

In general, support for mobility load balancing consists of one or more of following functions:

-	 Load reporting;

- Load balancing action based on handovers;

-	 Adapting handover and/or reselection configuration.

#### 15.X.1.2 Load reporting

Editor’s note: the content of this section is FFS.

The load reporting function is executed by exchanging load information over the NG/S1/Xn/X2/F1/E1 interfaces.

Editor’s note: load related information and its definition are FFS.

15.X.1.2.1 	Load reporting for Intra-system

The following load related information should be supported which consists of,

-	Radio resource usage (DL/UL/SUL GBR PRB usage, DL/UL/SUL non-GBR PRB usage, DL/UL/SUL total PRB usage);

-	TNL load indicator (UL/DL /SUL TNL load: low, mid, high, overload);

-	Cell Capacity Class value (UL/DL relative capacity indicator);

To achieve load reporting function, Resource Status Reporting Initiation & Resource Status Reporting procedures are used.

15.X.1.2.2 	Load reporting for Inter-system

The following load related information should be supported which consists of,

-	Cell Capacity Class value (UL/DL relative capacity indicator:  the same scale shall apply to E-UTRAN and NG-RAN cells when mapping cell capacities on this value);

To achieve load reporting function, Downlink RAN Configuration Transfer and Uplink RAN Configuration Transfer procedures are used.

End of the next change