3GPP TSG-RAN3 Meeting #105bis	R3-195438

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR |  | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.413 for Inter-RAT and Inter-system load reporting in MLB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of Inter-RAT and Inter-system load reporting in MLB |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the agreed CR R3-194769 :Addition of a new Inter-system SON Configuration Transfer IE in the DL and UL configuration transfer for inter-ssytem exchange. Add the Inter-system SON Information Request and Inter-system SON Information Reply to Inter-system SON Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of Inter-RAT and Inter-system load reporting in MLB function is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2, 9.3.3.x2, 9.3.3.Y1, 9.3.3.Y2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.300 CR TS 36.413 CR 1710 TS 36.423 CR 1373 TS 38.423 CR 0221 TS 38.463 CR 0142 TS 38.473 CR 0441 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

### 8.8.2	Downlink RAN Configuration Transfer

#### 8.8.2.1	General

The purpose of the Downlink RAN Configuration Transfer procedure is to transfer RAN configuration information from the AMF to the NG-RAN node. This procedure uses non-UE associated signalling.

#### 8.8.2.2	Successful Operation

Figure 8.8.2.2-1: Downlink RAN configuration transfer

The procedure is initiated with an DOWNLINK RAN CONFIGURATION TRANSFER message sent from the AMF to the NG-RAN node.

If the NG-RAN node receives, in the SON Configuration Transfer IE or in the EN-DC SON Configuration Transfer IE, the SON Information IE containing the SON Information Request IE, or in the Inter-system SON Configuration Transfer IE, the Inter-system SON information IE containing the Inter-system SON Information Request IE, it may transfer back the requested information either towards the NG-RAN node indicated in the Source RAN Node ID IE of the SON Configuration Transfer IE or towards an eNB indicated in the Source eNB-ID IE of the EN-DC SON Configuration Transfer IE by initiating the Uplink RAN Configuration Transfer procedure.

If the NG-RAN node receives, in the SON Configuration Transfer IE, the Xn TNL Configuration Info IE containing the Xn Extended Transport Layer Addresses IE, it may use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the NG-RAN node receives, in the SON Configuration Transfer IE, the SON Information IE containing the SON Information Reply IE including the Xn TNL Configuration Info IE as an answer to a former request, it may use it to initiate the Xn TNL establishment.

In case the IP-Sec Transport Layer Address IE is present and the GTP Transport Layer Addresses IE within the Xn Extended Transport Layer Addresses IE is not empty, GTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in the IP-Sec Transport Layer Address IE.

In case the IP-Sec Transport Layer Address IE is not present, GTP traffic is terminated at the endpoints given by the list of addresses in the Xn GTP Transport Layer Addresses IE within the Xn Extended Transport Layer Addresses IE.

In case the Xn GTP Transport Layer Addresses IE is empty and the IP-Sec Transport Layer Address IE is present, SCTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel endpoint given in the IP-Sec Transport Layer Address IE, within the Xn Extended Transport Layer Addresses IE.

If the NG-RAN node receives the SON Information IE containing the SON Information Report IE it may use it as specified in [FFS].

If the eNB receives the Inter-system SON Information IE containing the Inter-system SON Information Request IE it may use it as specified in [FFS].

If the eNB receives the Inter-system SON Information IE containing the Inter-system SON Information Reply IE it may use it as specified in [FFS].

If the NG-RAN node receives the Inter-system SON Information IE containing the Inter-System SON Information Report IE it may use it as specified in [FFS].

Editor’s note: The scenario of an RLF report generated by NG-RAN RRC being sent from E-UTRAN to NG-RAN is FFS

If the NG-RAN node is configured to use one IPsec tunnel for all NG and Xn traffic (IPsec star topology) then the traffic to the peer NG-RAN node shall be routed through this IPsec tunnel and the IP-Sec Transport Layer Address IE shall be ignored.

#### 8.8.2.3	Abnormal Conditions

Void.

Start of the next change

#### 9.3.3.X1	Inter-system SON Configuration Transfer

This IE contains the configuration information, used by e.g., SON functionality, transmitted between an NG-RAN node and an eNB and additionally includes the node identifier of the destination of this configuration information and the node identifier of the source of this information.

Editors’s note: the details in the IE below are all FFS

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-system SON Configuration Transfer |  |  |  |  |
| >CHOICE Transfer Type | M |  |  |  |
| >>from E-TURAN to NR |  |  |  |  |
| >>>Source eNB-ID |  | 1 |  |  |
| >>>>Global eNB ID | M |  | FFS |  |
| >>>>Selected TAI | M |  | FFS |  |
| >>>Target gNB-ID |  | 1 |  |  |
| >>>>Global RAN node ID | M |  | 9.3.1.5 |  |
| >>>>Selected TAI | M |  | FFS |  |
| >>from NR to E-TURAN |  |  |  |  |
| >>>Source gNB-ID |  | 1 |  |  |
| >>>>Global RAN node ID | M |  | 9.3.1.5 |  |
| >>>>Selected TAI | M |  | FFS |  |
| >>>Target eNB-ID |  | 1 |  |  |
| >>>>Global eNB ID | M |  | FFS |  |
| >>>>Selected TAI | M |  | FFS |  |
| >Inter-system SON Information | M |  | 9.3.3.X2 |  |

#### 9.3.3.X2	Inter-system SON Information

This IE identifies the nature of the configuration information transferred, i.e., a request, a reply or a report.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information | M |  |  |  |
| >Inter-system SON Information Request |  |  |  |  |
| >> Inter-system SON Information Request | M |  | 9.3.3.Y1 |  |
| >SON Information Reply |  |  |  |  |
| >> Inter-system SON Information Reply | M |  | 9.3.3.Y2 |  |
| >Inter-system SON Information Report |  |  |  |  |
| >>Inter-system SON Information Report | M |  | FFS |  |

Start of the next change

#### 9.3.3.Y1	Inter-system SON Information Request

This IE identifies the nature of the configuration information transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Request | M |  |  |  |
| >Inter-system load reporting |  |  |  |  |
| >>Cell List |  | 1 |  |  |
| >>> Cell Item |  | 1..<maxnoofCellList> |  |  |
| >>>> Inter System CGI | M |  | FFS |  |

Start of the next change

#### 9.3.3.Y2	Inter-system SON Information Reply

This IE identifies the nature of the configuration information transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Reply | M |  |  |  |
| > Inter-system load reporting |  |  |  |  |
| >>Cell List |  | 1 |  |  |
| >>> Cell Item |  | 1..<maxnoofCellList> |  |  |
| >>>> Inter System CGI | M |  | FFS |  |
| >>>> Conpisite Available Capacity |  |  | FFS |  |

End of the next change