3GPP TSG-RAN3 Meeting #105-Bis	R3-195479

# Chongqing, China, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0252 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UL data forwarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The UL Forwarding IE is optional in tabular and mandatory in ASN.1. The UL forwarding is optional function in handover. So, this IE should be optional. #### 9.2.1.17 Data Forwarding and Offloading Info from source NG-RAN node This IE contains information from a source NG-RAN node regarding per QoS flow proposed data forwarding and offloading. IE/Group Name; Presence; Range; IE type and reference; Semantics description. QoS Flows To Be Forwarded List; 1. >QoS Flows To Be Forwarded Item; 1 .. <maxnoofQoSFlows>. >>QoS Flow Identifier; M; 9.2.3.10. >>DL Forwarding; M; 9.2.3.34. >>UL Forwarding; O; 9.2.3.90. Source DRB to QoS Flow Mapping List; O; DRB to QoS Flow Mapping List 9.2.1.15. QoSFLowsToBeForwarded-Item ::= SEQUENCE { qosFlowIdentifier QoSFlowIdentifier, dl-dataforwarding DLForwarding, ul-dataforwarding ULForwarding, iE-Extension ProtocolExtensionContainer { {QoSFLowsToBeForwarded-Item-ExtIEs} } OPTIONAL, ... } There are three options to align them. - Changing ASN.1 to optional This is non-backward compatible, but is also the cleanest fix, if the change can be accepted. The IE is used for non-standalone deployment. - Changing tabular to M This might be OK as the result is that tunnels are set up, but the proposer does not have to use the tunnels. This is not an efficient way. - Extend the enumeration in ASN.1 This is strictly backward compatible. But functionally anyway an old node receiving this value does not understand it. So, there is no any advantage of taking this route. Option 1 is adopted in this CR. |
| Summary of change: |  | Change ul-dataforwarding to optional to align with tabular. Impact Analysis: Impacted 5G architecture option: NR-SA Impacted functionality: Handover Inter-operability: - This change is not backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL forwarding IE in inconsistent in tabular and ASN.1. If the product is implemented per ASN.1, an unnecessary tunnel for UL data forwarding may be established. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

QoSFLowsToBeForwarded-List ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoSFLowsToBeForwarded-Item

QoSFLowsToBeForwarded-Item ::= SEQUENCE {

qosFlowIdentifier			QoSFlowIdentifier,

dl-dataforwarding			DLForwarding,

ul-dataforwarding			ULForwarding										OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {QoSFLowsToBeForwarded-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFLowsToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}