3GPP TSG-RAN WG3 Meeting #105bis 	R3-195522

# Chongqing, China, 14-18 October 2019

Agenda item:	9.3.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on the need for multiple SCTP associations in X2

Document for:	Discussion

# 1	Introduction

The previous RAN3 meetings have agreed multiple SCTP support for all interfaces of the gNB-CU-CP, except the X2 interface for EN-DC. During the discussion on the multiple SCTP for X2 [1][2] in previous meetings, there were comments that X2 interface may need to be postponed, especially due to limited time for defining it in Rel-15.

In this contribution, we would like to re-open the discussion and we re-iterate on the importance of having a consistent node architecture in 5G, where all interfaces of the CU-CP should be aligned in the support of multiple SCTP associations, so that efficient virtualized and geo-redundant gNB-CU-CP deployments are possible.

# 2	Discussion

The arguments to use multiple SCTP associations for signalling, for a pair of nodes were related mainly to geo-redundancy, i.e. ability to scale the capacity more effectively. It is particularly beneficial when adding new capacity enhancements for the gNB-CU-CP, each with different SCTP Endpoints (i.e. different IP address, or same IP address with different port numbers). NG, F1, E1 and Xn enable using a separate SCTP association per each capacity enhancement. The current support for multiple SCTP support in RAN architecture is shown as below:

Figure 1: Overall architecture and Multiple-SCTP support

However, X2 still require a single association, thus forcing the suboptimal solutions, e.g. listed in [3]:

- internal routing in the gNB-CU-CP; or

- each “CU-CP instance” establishes its own E1 interface toward a different gNB-CU-UP.

Using any of those solutions will effectively limit benefits from having multiple SCTP associations over NG, F1, E1 and Xn.

Observation 1: not supporting multiple SCTP over X2 will effectively limit benefits from having multiple SCTP associations over NG, F1, E1 and Xn.

In addition, last RAN3 meeting has agreed to use dynamic port for additional SCTP association for NG, F1, E1, and Xn [2][3][4][5]. This enables the NG, F1, E1 and Xn interface sharing the same IP address, e.g. adr1, whenever new capacity enhancement for CU-CP capacity is added. A separate SCTP association using the same IP address (e.g. adr1), but different port number can be used for NG, F1, E1 and Xn. However, without the support of multiple SCTP for X2 interface, the dynamic port support will be useless. Current X2 only support the SCTP association using the IANA port number. This force the CU-CP to still have many different IP addresses, rather only use one IP address (e.g. adr1).

Observation 2: not supporting multiple SCTP over EN-DC X2 will effectively eliminate benefits from dynamic port support.

Based on above analysis, we proposed to support multiple SCTP for X2 interface, which enables all interfaces of the CU-CP be aligned in the support of multiple SCTP associations, so that efficient virtualized and geo-redundant gNB-CU-CP deployments are possible. The en-gNB can add/update/remove SCTP endpoints, but the eNB still only have one SCTP endpoint for EN-DC X2 interface.

Proposal: RAN3 agree multiple SCTP support for EN-DC X2 interface. The en-gNB can add/update/remove SCTP endpoints.

# 3	Conclusion

Observation 1: not supporting multiple SCTP over X2 will effectively limit benefits from having multiple SCTP associations over NG, F1, E1 and Xn.

Observation 2: not supporting multiple SCTP over EN-DC X2 will effectively eliminate benefits from dynamic port support.

Proposal: RAN3 agree multiple SCTP support for EN-DC X2 interface. The en-gNB can add/update/remove SCTP endpoints.

Stage-2 CR can be found in [8], and Stage-3 CR can be found in [9].

# 4	References

- R3-185495, Considerations on the need for multiple SCTP associations in X2 and Xn

- R3-185496, Enabling multiple SCTP associations (36.423 CR)

- R3-184306, E1 multiple SCTP associations - for 38.463

- 3GPP TS38.412

- 3GPP TS38.422

- 3GPP TS38.462

- 3GPP TS38.472

- R3-195523, CR for TS36.422

- R3-195524, CR for TS36.423