3GPP TSG-WG3 Meeting #105bis																R3-195530

Chongqing, China, 14-18 October 2019

Agenda item:	13.2.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on DL Forwarding Table Configuration

Document for:	Discussion and Decision

# 1	Introduction

The previous meeting did not conclude on whether uses UE-Associated signalling, or non-UE-Associated signalling to configure the DL Forwarding table. This contribution provides the further analysis.

# 2	Discussion

Regrading on how to configure the forwarding table in the Donor-DU and IAB node, there may be two options. Let’s consider following IAB, and consider the DL forwarding table in IAB11 as an example,

Figure 1: multiple-hop example

- Option 1: the forwarding table is considered as part the context of the IAB-MT’s serving DU

For example, the forwarding table using IAB21 as the next node is considering as the context of IAB21-MT’s serving DU (i.e. IAB11-DU). IAB11-DU maintains one forwarding table with entries using its child IABs as next node, e.g. {[dest=IAB31, next hop=IAB21]; [dest=IAB32, next hop=IAB21], [dest=IAB31, next hop=IAB22]}

In this option, Donor-CU can use the non-UE-associated F1AP procedure to manage the forwarding table.

- Option 2: the forwarding table is considered as part of the IAB-MT’s context in its serving DU, i.e. IAB-DU or Donor-DU

For example, in IAB11-DU, the IAB21-MT’s context has a forwarding table {[dest=IAB31, next hop=IAB21]; [dest=IAB32, next hop=IAB21]}. This means any forwarding using IAB21 as the next node will be part of IAB21-MT’s context in the IAB21-MT’s serving DU (i.e. IAB11-DU). In IAB11-DU, there are multiple forwarding tables, i.e. one per child IAB node.

In this option, Donor-CU uses UE-associated F1AP procedure, e.g. F1AP UE Context management procedure, to manage the forwarding table.

Both options work for DL. But in some scenarios, e.g. with redundant path, the Donor-CU may need to initiate 1 more additional procedures in Option 2 than Option 1. For example, when configure the dest=IAB31 in IAB11, CU initiates one UE-associated procedure to configure the forwarding table {[dest=IAB31, next hop=IAB21]}for IAB21-MT’s context. The CU initiates another UE-associated procedure to configure the forwarding table [dest=IAB31, next hop=IAB22] for IAB22-MT’s context.

In addition, Option 1 is efficient in case of the Topology Adaptation. In above example, when the IAB32 performs a Topology Adaptation, i.e. the connection is changed from IAB21 to IAB22, the Donor-CU only initiates one F1AP procedure to IAB11, to change the forwarding entry [dest=IAB32, next hop=IAB21] to [dest=IAB32, next hop=IAB22].  While in Option 2, the Donor-CU need to initiate 2 F1AP procedures to IAB11. 1) the 1st  F1AP procedure regarding to IAB21’s MT context, i.e. to remove the DL forwarding entry [dest=IAB32, next hop=IAB21]. 2) The Donor-CU initiate the 2nd F1AP procedure regarding to IAB22’s MT context, i.e. to add the DL forwarding entry [dest=IAB32, next hop=IAB22].

In a summary, both options work. Option 1 can save some signalling, and efficient in some scenarios. So we prefer to use non-UE-associated signalling.

Proposal 1: use non-UE-associated F1AP signalling to configure the DL forwarding table.

# 4	Conclusion

In this contribution configuration of DL forwarding table. The following proposals are made:

Proposal 1: use non-UE-associated F1AP signalling to configure the DL forwarding table.

# References

[1]