3GPP TSG-RAN3 Meeting #105bis	R3-195557

Chongqing, China, 14-18 October 2019

Title: 	(TP for MDT BL CR for TS 38.423):MDT in MR-DC

Source: 	Huawei

Agenda item:	10.3.2

Document for:	pCR

1. Introduction

According to the following description [1], RAN2 has agreed some principles of MDT in MR-DC.

#### 5.7.3.5 MDT for dual connectivity ⁻ Immediate MDT configurations are supported for DC scenario. ⁻ Logged MDT configurations can come from SN node in DC scenario. ⁻ The existing MDT framework is the baseline for the SCG cells related MDT configuration. ⁻ The triggers for MDT measurements associated to MCG and SCG are separate. ⁻ MN-SN coordination is required for MDT measurements’ configuration and reporting in DC framework. ⁻ If SRB3 is not configured, SN related measurements are transmitted to MN via SRB1/2 and then forwarded to SN. ⁻ If SRB3 is configured, MN related measurements are transmitted to MN via SRB1/2, SN related measurements are transmitted to SN via SRB3

RAN#84 has agreed the WI [2] on SON/MDT support for NR. The objectives of WI includes the specification of MDT for MR-DC scenario.

- Support of MDT features for identified use cases, including coverage optimization, QoS verification via MDT, indoor MDT improvement, location information reporting, and sensor data collection [RAN2, RAN3] - Specification of Logged MDT for both RRC_IDLE and RRC_INACTIVE UEs [RAN2, RAN3, RAN4] - Specification of Immediate MDT for RRC_CONNECTED UEs[RAN2, RAN3] - Specification of reporting e.g. RLF and accessibility measurements [RAN2, RAN4] - Specification of MDT for standalone, NR-DC and EN-DC scenario including CU-DU split architecture [RAN2, RAN3] - NOTE: NE-DC and NGEN-DC scenarios may be supported if the specifications above cover those without any additional stage 3 specification effort, but W1 specification work for MDT is not included in this WID.

In this contribution, we will discuss the MDT for MR-DC.

2. Discussion

2.1. logged MDT

According to the agreements of RAN2, the SN can configure the logged MDT.

Observation 1: SN can configure the logged MDT

The objective of WI includes the specification of MDT for NR-DC and EN-DC scenario. NE-DC and NGEN-DC scenarios may be supported if the specifications above cover those without any additional stage 3 specification effort.

For the signalling based logged MDT, only the MN will receive the MDT configuration from the CN. In NR-DC, the MN and SN belong to the same RAT. The MN can directly configure the signalling based logged MDT. The MN does not need to send the signalling based MDT configuration to the SN and the signalling based logged MDT configuration from the CN does not need to include the MDT configuration for the SN. In EN-DC, the UE cannot camp on the NR cells. Therefore the SN does not need to configure the logged MDT.

Proposal 1: For the signalling based logged MDT in NR-DC and EN-DC:

- The MN does not send the signalling based logged MDT configuration to the SN.

- In EN-DC, the SN does not configure the logged MDT for the UE.

For the management based logged MDT, the SN can receive the logged MDT configuration from the OAM. Also we think the node should configure the MDT according to its configuration received from the OAM. Therefore the MN does not need to send the management based logged MDT configuration to the SN. In NR-DC, the SN can select and configure the logged MDT according to the configuration received from the OAM. In EN-DC, the UE cannot camp on the NR cells. Therefore the SN does not need to configure the logged MDT.

Proposal 2: For the management based logged MDT:

- In NR-DC, the SN selects the UE to configure the logged MDT according to the configuration received from OAM.

- In EN-DC, the SN does not configure the logged MDT for the UE.

In LTE, the coverage use cases include the coverage mapping of Scell in CA scenario. Scell may be deployed in area where high traffic occurs. The location where a Scell is available to be added may show whether the deployment of Scell is according to the needs of capacity increase. In NR, the operators may want to know the coverage of NR. The reason is that the deployment of SN cell is related to capacity expansion and network planning. Also the operators want to know the coverage of NR SA before deploying the NR SA. Therefore RAN2 may need to consider how to collect the coverage of NR in EN-DC.

In NSA, both the MN and SN can configure the immediate MDT measurement. In general, the network will configure the UE to be in MR-DC when the quality of SN is above one threshold. Therefore the NR SA coverage is different from the MR-DC coverage (as shown in Figure 1). Therefore the operators cannot get the coverage of NR SA.

Figure 1: Different coverage of EN-DC and NR SA

In LTE, the network configures the logged MDT measurement via RRC messages. The UE logs the measurement results of serving cell and the available measurement results of inter-RAT neighbouring cells when UE camps on the LTE cells. In R15, UE cannot camp on the NR cell in NSA. Therefore UE will not perform the logged MDT configuration from SN. In our understanding, the MN can configure the logged MDT measurement of SN and the UE can measure the NR cells like the early measurement reporting. Then the operator can get more data about the coverage of SN and get the coverage of NR SA.

Proposal 3: In EN-DC, the OAM/CN can inform the logged MDT configuration of SN to the MN in order to collect the coverage of SN.

2.2. lmmediate MDT

For each MR-DC UE, only the MN has one control plane connection with the CN entity. In the SI, RAN2 has agreed that all the immediate measurements of LTE are supported in NR. We analyses the signalling based immediate measurement in the following table.

Table 1 Analysis of the signaling based immediate measurement in MR-DC

| measurements | Analysis |
| --- | --- |
| DL signal quantities measurement results(M1) | It is measured by the UE and reported to the RRC entity of the node. In our understanding, the operators may want to know the dl signal quantities of the SN and want to know the correlation of the data volume and the dl signal quantities. Therefore the MN need send the measurement requirement to the SN |
| PHR(M2) | It is measured by the UE. One of the PHR MAC CE format includes the PHR of the serving cells in the SN. Therefore the MN can configure the measurement. The MN does not need send the measurement requirement to the SN |
| Received Interference Power measurement(M3) | It is measured by the PHY of the NG-RAN. In our understanding, the operators may want to know the interference of the SN and want to know the correlation of the data volume and the interference. In the last RAN2 meeting, RAN2 sends one LS to check the feasibility of this measurement. Therefore it depends on RAN2 decision. Therefore if RAN2 supports this measurement, the MN need send the measurement requirement to the SN |
| Data Volume measurement separately for DL and UL(M4) | They are measured per DRB in the PDCP layer. In NR-DC and EN-DC, some DRBs are terminated in the SN. For these DRBs, they are measured by the SN. Therefore MN need send the measurement requirement to the SN |
| Scheduled IP Throughput for MDT measurement separately for DL and UL(M5) | In LTE, this measurements involves the PDCP/RLC/MAC layer. And also SA5 has defined a new measurement (i.e. Scheduled IP Throughput separately for DL and UL) which only involves the RLC layer and measured at RLC layer. In NR-DC and EN-DC, the RLC entities of some DRBs are in the SN. For these DRBs, they are measured by the SN. Therefore MN need send the measurement requirement to the SN |
| Packet Delay measurement separately for DL and UL(M6) | RAN2 has agreed the delay measurement in L2 measurement. The measurements are measured per DRB. The delay measurement includes the delay in PDCP layer and RLC/MAC/PHY layer. In our understanding, the measurement in MDT can reuse the definition in L2 measurement. Therefore MN need send the measurement requirement to the SN. |
| Packet loss rate measurement separately for DL and UL(M7) | For UL, in LTE, it involves the PDCP layer. And in TS 28.552 SA5 has defined a new measurement (i.e. UL PDCP SDU loss rate) that it is measured in the PDCP layer. For DL, in LTE, it involves the PDCP/RLC/MAC layer. And in TS 28.552 SA5 has defined two new measurements (i.e. DL PDCP SDU Drop rate in gNB-CU-UP and DL Packet drop rate in gNB-DU). In NR-DC and EN-DC, some DRBs involves the SN. For these DRBs, MN need send the measurement requirements to the SN. Therefore MN need send the measurement requirement to the SN |
| RSSI measurement by UE | It is measured by the UE. Therefore the MN can configure the measurement. The MN does not need send the measurement requirement to the SN |
| RTT measurement by UE | It is measured by the UE. Therefore the MN can configure the measurement. The MN does not need send the measurement requirement to the SN |

According to the above analysis, we think the MN need send some signalling based immediate measurement requirement to the SN.

For the management based immediate MDT, we think the operators also want to know the performance of every DRB and want to know the correlation between the performance of DRB and the DL/UL signal quantities. Therefore the above analysis are also suitable to the management based immediate MDT.

Proposal 4: For the signalling based and management based MDT, add the following measurement requirement interaction between MN and SN:

- DL signal quantities measurement

- Received Interference Power measurement(FFS)

- Data volume measurement separately for DL and UL

- Scheduled IP Throughput separately for DL and UL

- Packet Delay measurement separately for DL and UL

- Packet loss rate measurement separately for DL and UL

In EN-DC, RAN3 has introduced the TRACE START/DEACTIVE TRACE message in X2. We think RAN3 also can introduce these messages in Xn.

- Proposal 5: Introduce the TRACE START and DEACTIVE TRACE messages in Xn

In our understanding, there are some measurements in the QoS verification use cases (e.g. Data Volume, Date Loss and Latency measurements) that are collected per UE or per 5QI/QCI/DRB per UE.  In order to get the exact measurement results or correlate the collection of different 5QI/QCI to the same UE, we think TCE should correlate all the measurement results of the same UE. But some measurements (e.g. Data Volume, Date Loss and Latency measurements) are measured periodically. If all the measurements results are first sent to one node (e.g. MN) and then the node sends the combined results to TCE, it will increase the load of the Xn/X2. Therefore we think the MN and SN can send the measurement results to TCE respectively.

In order to get the exact measurement results or correlate the collection of different 5QI/QCI to the same UE, one identification is needed to correlate the same UE. For the signalling based MDT, the trace ID can uniquely identify the trace of one UE. Therefore the MN can send the trace ID to the SN. And the MN and SN send the measurement results and the trace ID to the TCE. For the management based MDT, the trace ID cannot uniquely identify the trace of one UE. It is the network implement to decide which information are sent to the TCE in order to correlate the measurement results of the same UE(e.g. the MN UE XnAP ID and the SN UE XnAP ID).

- Proposal 6:The MN and SN send the immediate MDT measurement results to the TCE respectively

4. Conclusion

Based on the discussion in this paper, we have the following proposals:

Proposal 1: For the signalling based logged MDT in NR-DC and EN-DC:

- The MN does not send the signalling based logged MDT configuration to the SN.

- In EN-DC, the SN does not configure the logged MDT for the UE.

Proposal 2: For the management based logged MDT:

- In NR-DC, the SN selects the UE to configure the logged MDT according to the configuration received from OAM.

- In EN-DC, the SN does not configure the logged MDT for the UE.

Proposal 3: In EN-DC, the OAM/CN can inform the logged MDT configuration of SN to the MN in order to collect the coverage of SN.

Proposal 4: For the signalling based and management based MDT, add the following measurement requirement interaction between MN and SN:

- DL signal quantities measurement

- Received Interference Power measurement(FFS)

- Data volume measurement separately for DL and UL

- Scheduled IP Throughput separately for DL and UL

- Packet Delay measurement separately for DL and UL

- Packet loss rate measurement separately for DL and UL

- Proposal 5: Introduce the TRACE START and DEACTIVE TRACE messages in Xn

- Proposal 6:The MN and SN send the immediate MDT measurement results to the TCE respectively

5. Reference

- TR 37.816-g00

- RP-191594, New WID on SON/MDT support for NR

Annex – TP

Change Start

# 8	XnAP procedures

## 8.1	Elementary procedures

---------skip the unchanged text-------------

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |

Next Change

### 8.3.x	Trace Start

#### 8.3.x.1	General

The purpose of the Trace Start procedure is to allow the M-NG-RAN to request the S-NG-RAN or to allow the S-NG-RAN to request the M-NG-RAN to initiate a trace session for a UE. The procedure uses UE-associated signalling.

#### 8.3.x.2	Successful Operation

Figure 8.7.19.2-1: Trace Start, successful operation

The Trace Start procedure is initiated by the M-NG-RAN sending the TRACE START message to the S-NG-RAN or by the S-NG-RAN sending the TRACE START message to the M-NG-RAN for that specific UE. Upon reception of the TRACE START message, the NG-RAN shall initiate the requested trace session as described in TS 32.422 [X].

#### 8.3.x.3	Abnormal Conditions

Void.

### 8.3.y	Deactivate Trace

#### 8.3.y.1	General

The purpose of the Deactivate Trace procedure is to allow M-NG-RAN to request the S-NG-RAN or to allow the S-NG-RAN to stop the trace session for the indicated trace reference. The procedure uses UE-associated signalling.

#### 8.3.y.2	Successful Operation

Figure 8.7.20.2-1: Deactivate Trace, successful opration

The Deactivate Trace procedure is initiated by the M-NG-RAN by sending the DEACTIVATE TRACE to the S-NG-RAN or by the S-NG-RAN sending the DEACTIVATE TRACE message to the M-NG-RAN for that specific UE. Upon reception of the DEACTIVATE TRACE message, the NG-RAN shall stop the trace session for the indicated trace reference in the E-UTRAN Trace ID IE.

#### 8.3.y.3	Abnormal Conditions

Void.

Next Change

#### 9.1.2.2x	TRACE START

This message is sent by the M-NG-RAN or the S-NG-RAN to initiate a trace session for a UE.

Direction: M-NG-RAN node  S-NG-RAN node or S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Trace Activation | M |  | 9.2.2 |  | YES | ignore |

#### 9.1.2.y	DEACTIVATE TRACE

This message is sent by the M-NG-RAN or the S-NG-RAN to deactivate a trace session.

Direction: M-NG-RAN node  S-NG-RAN node or S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Trace ID | M |  | OCTET STRING (SIZE(8)) | As per E-UTRAN Trace ID in Trace Activation IE | YES | ignore |

Next Change

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextFailure,

XnUAddressIndication,

SecondaryRATDataUsageReport,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnRemovalRequest,

XnRemovalResponse,

XnRemovalFailure,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

E-UTRA-NR-CellResourceCoordinationRequest,

E-UTRA-NR-CellResourceCoordinationResponse,

ActivityNotification,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage,

DeactivateTrace,

TraceStart

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage,

id-deactivateTrace,

id-traceStart

FROM XnAP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode	UNIQUE,

&criticality			Criticality		DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

XnAP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURES XNAP-ELEMENTARY-PROCEDURE ::= {

XNAP-ELEMENTARY-PROCEDURES-CLASS-1			|

XNAP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-1 XNAP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

retrieveUEContext										|

sNGRANnodeAdditionPreparation							|

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

mNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeChange										|

xnRemoval												|

xnSetup													|

nGRANnodeConfigurationUpdate							|

e-UTRA-NR-CellResourceCoordination						|

cellActivation											|

reset													,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport				|

deactivateTrace							|

traceStart								,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

sNStatusTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNStatusTransfer

PROCEDURE CODE			id-sNStatusTransfer

CRITICALITY				ignore

}

handoverCancel	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverCancel

PROCEDURE CODE			id-handoverCancel

CRITICALITY				ignore

}

retrieveUEContext	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RetrieveUEContextRequest

SUCCESSFUL OUTCOME		RetrieveUEContextResponse

UNSUCCESSFUL OUTCOME	RetrieveUEContextFailure

PROCEDURE CODE			id-retrieveUEContext

CRITICALITY				reject

}

rANPaging	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RANPaging

PROCEDURE CODE			id-rANPaging

CRITICALITY				reject

}

xnUAddressIndication	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnUAddressIndication

PROCEDURE CODE			id-xnUAddressIndication

CRITICALITY				reject

}

uEContextRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextRelease

PROCEDURE CODE			id-uEContextRelease

CRITICALITY				reject

}

sNGRANnodeAdditionPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeAdditionRequest

SUCCESSFUL OUTCOME		SNodeAdditionRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeAdditionRequestReject

PROCEDURE CODE			id-sNGRANnodeAdditionPreparation

CRITICALITY				reject

}

sNGRANnodeReconfigurationCompletion	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReconfigurationComplete

PROCEDURE CODE			id-sNGRANnodeReconfigurationCompletion

CRITICALITY				reject

}

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeModificationRequest

SUCCESSFUL OUTCOME		SNodeModificationRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeModificationRequestReject

PROCEDURE CODE			id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation

CRITICALITY				reject

}

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeModificationRequired

SUCCESSFUL OUTCOME		SNodeModificationConfirm

UNSUCCESSFUL OUTCOME	SNodeModificationRefuse

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation

CRITICALITY				reject

}

mNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequest

SUCCESSFUL OUTCOME		SNodeReleaseRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeReleaseReject

PROCEDURE CODE			id-mNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

sNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequired

SUCCESSFUL OUTCOME		SNodeReleaseConfirm

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

sNGRANnodeCounterCheck	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeCounterCheckRequest

PROCEDURE CODE			id-sNGRANnodeCounterCheck

CRITICALITY				reject

}

sNGRANnodeChange		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeChangeRequired

SUCCESSFUL OUTCOME		SNodeChangeConfirm

UNSUCCESSFUL OUTCOME	SNodeChangeRefuse

PROCEDURE CODE			id-sNGRANnodeChange

CRITICALITY				reject

}

rRCTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCTransfer

PROCEDURE CODE			id-rRCTransfer

CRITICALITY				reject

}

xnRemoval	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnRemovalRequest

SUCCESSFUL OUTCOME		XnRemovalResponse

UNSUCCESSFUL OUTCOME		XnRemovalFailure

PROCEDURE CODE			id-xnRemoval

CRITICALITY				reject

}

xnSetup	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnSetupRequest

SUCCESSFUL OUTCOME		XnSetupResponse

UNSUCCESSFUL OUTCOME		XnSetupFailure

PROCEDURE CODE			id-xnSetup

CRITICALITY				reject

}

nGRANnodeConfigurationUpdate	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGRANNodeConfigurationUpdate

SUCCESSFUL OUTCOME		NGRANNodeConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	NGRANNodeConfigurationUpdateFailure

PROCEDURE CODE			id-nGRANnodeConfigurationUpdate

CRITICALITY				reject

}

e-UTRA-NR-CellResourceCoordination	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-UTRA-NR-CellResourceCoordinationRequest

SUCCESSFUL OUTCOME		E-UTRA-NR-CellResourceCoordinationResponse

PROCEDURE CODE			id-e-UTRA-NR-CellResourceCoordination

CRITICALITY				reject

}

cellActivation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellActivationRequest

SUCCESSFUL OUTCOME		CellActivationResponse

UNSUCCESSFUL OUTCOME	CellActivationFailure

PROCEDURE CODE			id-cellActivation

CRITICALITY				reject

}

reset	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResetRequest

SUCCESSFUL OUTCOME		ResetResponse

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

errorIndication	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

notificationControl			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NotificationControlIndication

PROCEDURE CODE			id-notificationControl

CRITICALITY				ignore

}

activityNotification		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ActivityNotification

PROCEDURE CODE			id-activityNotification

CRITICALITY				ignore

}

privateMessage			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

secondaryRATDataUsageReport	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SecondaryRATDataUsageReport

PROCEDURE CODE			id-secondaryRATDataUsageReport

CRITICALITY				reject

}

deactivateTrace XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-deactivateTrace

CRITICALITY				ignore

}

traceStart XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-traceStart

CRITICALITY				ignore

}

END

-- ASN1STOP

Next Change

### 9.3.4	PDU Definitions

id-PDUSessionNotAdmitted-SNModResponse,

id-admittedSplitSRB,

id-admittedSplitSRBrelease,

id-PDUSessionAdmittedModSNModConfirm,

id-PDUSessionReleasedSNModConfirm,

id-s-ng-RANnode-SecurityKey,

id-PDUSessionToBeModifiedSNModRequired,

id-S-NG-RANnodeUE-AMBR,

id-PDUSessionToBeReleasedSNModRequired,

id-target-S-NG-RANnodeID,

id-S-NSSAI,

id-MR-DC-ResourceCoordinationInfo,

id-RANPagingFailure,

id-UERadioCapabilityForPaging,

id-PDUSessionDataForwarding-SNModResponse,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-NE-DC-TDM-Pattern,

id-InterfaceInstanceIndication,

id-TraceID,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows

FROM XnAP-Constants;

Next Change

-- **************************************************************

--

-- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs XNAP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

-- TRACE START

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceStartIEs} },

...

}

TraceStartIEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory	}|

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID				PRESENCE mandatory	}|

{ ID id-TraceActivation			CRITICALITY ignore	TYPE TraceActivation				PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- DEACTIVATE TRACE

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DeactivateTraceIEs} },

...

}

DeactivateTraceIEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory	}|

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID				PRESENCE mandatory	}|

{ ID id-TraceID			CRITICALITY ignore	TYPE TraceID					PRESENCE mandatory	},

...

}

END

-- ASN1STOP

Next Change

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= XX

id-traceStart													ProcedureCode ::= YY

Next Change

id-UERadioCapabilityForPaging																		ProtocolIE-ID ::= 122

id-PDUSessionDataForwarding-SNModResponse															ProtocolIE-ID ::= 123

id-DRBsNotAdmittedSetupModifyList																	ProtocolIE-ID ::= 124

id-Secondary-MN-Xn-U-TNLInfoatM																		ProtocolIE-ID ::= 125

id-NE-DC-TDM-Pattern																				ProtocolIE-ID ::= 126

id-PDUSessionCommonNetworkInstance																	ProtocolIE-ID ::= 127

id-BPLMN-ID-Info-EUTRA																				ProtocolIE-ID ::= 128

id-BPLMN-ID-Info-NR																					ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication																		ProtocolIE-ID ::= 130

id-TraceID,																		ProtocolIE-ID ::= XXX

END

-- ASN1STOP

End Change