3GPP TSG-RAN3 Meeting #105bis	R3-195574

Chongqing, China, 14-18 October 2019

Title: 	(TP for LTE_feMob BL CR for TS 36.423): Handover modification for CHO

Source: 	Huawei

Agenda item:	15.3.1

Document for:	pCR

1. Introduction

In RAN3#105 meeting [1], some details for CHO cancelling are agreed as below:

CHO Cancel procedure from the target node shall not be used in legacy HO

Both source and target nodes are allowed to initiate per-target cell CHO cancel; target node is only allowed to cancel a prepared CHO

This paper tries to introduce more changes according to the proposals in [1], the mina update is:

- to introduce the Conditional Handover Modification Request

4. Reference

[1] R3-195573, (TP for NR_Mob_enh BL CR for TS 38.423): Handover modification for CHO, Huawei.

Change Begins

# 7	Functions of X2AP

The X2AP protocol provides the following functions:

-	Mobility Management. This function allows the eNB to move the responsibility of a certain UE to another eNB. Forwarding of user plane data, Status Transfer and UE Context Release function are parts of the mobility management.

-	Dual Connectivity. This function allows the eNB to request another eNB to provide radio resources for a certain UE while keeping responsibility for that UE.

-	E-UTRA-NR Dual Connectivity. This function allows the eNB to request another en-gNB to provide radio resources for a certain UE while keeping responsibility for that UE.

-	Load Management. This function is used by eNBs to indicate resource status, overload and traffic load to each other.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Resetting the X2. This function is used to reset the X2 interface.

-	Setting up the X2. This function is used to exchange necessary data for the eNB or en-gNB for setup the X2 interface and implicitly perform an X2 Reset.

-	eNB Configuration Update. This function allows updating of application level data needed for two eNBs to interoperate correctly over the X2 interface.

-	Mobility Parameters Management. This function allows the eNB to coordinate adaptation of mobility parameter settings with a peer eNB.

-	Mobility Robustness Optimisation. This function allows reporting of information related to mobility failure events.

-	Energy Saving. This function allows decreasing energy consumption by enabling indication of cell activation/deactivation over the X2 interface.

-	X2 Release. This function allows an eNB to be aware that the signalling connection to a peer eNB is unavailable.

-	Message Transfer. This function allows indirect transport of X2AP messages to a peer eNB.

-	Registration. This function allows registration of eNB in case indirect transport of X2AP messages is supported.

-	Removing the X2. This function allows removing the signalling connection between two eNBs or between eNB and en-gNB in a controlled manner.

-	Inter-eNB UE Context Retrieval. This function allows retrieval of a UE context in case of resumption or re-establishment of an RRC connection.

-	Secondary RAT Data Usage Report. This function allows eNB to get the uplink and downlink data volumes for the Secondary RAT on a per E-RAB basis.

-	E-UTRA - NR Spectrum Sharing. This function allows uplink and downlink spectrum sharing between a number of E - UTRA and a number of NR cells with overlapping coverage.

-	EN-DC Configuration Transfer. This function supports en-gNB X2 TNL address discovery.

The mapping between the above functions and X2 EPs is shown in the table below.

Table 7-1: Mapping between X2AP functions and X2AP EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| Mobility Management | a) Handover Preparation b) SN Status Transfer c) UE Context Release d) Handover Cancel e) Handover Success f) Conditional Handover Modification Request |
| Dual Connectivity | a) SeNB Addition Preparation b) SeNB Reconfiguration Completion c) MeNB initiated SeNB Modification Preparation d) SeNB initiated SeNB Modification e) MeNB initiated SeNB Release f) SeNB initiated SeNB Release g) SeNB Counter Check |
| E-UTRA-NR Dual Connectivity | a) SgNB Addition Preparation b) SgNB Reconfiguration Completion c) MeNB initiated SgNB Modification Preparation d) SgNB initiated SgNB Modification e) SgNB change f) MeNB initiated SgNB Release g) SgNB initiated SgNB Release h) SgNB Counter Check i) RRC transfer j) EN-DC X2 Setup k) EN-DC Configuration Update l) EN-DC Cell Activation m) SgNB Activity Notification n) EN-DC X2 Removal o) gNB Status Indication |
| Load Management | a) Load Indication b) Resource Status Reporting Initiation c) Resource Status Reporting |
| Reporting of General Error Situations | Error Indication |
| Resetting the X2 | Reset |
| Setting up the X2 | X2 Setup |
| eNB Configuration Update | a) eNB Configuration Update b) Cell Activation |
| Mobility Parameters Management | Mobility Settings Change |
| Mobility Robustness Optimisation | a) Radio Link Failure Indication b) Handover Report |
| Energy Saving | a) eNB Configuration Update b) Cell Activation |
| X2 Release | X2 Release |
| Message Transfer Registration | X2AP Message Transfer |
| Removing the X2 | X2 Removal |
| Inter-eNB UE Context Retrieval | a) Retrieve UE Context b) Data Forwarding Address Indication |
| Secondary RAT Data Usage Report | Secondary RAT Data Usage Report |
| E-UTRA – NR Spectrum Sharing | E-UTRA - NR Cell Resource Coordination |
| EN-DC Configuration Transfer | EN-DC Configuration Transfer |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

8	X2AP procedures

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| X2 Setup | X2 SETUP REQUEST | X2 SETUP RESPONSE | X2 SETUP FAILURE |
| eNB Configuration Update | ENB CONFIGURATION UPDATE | ENB CONFIGURATION UPDATE ACKNOWLEDGE | ENB CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| SeNB Addition Preparation | SENB ADDITION REQUEST | SENB ADDITION REQUEST ACKNOWLEDGE | SENB ADDITION REQUEST REJECT |
| MeNB initiated SeNB Modification Preparation | SENB MODIFICATION REQUEST | SENB MODIFICATION REQUEST ACKNOWLEDGE | SENB MODIFICATION REQUEST REJECT |
| SeNB initiated SeNB Modification | SENB MODIFICATION REQUIRED | SENB MODIFICATION CONFIRM | SENB MODIFICATION REFUSE |
| SeNB initiated SeNB Release | SENB RELEASE REQUIRED | SENB RELEASE CONFIRM |  |
| X2 Removal | X2 REMOVAL REQUEST | X2 REMOVAL RESPONSE | X2 REMOVAL FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| SgNB Addition Preparation | SGNB ADDITION REQUEST | SGNB ADDITION REQUEST ACKNOWLEDGE | SGNB ADDITION REQUEST REJECT |
| MeNB initiated SgNB Modification Preparation | SGNB MODIFICATION REQUEST | SGNB MODIFICATION REQUEST ACKNOWLEDGE | SGNB MODIFICATION REQUEST REJECT |
| SgNB initiated SgNB Modification | SGNB MODIFICATION REQUIRED | SGNB MODIFICATION CONFIRM | SGNB MODIFICATION REFUSE |
| SgNB change | SGNB CHANGE REQUIRED | SGNB CHANGE CONFIRM | SGNB CHANGE REFUSE |
| MeNB initiated SgNB Release | SGNB RELEASE REQUEST | SGNB RELEASE REQUEST ACKNOWLEDGE | SGNB RELEASE REQUEST REJECT |
| SgNB initiated SgNB Release | SGNB RELEASE REQUIRED | SGNB RELEASE CONFIRM |  |
| EN-DC X2 Setup | EN-DC X2 SETUP REQUEST | EN-DC X2 SETUP RESPONSE | EN-DC X2 SETUP FAILURE |
| EN-DC Configuration Update | EN-DC CONFIGURATION UPDATE | EN-DC CONFIGURATION UPDATE ACKNOWLEDGE | EN-DC CONFIGURATION UPDATE FAILURE |
| EN-DC Cell Activation | EN-DC CELL ACTIVATION REQUEST | EN-DC CELL ACTIVATION RESPONSE | EN-DC CELL ACTIVATION FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| EN-DC X2 Removal | EN-DC X2 REMOVAL REQUEST | EN-DC X2 REMOVAL RESPONSE | EN-DC X2 REMOVAL FAILURE |
| Conditional Handover Modification Request | CONDITIONAL HANDOVER MODIFICATION REQUEST | CONDITIONAL HANDOVER MODIFICATION RESPONSE | CONDITIONAL HANDOVER MODIFICATION FAILURE |

*** Next change, skipped text not changed ***

8.2.X	Conditional Handover Modification Request

8.2.X.1	General

The purpose of the Conditional Handover Modification procedure is to enable the source RAN node to modify an ongoing conditional handover preparation or an already prepared conditional handover.

The procedure uses UE-associated signalling.

8.2.X.2	Successful Operation

Figure 8.2.X.2-1: Conditional Handover Modification Request, successful operation

The source eNB initiates the procedure by sending the CONDITIONAL HANDOVER MODIFICATION REQUEST HANDOVER message to the target eNB. If the target eNB succeeds to partially or fully acknowledge the requested modifications on the prepared or ongoing conditional handover indicated by the Target Cell ID IE, it replies to the source eNB with the CONDITIONAL HANDOVER MODIFICATION RESPONSE message.

The target eNB shall include the E-RABs for which resources have been prepared at the target cell in the E-RABs Admitted List IE. The target eNB shall include the E-RABs that have not been admitted in the E-RABs Not Admitted List IE with an appropriate cause value. The target eNB shall include the E-RABs that have been successfully modified in the E-RABs Modified List IE.

For each bearer in the E-RABs Admitted List IE, the target eNB may include the UL GTP Tunnel Endpoint IE to indicate that it requests data forwarding of uplink packets to be performed for that bearer.

If the UE Aggregate Maximum Bit Rate IE is included in the CONDITIONAL HANDOVER MODIFICATION REQUEST message, the target eNB shall take into account.

If the UE Security Capabilities IE is included in CONDITIONAL HANDOVER MODIFICATION REQUEST message, the target eNB shall take them into use together with the received keys indicated in AS Security Information IE, according to TS 33.401 [18].

8.2.X.3	Unsuccessful Operation

Figure 8.2.X.2-2: Conditional Handover Modification Request, unsuccessful operation

If the target eNB does not admit any modification requested by the source eNB, or in case none of the requested modifications of the request can be successfully performed, the target eNB shall send the CONDITIONAL HANDOVER MODIFICATION REQUEST FAILURE message to the source eNB. The message shall contain the Cause IE with an appropriate value.

8.2.X.4	Abnormal Conditions

If the supported algorithms for encryption defined in the Encryption Algorithms IE in the UE Security Capabilities IE in the UE Context Information IE, plus the mandated support of EEA0 in all UEs (TS 33.401 [18]), do not match any algorithms defined in the configured list of allowed encryption algorithms in the target eNB (TS 33.401 [18]), the target eNB shall reject the procedure using the CONDITIONAL HANDOVER MODIFICATION FAILURE message.

*** Next change, skipped text not changed ***

9.1.1.X	CONDITIONAL HANDOVER MODIFICATION REQUEST

This message is sent by the source eNB to the target eNB to modify an ongoing conditional handover preparation or an already prepared conditional handover.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | reject |
| Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| UE CHO Context Information |  | 1 |  |  | YES | reject |
| >UE Aggregate Maximum Bit Rate | O |  | 9.2.12 |  | – |  |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | – |  |
| >UE Security Capabilities | O |  | 9.2.29 |  | – |  |
| >AS Security Information | O |  | 9.2.30 |  | – |  |
| >E-RABs To Be Setup List |  | 0..1 |  |  | – |  |
| >>E-RABs To Be Setup Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – |  |
| >>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – |  |
| >>>Bearer Type | O |  | 9.2.92 |  | YES | reject |
| >E-RABs To Be Modified List |  | 0..1 |  |  | – |  |
| >>E-RABs To Be Modified Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>E-RAB Level QoS Parameters | O |  | 9.2.9 | Includes necessary QoS parameters | – |  |
| >>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>UL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – |  |
| >RRC Context | O |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [9], or the RRC HandoverPreparationInformation-NB message as defined in 10.6.2 of TS 36.331 [9]. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional handover. Value is FFS. |

*** Next change, skipped text not changed ***

9.1.1.y	CONDITIONAL HANDOVER MODIFICATION RESPONSE

This message is sent by the target eNB to the source eNB to confirm the modification on an ongoing conditional handover preparation or an already prepared conditional handover.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | reject |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | reject |
| Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| UE CHOContext Response Information |  | 1 |  |  | YES | reject |
| >E-RABs Admitted |  | 1 |  |  | – |  |
| >>E-RABs Admitted Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – |  |
| >>>Bearer Type | O |  | 9.2.92 |  | YES | reject |
| >E-RABs Modified List |  | 0..1 |  |  | – |  |
| >>E-RABs Modified Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>UL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – |  |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |

*** Next change, skipped text not changed ***

9.1.1.z	CONDITIONAL HANDOVER MODIFICATION FAILURE

This message is sent by the target eNB to inform the source eNB that the Conditional Handover Modification has failed.

Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |

*** Next change, skipped text not changed ***

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

X2AP-ELEMENTARY-PROCEDURES X2AP-ELEMENTARY-PROCEDURE ::= {

X2AP-ELEMENTARY-PROCEDURES-CLASS-1			|

X2AP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

X2AP-ELEMENTARY-PROCEDURES-CLASS-1 X2AP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

reset													|

x2Setup													|

resourceStatusReportingInitiation						|

eNBConfigurationUpdate									|

mobilitySettingsChange									|

cellActivation											|

seNBAdditionPreparation									|

meNBinitiatedSeNBModificationPreparation				|

seNBinitiatedSeNBModification							|

seNBinitiatedSeNBRelease								|

x2Removal												|

retrieveUEContext										|

sgNBAdditionPreparation									|

meNBinitiatedSgNBModificationPreparation					|

sgNBinitiatedSgNBModification							|

meNBinitiatedSgNBRelease									|

sgNBinitiatedSgNBRelease									|

sgNBChange												|

endcX2Setup												|

endcConfigurationUpdate									|

endcCellActivation										,

...

}

X2AP-ELEMENTARY-PROCEDURES-CLASS-2 X2AP-ELEMENTARY-PROCEDURE ::= {

snStatusTransfer						|

uEContextRelease						|

handoverCancel							|

errorIndication							|

resourceStatusReporting					|

loadIndication							|

privateMessage							|

rLFIndication							|

handoverReport							|

x2Release								|

x2APMessageTransfer						|

seNBReconfigurationCompletion			|

meNBinitiatedSeNBRelease				|

seNBCounterCheck						|

sgNBReconfigurationCompletion			|

sgNBCounterCheck							|

rRCTransfer								|

secondaryRATDataUsageReport				,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

snStatusTransfer X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNStatusTransfer

PROCEDURE CODE			id-snStatusTransfer

CRITICALITY				ignore

}

uEContextRelease X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextRelease

PROCEDURE CODE			id-uEContextRelease

CRITICALITY				ignore

}

handoverCancel X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverCancel

PROCEDURE CODE			id-handoverCancel

CRITICALITY				ignore

}

handoverReport X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverReport

PROCEDURE CODE			id-handoverReport

CRITICALITY				ignore

}

errorIndication X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

reset	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResetRequest

SUCCESSFUL OUTCOME		ResetResponse

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

x2Setup	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		X2SetupRequest

SUCCESSFUL OUTCOME		X2SetupResponse

UNSUCCESSFUL OUTCOME	X2SetupFailure

PROCEDURE CODE			id-x2Setup

CRITICALITY				reject

}

loadIndication X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		LoadInformation

PROCEDURE CODE			id-loadIndication

CRITICALITY				ignore

}

eNBConfigurationUpdate		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENBConfigurationUpdate

SUCCESSFUL OUTCOME		ENBConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	ENBConfigurationUpdateFailure

PROCEDURE CODE			id-eNBConfigurationUpdate

CRITICALITY				reject

}

resourceStatusReportingInitiation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				ResourceStatusRequest

SUCCESSFUL OUTCOME				ResourceStatusResponse

UNSUCCESSFUL OUTCOME			ResourceStatusFailure

PROCEDURE CODE					id-resourceStatusReportingInitiation

CRITICALITY						reject

}

resourceStatusReporting X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusReporting

CRITICALITY				ignore

}

rLFIndication X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RLFIndication

PROCEDURE CODE			id-rLFIndication

CRITICALITY				ignore

}

privateMessage			X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

mobilitySettingsChange	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MobilityChangeRequest

SUCCESSFUL OUTCOME		MobilityChangeAcknowledge

UNSUCCESSFUL OUTCOME	MobilityChangeFailure

PROCEDURE CODE			id-mobilitySettingsChange

CRITICALITY				reject

}

cellActivation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellActivationRequest

SUCCESSFUL OUTCOME		CellActivationResponse

UNSUCCESSFUL OUTCOME	CellActivationFailure

PROCEDURE CODE			id-cellActivation

CRITICALITY				reject

}

x2Release X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		X2Release

PROCEDURE CODE			id-x2Release

CRITICALITY				reject

}

x2APMessageTransfer X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		X2APMessageTransfer

PROCEDURE CODE			id-x2APMessageTransfer

CRITICALITY				reject

}

seNBAdditionPreparation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBAdditionRequest

SUCCESSFUL OUTCOME		SeNBAdditionRequestAcknowledge

UNSUCCESSFUL OUTCOME	SeNBAdditionRequestReject

PROCEDURE CODE			id-seNBAdditionPreparation

CRITICALITY				reject

}

seNBReconfigurationCompletion	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBReconfigurationComplete

PROCEDURE CODE			id-seNBReconfigurationCompletion

CRITICALITY				ignore

}

meNBinitiatedSeNBModificationPreparation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBModificationRequest

SUCCESSFUL OUTCOME		SeNBModificationRequestAcknowledge

UNSUCCESSFUL OUTCOME	SeNBModificationRequestReject

PROCEDURE CODE			id-meNBinitiatedSeNBModificationPreparation

CRITICALITY				reject

}

seNBinitiatedSeNBModification	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBModificationRequired

SUCCESSFUL OUTCOME		SeNBModificationConfirm

UNSUCCESSFUL OUTCOME	SeNBModificationRefuse

PROCEDURE CODE			id-seNBinitiatedSeNBModification

CRITICALITY				reject

}

meNBinitiatedSeNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBReleaseRequest

PROCEDURE CODE			id-meNBinitiatedSeNBRelease

CRITICALITY				ignore

}

seNBinitiatedSeNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBReleaseRequired

SUCCESSFUL OUTCOME		SeNBReleaseConfirm

PROCEDURE CODE			id-seNBinitiatedSeNBRelease

CRITICALITY				reject

}

seNBCounterCheck	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBCounterCheckRequest

PROCEDURE CODE			id-seNBCounterCheck

CRITICALITY				reject

}

x2Removal	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		X2RemovalRequest

SUCCESSFUL OUTCOME		X2RemovalResponse

UNSUCCESSFUL OUTCOME	X2RemovalFailure

PROCEDURE CODE			id-x2Removal

CRITICALITY				reject

}

retrieveUEContext	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RetrieveUEContextRequest

SUCCESSFUL OUTCOME		RetrieveUEContextResponse

UNSUCCESSFUL OUTCOME	RetrieveUEContextFailure

PROCEDURE CODE			id-retrieveUEContext

CRITICALITY				reject

}

sgNBAdditionPreparation		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBAdditionRequest

SUCCESSFUL OUTCOME		SgNBAdditionRequestAcknowledge

UNSUCCESSFUL OUTCOME	SgNBAdditionRequestReject

PROCEDURE CODE			id-sgNBAdditionPreparation

CRITICALITY				reject

}

sgNBReconfigurationCompletion	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBReconfigurationComplete

PROCEDURE CODE			id-sgNBReconfigurationCompletion

CRITICALITY				ignore

}

meNBinitiatedSgNBModificationPreparation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBModificationRequest

SUCCESSFUL OUTCOME		SgNBModificationRequestAcknowledge

UNSUCCESSFUL OUTCOME	SgNBModificationRequestReject

PROCEDURE CODE			id-meNBinitiatedSgNBModificationPreparation

CRITICALITY				reject

}

sgNBinitiatedSgNBModification		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBModificationRequired

SUCCESSFUL OUTCOME		SgNBModificationConfirm

UNSUCCESSFUL OUTCOME	SgNBModificationRefuse

PROCEDURE CODE			id-sgNBinitiatedSgNBModification

CRITICALITY				reject

}

meNBinitiatedSgNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBReleaseRequest

SUCCESSFUL OUTCOME		SgNBReleaseRequestAcknowledge

UNSUCCESSFUL OUTCOME	SgNBReleaseRequestReject

PROCEDURE CODE			id-meNBinitiatedSgNBRelease

CRITICALITY				ignore

}

sgNBinitiatedSgNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBReleaseRequired

SUCCESSFUL OUTCOME		SgNBReleaseConfirm

PROCEDURE CODE			id-sgNBinitiatedSgNBRelease

CRITICALITY				reject

}

sgNBCounterCheck	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBCounterCheckRequest

PROCEDURE CODE			id-sgNBCounterCheck

CRITICALITY				reject

}

sgNBChange	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBChangeRequired

SUCCESSFUL OUTCOME		SgNBChangeConfirm

UNSUCCESSFUL OUTCOME	SgNBChangeRefuse

PROCEDURE CODE			id-sgNBChange

CRITICALITY				reject

}

rRCTransfer	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCTransfer

PROCEDURE CODE			id-rRCTransfer

CRITICALITY				reject

}

endcX2Setup X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCX2SetupRequest

SUCCESSFUL OUTCOME		ENDCX2SetupResponse

UNSUCCESSFUL OUTCOME	ENDCX2SetupFailure

PROCEDURE CODE			id-endcX2Setup

CRITICALITY				reject

}

endcConfigurationUpdate		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCConfigurationUpdate

SUCCESSFUL OUTCOME		ENDCConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	ENDCConfigurationUpdateFailure

PROCEDURE CODE			id-endcConfigurationUpdate

CRITICALITY				reject

}

secondaryRATDataUsageReport	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SecondaryRATDataUsageReport

PROCEDURE CODE			id-secondaryRATDataUsageReport

CRITICALITY				reject

}

endcCellActivation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCCellActivationRequest

SUCCESSFUL OUTCOME		ENDCCellActivationResponse

UNSUCCESSFUL OUTCOME	ENDCCellActivationFailure

PROCEDURE CODE			id-endcCellActivation

CRITICALITY				reject

}

END

*** Next change, skipped text not changed ***

9.3.4	PDU Definitions

-- **************************************************************

--

#### -- PDU definitions for X2AP.

--

-- **************************************************************

X2AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ABSInformation,

ABS-Status,

AS-SecurityInformation,

BearerType,

Cause,

CompositeAvailableCapacityGroup,

Correlation-ID,

COUNTvalue,

CellReportingIndicator,

CriticalityDiagnostics,

CRNTI,

CSGMembershipStatus,

CSG-Id,

DeactivationIndication,

DL-Forwarding,

DynamicDLTransmissionInformation,

ECGI,

E-RAB-ID,

E-RAB-Level-QoS-Parameters,

E-RAB-List,

EUTRANTraceID,

GlobalENB-ID,

GTPtunnelEndpoint,

GUGroupIDList,

GUMMEI,

HandoverReportType,

HandoverRestrictionList,

Masked-IMEISV,

InvokeIndication,

LocationReportingInformation,

MDT-Configuration,

ManagementBasedMDTallowed,

MDTPLMNList,

Neighbour-Information,

PCI,

PDCP-SN,

PLMN-Identity,

ReceiveStatusofULPDCPSDUs,

Registration-Request,

RelativeNarrowbandTxPower,

RadioResourceStatus,

RRCConnReestabIndicator,

RRCConnSetupIndicator,

UE-RLF-Report-Container,

UEAppLayerMeasConfig,

RRC-Context,

ServedCell-Information,

ServedCells,

ShortMAC-I,

SRVCCOperationPossible,

SubscriberProfileIDforRFP,

TargetCellInUTRAN,

TargeteNBtoSource-eNBTransparentContainer,

TimeToWait,

TraceActivation,

TraceDepth,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-HistoryInformation,

UE-HistoryInformationFromTheUE,

UE-S1AP-ID,

UESecurityCapabilities,

UE-X2AP-ID,

UL-HighInterferenceIndicationInfo,

UL-InterferenceOverloadIndication,

HWLoadIndicator,

S1TNLLoadIndicator,

Measurement-ID,

ReportCharacteristics,

MobilityParametersInformation,

MobilityParametersModificationRange,

ReceiveStatusOfULPDCPSDUsExtended,

COUNTValueExtended,

SubframeAssignment,

ExtendedULInterferenceOverloadInfo,

ExpectedUEBehaviour,

SeNBSecurityKey,

MeNBtoSeNBContainer,

SeNBtoMeNBContainer,

SCGChangeIndication,

CoMPInformation,

ReportingPeriodicityRSRPMR,

RSRPMRList,

UE-RLF-Report-Container-for-extended-bands,

ProSeAuthorized,

CoverageModificationList,

ReportingPeriodicityCSIR,

CSIReportList,

ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

COUNTvaluePDCP-SNlength18,

LHN-ID,

UE-ContextKeptIndicator,

UE-X2AP-ID-Extension,

SIPTOBearerDeactivationIndication,

TunnelInformation,

V2XServicesAuthorized,

X2BenefitValue,

ResumeID,

EUTRANCellIdentifier,

MakeBeforeBreakIndicator,

WTID,

WT-UE-XwAP-ID,

UESidelinkAggregateMaximumBitRate,

SgNBSecurityKey,

MeNBtoSgNBContainer,

SgNBtoMeNBContainer,

MCGSplitSRBs,

RRCContainer,

SRBType,

GlobalGNB-ID,

GNB-ID,

SCGConfigurationQuery,

MCGSplitSRB,

UENRMeasurement,

EN-DC-ResourceConfiguration,

TAC,

NRARFCN,

NRCGI,

NRPCI,

NRUESecurityCapabilities,

PDCPChangeIndication,

ULConfiguration,

SgNB-UE-X2AP-ID,

SecondaryRATUsageReportList,

ActivationID,

MeNBResourceCoordinationInformation,

SgNBResourceCoordinationInformation,

NR-TxBW,

BroadcastPLMNs-Item,

RLCMode

FROM X2AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

X2AP-PRIVATE-IES,

X2AP-PROTOCOL-EXTENSION,

X2AP-PROTOCOL-IES,

X2AP-PROTOCOL-IES-PAIR

FROM X2AP-Containers

id-ABSInformation,

id-ActivatedCellList,

id-BearerType,

id-Cause,

id-CellInformation,

id-CellInformation-Item,

id-CellMeasurementResult,

id-CellMeasurementResult-Item,

id-CellToReport,

id-CellToReport-Item,

id-CompositeAvailableCapacityGroup,

id-CriticalityDiagnostics,

id-DeactivationIndication,

id-DynamicDLTransmissionInformation,

id-E-RABs-Admitted-Item,

id-E-RABs-Admitted-List,

id-E-RABs-NotAdmitted-List,

id-E-RABs-SubjectToStatusTransfer-List,

id-E-RABs-SubjectToStatusTransfer-Item,

id-E-RABs-ToBeSetup-Item,

id-GlobalENB-ID,

id-GUGroupIDList,

id-GUGroupIDToAddList,

id-GUGroupIDToDeleteList,

id-GUMMEI-ID,

id-Masked-IMEISV,

id-InvokeIndication,

id-New-eNB-UE-X2AP-ID,

id-Old-eNB-UE-X2AP-ID,

id-Registration-Request,

id-ReportingPeriodicity,

id-ServedCells,

id-ServedCellsToActivate,

id-ServedCellsToAdd,

id-ServedCellsToModify,

id-ServedCellsToDelete,

id-SRVCCOperationPossible,

id-TargetCell-ID,

id-TargeteNBtoSource-eNBTransparentContainer,

id-TimeToWait,

id-TraceActivation,

id-UE-ContextInformation,

id-UE-CHOContextInformation,

id-UE-CHOContextResponseInformation,

id-UE-HistoryInformation,

id-UE-X2AP-ID,

id-Measurement-ID,

id-ReportCharacteristics,

id-ENB1-Measurement-ID,

id-ENB2-Measurement-ID,

id-ENB1-Cell-ID,

id-ENB2-Cell-ID,

id-ENB2-Proposed-Mobility-Parameters,

id-ENB1-Mobility-Parameters,

id-ENB2-Mobility-Parameters-Modification-Range,

id-FailureCellPCI,

id-Re-establishmentCellECGI,

id-FailureCellCRNTI,

id-ShortMAC-I,

id-SourceCellECGI,

id-FailureCellECGI,

id-HandoverReportType,

id-UE-RLF-Report-Container,

id-PartialSuccessIndicator,

id-MeasurementInitiationResult-List,

id-MeasurementInitiationResult-Item,

id-MeasurementFailureCause-Item,

id-CompleteFailureCauseInformation-List,

id-CompleteFailureCauseInformation-Item,

id-CSGMembershipStatus,

id-CSG-Id,

id-MDTConfiguration,

id-ManagementBasedMDTallowed,

id-ABS-Status,

id-RRCConnSetupIndicator,

id-RRCConnReestabIndicator,

id-TargetCellInUTRAN,

id-MobilityInformation,

id-SourceCellCRNTI,

id-ManagementBasedMDTPLMNList,

id-ReceiveStatusOfULPDCPSDUsExtended,

id-ULCOUNTValueExtended,

id-DLCOUNTValueExtended,

id-IntendedULDLConfiguration,

id-ExtendedULInterferenceOverloadInfo,

id-RNL-Header,

id-x2APMessage,

id-UE-HistoryInformationFromTheUE,

id-ExpectedUEBehaviour,

id-MeNB-UE-X2AP-ID,

id-SeNB-UE-X2AP-ID,

id-UE-SecurityCapabilities,

id-SeNBSecurityKey,

id-SeNBUEAggregateMaximumBitRate,

id-ServingPLMN,

id-E-RABs-ToBeAdded-List,

id-E-RABs-ToBeAdded-Item,

id-MeNBtoSeNBContainer,

id-E-RABs-Admitted-ToBeAdded-List,

id-E-RABs-Admitted-ToBeAdded-Item,

id-SeNBtoMeNBContainer,

id-ResponseInformationSeNBReconfComp,

id-UE-ContextInformationSeNBModReq,

id-E-RABs-ToBeAdded-ModReqItem,

id-E-RABs-ToBeModified-ModReqItem,

id-E-RABs-ToBeReleased-ModReqItem,

id-E-RABs-Admitted-ToBeAdded-ModAckList,

id-E-RABs-Admitted-ToBeModified-ModAckList,

id-E-RABs-Admitted-ToBeReleased-ModAckList,

id-E-RABs-Admitted-ToBeAdded-ModAckItem,

id-E-RABs-Admitted-ToBeModified-ModAckItem,

id-E-RABs-Admitted-ToBeReleased-ModAckItem,

id-SCGChangeIndication,

id-E-RABs-ToBeReleased-ModReqd,

id-E-RABs-ToBeReleased-ModReqdItem,

id-E-RABs-ToBeReleased-List-RelReq,

id-E-RABs-ToBeReleased-RelReqItem,

id-E-RABs-ToBeReleased-List-RelConf,

id-E-RABs-ToBeReleased-RelConfItem,

id-E-RABs-SubjectToCounterCheck-List,

id-E-RABs-SubjectToCounterCheckItem,

id-CoMPInformation,

id-ReportingPeriodicityRSRPMR,

id-RSRPMRList,

id-UE-RLF-Report-Container-for-extended-bands,

id-ProSeAuthorized,

id-CoverageModificationList,

id-ReportingPeriodicityCSIR,

id-CSIReportList,

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

id-ULCOUNTValuePDCP-SNlength18,

id-DLCOUNTValuePDCP-SNlength18,

id-LHN-ID,

id-Correlation-ID,

id-SIPTO-Correlation-ID,

id-UE-ContextReferenceAtSeNB,

id-UE-ContextReferenceAtWT,

id-UE-ContextKeptIndicator,

id-WT-UE-ContextKeptIndicator,

id-New-eNB-UE-X2AP-ID-Extension,

id-Old-eNB-UE-X2AP-ID-Extension,

id-MeNB-UE-X2AP-ID-Extension,

id-SeNB-UE-X2AP-ID-Extension,

id-SIPTO-BearerDeactivationIndication,

id-Tunnel-Information-for-BBF,

id-SIPTO-L-GW-TransportLayerAddress,

id-GW-TransportLayerAddress,

id-X2RemovalThreshold,

id-CellReportingIndicator,

id-V2XServicesAuthorized,

id-resumeID,

id-UE-ContextInformationRetrieve,

id-E-RABs-ToBeSetupRetrieve-Item,

id-NewEUTRANCellIdentifier,

id-MakeBeforeBreakIndicator,

id-UESidelinkAggregateMaximumBitRate,

id-uL-GTPtunnelEndpoint,

id-SgNBSecurityKey,

id-SgNBUEAggregateMaximumBitRate,

id-E-RABs-ToBeAdded-SgNBAddReqList,

id-MeNBtoSgNBContainer,

id-SgNB-UE-X2AP-ID,

id-RequestedMCGSplitSRBs,

id-E-RABs-ToBeAdded-SgNBAddReq-Item,

id-E-RABs-Admitted-ToBeAdded-SgNBAddResList,

id-SgNBtoMeNBContainer,

id-AdmittedMCGSplitSRBs	,

id-E-RABs-Admitted-ToBeAdded-SgNBAddRes-Item,

id-ResponseInformationSgNBReconfComp,

id-UE-ContextInformation-SgNBModReq,

id-E-RABs-ToBeAdded-SgNBModReq-Item,

id-E-RABs-ToBeModified-SgNBModReq-Item,

id-E-RABs-ToBeReleased-SgNBModReq-Item,

id-E-RABs-Admitted-ToBeAdded-SgNBModAckList,

id-E-RABs-Admitted-ToBeModified-SgNBModAckList,

id-E-RABs-Admitted-ToBeReleased-SgNBModAckList,

id-E-RABs-Admitted-ToBeAdded-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeModified-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeReleased-SgNBModAck-Item,

id-E-RABs-ToBeReleased-SgNBModReqdList,

id-E-RABs-ToBeModified-SgNBModReqdList,

id-E-RABs-ToBeReleased-SgNBModReqd-Item,

id-E-RABs-ToBeModified-SgNBModReqd-Item,

id-E-RABs-ToBeReleased-SgNBChaConfList,

id-E-RABs-ToBeReleased-SgNBChaConf-Item,

id-E-RABs-ToBeReleased-SgNBRelReqList,

id-E-RABs-ToBeReleased-SgNBRelReq-Item,

id-E-RABs-ToBeReleased-SgNBRelConfList,

id-E-RABs-ToBeReleased-SgNBRelConf-Item,

id-E-RABs-SubjectToSgNBCounterCheck-List,

id-E-RABs-SubjectToSgNBCounterCheck-Item,

id-Target-SgNB-ID,

id-RRCContainer,

id-SRBType,

id-HandoverRestrictionList,

id-SCGConfigurationQuery,

id-MCGSplitSRB,

id-UENRMeasurement,

id-InitiatingNodeType-EndcX2Setup,

id-InitiatingNodeType-EndcConfigUpdate,

id-RespondingNodeType-EndcX2Setup,

id-RespondingNodeType-EndcConfigUpdate,

id-NRUESecurityCapabilities,

id-PDCPChangeIndication,

id-ServedEUTRAcellsENDCX2ManagementList,

id-ServedEUTRAcellsToModifyListENDCConfUpd,

id-ServedEUTRAcellsToDeleteListENDCConfUpd,

id-ServedNRcellsToModifyListENDCConfUpd,

id-ServedNRcellsToDeleteListENDCConfUpd,

id-CellAssistanceInformation,

id-Globalen-gNB-ID,

id-ServedNRcellsENDCX2ManagementList,

id-Old-SgNB-UE-X2AP-ID,

id-UE-ContextReferenceAtSgNB,

id-SecondaryRATUsageReportList,

id-ActivationID,

id-ServedNRCellsToActivate,

id-ActivatedNRCellList,

id-MeNBResourceCoordinationInformation,

id-SgNBResourceCoordinationInformation,

id-UEAppLayerMeasConfig,

maxCellineNB,

maxnoofBearers,

maxnoofPDCP-SN,

maxFailedMeasObjects,

maxnoofCellIDforMDT,

maxnoofTAforMDT,

maxofNRNeighbours,

maxCellinengNB,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC

FROM X2AP-Constants;

*** Next change, skipped text not changed ***

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

X2AP-ELEMENTARY-PROCEDURES X2AP-ELEMENTARY-PROCEDURE ::= {

X2AP-ELEMENTARY-PROCEDURES-CLASS-1			|

X2AP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

X2AP-ELEMENTARY-PROCEDURES-CLASS-1 X2AP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

reset													|

x2Setup													|

resourceStatusReportingInitiation						|

eNBConfigurationUpdate									|

mobilitySettingsChange									|

cellActivation											|

seNBAdditionPreparation									|

meNBinitiatedSeNBModificationPreparation				|

seNBinitiatedSeNBModification							|

seNBinitiatedSeNBRelease								|

x2Removal												|

retrieveUEContext										|

sgNBAdditionPreparation									|

meNBinitiatedSgNBModificationPreparation					|

sgNBinitiatedSgNBModification							|

meNBinitiatedSgNBRelease									|

sgNBinitiatedSgNBRelease									|

sgNBChange												|

endcX2Setup												|

endcConfigurationUpdate									|

endcCellActivation										,

...

}

X2AP-ELEMENTARY-PROCEDURES-CLASS-2 X2AP-ELEMENTARY-PROCEDURE ::= {

snStatusTransfer						|

uEContextRelease						|

handoverCancel							|

errorIndication							|

resourceStatusReporting					|

loadIndication							|

privateMessage							|

rLFIndication							|

handoverReport							|

x2Release								|

x2APMessageTransfer						|

seNBReconfigurationCompletion			|

meNBinitiatedSeNBRelease				|

seNBCounterCheck						|

sgNBReconfigurationCompletion			|

sgNBCounterCheck							|

rRCTransfer								|

secondaryRATDataUsageReport				,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

snStatusTransfer X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNStatusTransfer

PROCEDURE CODE			id-snStatusTransfer

CRITICALITY				ignore

}

uEContextRelease X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextRelease

PROCEDURE CODE			id-uEContextRelease

CRITICALITY				ignore

}

handoverCancel X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverCancel

PROCEDURE CODE			id-handoverCancel

CRITICALITY				ignore

}

handoverReport X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverReport

PROCEDURE CODE			id-handoverReport

CRITICALITY				ignore

}

errorIndication X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

reset	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResetRequest

SUCCESSFUL OUTCOME		ResetResponse

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

x2Setup	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		X2SetupRequest

SUCCESSFUL OUTCOME		X2SetupResponse

UNSUCCESSFUL OUTCOME	X2SetupFailure

PROCEDURE CODE			id-x2Setup

CRITICALITY				reject

}

loadIndication X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		LoadInformation

PROCEDURE CODE			id-loadIndication

CRITICALITY				ignore

}

eNBConfigurationUpdate		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENBConfigurationUpdate

SUCCESSFUL OUTCOME		ENBConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	ENBConfigurationUpdateFailure

PROCEDURE CODE			id-eNBConfigurationUpdate

CRITICALITY				reject

}

resourceStatusReportingInitiation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				ResourceStatusRequest

SUCCESSFUL OUTCOME				ResourceStatusResponse

UNSUCCESSFUL OUTCOME			ResourceStatusFailure

PROCEDURE CODE					id-resourceStatusReportingInitiation

CRITICALITY						reject

}

resourceStatusReporting X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusReporting

CRITICALITY				ignore

}

rLFIndication X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RLFIndication

PROCEDURE CODE			id-rLFIndication

CRITICALITY				ignore

}

privateMessage			X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

mobilitySettingsChange	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MobilityChangeRequest

SUCCESSFUL OUTCOME		MobilityChangeAcknowledge

UNSUCCESSFUL OUTCOME	MobilityChangeFailure

PROCEDURE CODE			id-mobilitySettingsChange

CRITICALITY				reject

}

cellActivation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellActivationRequest

SUCCESSFUL OUTCOME		CellActivationResponse

UNSUCCESSFUL OUTCOME	CellActivationFailure

PROCEDURE CODE			id-cellActivation

CRITICALITY				reject

}

x2Release X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		X2Release

PROCEDURE CODE			id-x2Release

CRITICALITY				reject

}

x2APMessageTransfer X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		X2APMessageTransfer

PROCEDURE CODE			id-x2APMessageTransfer

CRITICALITY				reject

}

seNBAdditionPreparation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBAdditionRequest

SUCCESSFUL OUTCOME		SeNBAdditionRequestAcknowledge

UNSUCCESSFUL OUTCOME	SeNBAdditionRequestReject

PROCEDURE CODE			id-seNBAdditionPreparation

CRITICALITY				reject

}

seNBReconfigurationCompletion	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBReconfigurationComplete

PROCEDURE CODE			id-seNBReconfigurationCompletion

CRITICALITY				ignore

}

meNBinitiatedSeNBModificationPreparation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBModificationRequest

SUCCESSFUL OUTCOME		SeNBModificationRequestAcknowledge

UNSUCCESSFUL OUTCOME	SeNBModificationRequestReject

PROCEDURE CODE			id-meNBinitiatedSeNBModificationPreparation

CRITICALITY				reject

}

seNBinitiatedSeNBModification	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBModificationRequired

SUCCESSFUL OUTCOME		SeNBModificationConfirm

UNSUCCESSFUL OUTCOME	SeNBModificationRefuse

PROCEDURE CODE			id-seNBinitiatedSeNBModification

CRITICALITY				reject

}

meNBinitiatedSeNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBReleaseRequest

PROCEDURE CODE			id-meNBinitiatedSeNBRelease

CRITICALITY				ignore

}

seNBinitiatedSeNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBReleaseRequired

SUCCESSFUL OUTCOME		SeNBReleaseConfirm

PROCEDURE CODE			id-seNBinitiatedSeNBRelease

CRITICALITY				reject

}

seNBCounterCheck	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SeNBCounterCheckRequest

PROCEDURE CODE			id-seNBCounterCheck

CRITICALITY				reject

}

x2Removal	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		X2RemovalRequest

SUCCESSFUL OUTCOME		X2RemovalResponse

UNSUCCESSFUL OUTCOME	X2RemovalFailure

PROCEDURE CODE			id-x2Removal

CRITICALITY				reject

}

retrieveUEContext	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RetrieveUEContextRequest

SUCCESSFUL OUTCOME		RetrieveUEContextResponse

UNSUCCESSFUL OUTCOME	RetrieveUEContextFailure

PROCEDURE CODE			id-retrieveUEContext

CRITICALITY				reject

}

sgNBAdditionPreparation		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBAdditionRequest

SUCCESSFUL OUTCOME		SgNBAdditionRequestAcknowledge

UNSUCCESSFUL OUTCOME	SgNBAdditionRequestReject

PROCEDURE CODE			id-sgNBAdditionPreparation

CRITICALITY				reject

}

sgNBReconfigurationCompletion	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBReconfigurationComplete

PROCEDURE CODE			id-sgNBReconfigurationCompletion

CRITICALITY				ignore

}

meNBinitiatedSgNBModificationPreparation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBModificationRequest

SUCCESSFUL OUTCOME		SgNBModificationRequestAcknowledge

UNSUCCESSFUL OUTCOME	SgNBModificationRequestReject

PROCEDURE CODE			id-meNBinitiatedSgNBModificationPreparation

CRITICALITY				reject

}

sgNBinitiatedSgNBModification		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBModificationRequired

SUCCESSFUL OUTCOME		SgNBModificationConfirm

UNSUCCESSFUL OUTCOME	SgNBModificationRefuse

PROCEDURE CODE			id-sgNBinitiatedSgNBModification

CRITICALITY				reject

}

meNBinitiatedSgNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBReleaseRequest

SUCCESSFUL OUTCOME		SgNBReleaseRequestAcknowledge

UNSUCCESSFUL OUTCOME	SgNBReleaseRequestReject

PROCEDURE CODE			id-meNBinitiatedSgNBRelease

CRITICALITY				ignore

}

sgNBinitiatedSgNBRelease	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBReleaseRequired

SUCCESSFUL OUTCOME		SgNBReleaseConfirm

PROCEDURE CODE			id-sgNBinitiatedSgNBRelease

CRITICALITY				reject

}

sgNBCounterCheck	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBCounterCheckRequest

PROCEDURE CODE			id-sgNBCounterCheck

CRITICALITY				reject

}

sgNBChange	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SgNBChangeRequired

SUCCESSFUL OUTCOME		SgNBChangeConfirm

UNSUCCESSFUL OUTCOME	SgNBChangeRefuse

PROCEDURE CODE			id-sgNBChange

CRITICALITY				reject

}

rRCTransfer	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCTransfer

PROCEDURE CODE			id-rRCTransfer

CRITICALITY				reject

}

endcX2Setup X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCX2SetupRequest

SUCCESSFUL OUTCOME		ENDCX2SetupResponse

UNSUCCESSFUL OUTCOME	ENDCX2SetupFailure

PROCEDURE CODE			id-endcX2Setup

CRITICALITY				reject

}

endcConfigurationUpdate		X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCConfigurationUpdate

SUCCESSFUL OUTCOME		ENDCConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	ENDCConfigurationUpdateFailure

PROCEDURE CODE			id-endcConfigurationUpdate

CRITICALITY				reject

}

secondaryRATDataUsageReport	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SecondaryRATDataUsageReport

PROCEDURE CODE			id-secondaryRATDataUsageReport

CRITICALITY				reject

}

endcCellActivation	X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENDCCellActivationRequest

SUCCESSFUL OUTCOME		ENDCCellActivationResponse

UNSUCCESSFUL OUTCOME	ENDCCellActivationFailure

PROCEDURE CODE			id-endcCellActivation

CRITICALITY				reject

}

END

*** Next change, skipped text not changed ***

-- **************************************************************

--

#### -- Handover Report

--

-- **************************************************************

HandoverReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{HandoverReport-IEs}},

...

}

HandoverReport-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-HandoverReportType							CRITICALITY ignore	TYPE HandoverReportType									PRESENCE mandatory}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory}|

{ ID id-SourceCellECGI								CRITICALITY ignore	TYPE ECGI													PRESENCE mandatory}|

{ ID id-FailureCellECGI								CRITICALITY ignore	TYPE ECGI													PRESENCE mandatory}|

{ ID id-Re-establishmentCellECGI					CRITICALITY ignore	TYPE ECGI													PRESENCE conditional} -- The IE shall be present if the Handover Report Type IE is set to “HO to Wrong Cell” -- |

{ ID id-TargetCellInUTRAN							CRITICALITY ignore	TYPE TargetCellInUTRAN									PRESENCE conditional} -- The IE shall be present if the Handover Report Type IE is set to "InterRAT ping-pong" --|

{ ID id-SourceCellCRNTI								CRITICALITY ignore	TYPE CRNTI													PRESENCE optional}|

{ ID id-MobilityInformation							CRITICALITY ignore	TYPE MobilityInformation									PRESENCE optional}|

{ ID id-UE-RLF-Report-Container						CRITICALITY ignore	TYPE UE-RLF-Report-Container							PRESENCE optional}|

{ ID id-UE-RLF-Report-Container-for-extended-bands	CRITICALITY ignore	TYPE UE-RLF-Report-Container-for-extended-bands	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- CONDITIONAL HANDOVER MODIFICATION REQUEST

--

-- **************************************************************

ConditionalHandoverModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequest-IEs}},

...

}

ConditionalHandoverModificationRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID				CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID				CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause								PRESENCE mandatory}|

{ ID id-TargetCell-ID					CRITICALITY reject	TYPE ECGI								PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension	CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension	CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-UE-CHOContextInformation			CRITICALITY reject	TYPE UE-CHOContextInformation				PRESENCE mandatory},

...

}

UE-CHOContextInformation ::= SEQUENCE {

uEaggregateMaximumBitRate			UEAggregateMaximumBitRate		OPTIONAL,

subscriberProfileIDforRFP			SubscriberProfileIDforRFP		OPTIONAL,

uESecurityCapabilities				UESecurityCapabilities			OPTIONAL,

aS-SecurityInformation				AS-SecurityInformation			OPTIONAL,

e-RABs-ToBeSetup-List				E-RABs-ToBeSetup-List			OPTIONAL,

e-RABs-ToBeModified-List			E-RABs-ToBeSetup-List			OPTIONAL,

rRC-Context							RRC-Context						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {UE-CHOContextInformation-ExtIEs} } OPTIONAL,

...

}

UE-CHOContextInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeModified-ItemIEs} }

E-RABs-ToBeModified-ItemIEs 	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeModified-Item	 CRITICALITY ignore 	TYPE E-RABs-ToBeModified-Item 	PRESENCE mandatory },

...

}

E-RABs-ToBeModified-Item ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters					OPTIONAL,

dL-Forwarding					DL-Forwarding								OPTIONAL,

uL-GTPtunnelEndpoint			GTPtunnelEndpoint							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeSetup-ItemExtIEs} } OPTIONAL,

...

}

-- **************************************************************

--

#### -- CONDITIONAL HANDOVER MODIFICATION RESPONSE

--

-- **************************************************************

ConditionalHandoverModificationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextModificationResponseIEs} },

...

}

ConditionalHandoverModificationResponse-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID				CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID				CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension	CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension	CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-UE-CHOContextResponseInformation	CRITICALITY reject	TYPE UE-CHOContextResponseInformation	PRESENCE mandatory}|

{ ID id-E-RABs-NotAdmitted-List						CRITICALITY ignore	TYPE E-RAB-List		PRESENCE optional},

...

}

UE-CHOContextResponseInformation::= SEQUENCE {

e-RABs-Admitted-List		E-RABs-Admitted-List			OPTIONAL,

e-RABs-Modified-List		E-RABs-Modified -List			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {UE-CHOContextResponseInformation -ExtIEs} } OPTIONAL,

...

}

UE-CHOContextResponseInformation -ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

e-RABs-Modified-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Modified-ItemIEs} }

E-RABs-Modified-ItemIEs 	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Modified-List-Item	 CRITICALITY ignore 	TYPE E-RABs-Modified-Item 	PRESENCE mandatory },

...

}

E-RABs-Modified-Item ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

dL-Forwarding					DL-Forwarding								OPTIONAL,

uL-GTPtunnelEndpoint			GTPtunnelEndpoint							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeSetup-ItemExtIEs} } OPTIONAL,

...

}

-- **************************************************************

--

#### -- CONDITIONAL HANDOVER MODIFICATION FAILURE

--

-- **************************************************************

ConditionalHandoverModificationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { ConditionalHandoverModificationFailureIEs} },

...

}

ConditionalHandoverModificationFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID				CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension	CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

End Change