3GPP TSG-RAN3 Meeting #105bis	R3-195578

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | <CR#> | rev | <Rev#> | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for LTE_feMob BL CR for TS 36.300): CHO procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In previous meetings, it has been agreed to reuse existing HO Preparation procedure for CHO preparation, reuse existing HO cancel for canceling CHO from source, and introduce the Handover Success procedure to inform the source NG-RAN node that the UE has successfully attached to the target NG-RAN node, All the agreements have not been captured in TS36.300 for LTE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the text procedural for CHO preparation, CHO canceling and Conditional Handover Success procedure in TS36.300. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text procedural for CHO preparation, CHO canceling and Conditional Handover Success procedure in TS36.300 is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.2.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

#### 10.1.2.1a	Conditional Handover

Editor’s note: This section is a placeholder for CHO, and may be moved or removed depending on how RAN2 describes CHO, whether by a new section or within the section 10.1.2.1 for the legacy HO.

Conditional Handover (CHO) is defined as having a configured condition that determines when/whether the corresponding handover command is executed. Upon receiving the CHO configuration, UE starts to evaluate the condition(s) and executes the HO command once the condition is met.

##### 10.1.2.1a.1	C-plane handling

The figure below depicts the basic conditional handover procedure for UEs in ECM-CONNECTED:

# Figure 9.2.3.x.1-1: Intra-MME/ Serving Gateway CHO

0.	The UE context within the source eNB contains information regarding roaming and access restrictions which were provided either at connection establishment or at the last TA update.

1.	The source eNB configures the UE measurement procedures and the UE reports according to the measurement configuration.

2.	The source eNB makes CHO decision based on MeasurementReport and RRM information.

3.	The source eNB issues a Handover Request message which includes the CHO indidation to the candidate eNB passing a transparent RRC container with necessary information to prepare the handover at the candidate side. In CHO, the source eNB can issue multiple Handover Request messages to multiple candidate eNBs. And then, admission control may be performed by the multiple candidate eNBs.

4.	The candidate eNB prepares the conditional handover with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source eNB. In CHO, each candidate eNB can send the HANDOVER REQUEST ACKNOWLEDGE message to the source eNB.

FFS the detailed information in HANDOVER REQUEST ACKNOWLEDGE message.

5.	The source eNB triggers the Uu conditional handover by sending the RRC Connection Reconfiguration message which includes CHO configurations to the UE.

FFS the detailed information of CHO configurations e.g. CHO execution and candidate target cell configuration.

6.	The UE evaluates the CHO configurations and synchronises to the target cell which fulfils the corresponding CHO execution condition.

7.	The UE performs RACH procedure with the target cell and completes the conditional handover procedure by sending one RRC message to target eNB. FFS whether the RRC message can be RRCConnectionReconfigurationComplete message.

8.	When conditional handover procedure is complete, the target eNB sends the Handover Success message to inform the source eNB that the UE has successfully attached to the target eNB.

9.	When reveiving the Handover Success message, the source eNB sends the SN STATUS TRANSFER message to the target eNB.

FFS when data forwarding starts considering early data forwarding and late data forwarding.

Note 1: The Handover Cancel procedure can be used to enable a source eNB to cancel an ongoing conditional handover preparation or an already prepared conditional handover.

Note 2: The Conditional Handover Cancel procedure can be used to enable a target eNB to cancel an already prepared conditional handover.

10.	The target eNB sends a PATH SWITCH REQUEST message to MME to trigger EPC to switch the DL data path towards the target eNB and to establish an S1 interface instance towards the target eNB.

11.	The S-GW switches the DL data path towards the target eNB. The S-GW sends one or more "end marker" packets on the old path to the source eNB and then can release any U-plane/TNL resources towards the source eNB.

12.	The MME confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

13.	Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message from the MME, the target eNB sends the UE CONTEXT RELEASE to inform the source eNB about the success of the handover. The source eNB can then release radio and C-plane related resources associated to the UE context.

End Change