3GPP TSG-RAN WG3 #105bis	R3-195607

# Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0268 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.413 on NPN paging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN |  |  |  |  |  | Date: |  |  | 2019-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To support NG paging for NPN |  |  |  |  |  |  |  |  |
| Summary of change: |  | add a list of NIDs for SNPN and a list of CAG IDs for PNI-NPN in NGAP paging message to avoid paging UE at inappropriate NPN cells |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG paging for NPN is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.1.2 9.2.4.1 9.3.1.X 9.3.1.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.423,TS38.473,TS38.300 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2-1: Paging

The AMF initiates the Paging procedure by sending the PAGING message to the NG-RAN node.

At the reception of the PAGING message, the NG-RAN node shall perform paging of the UE in cells which belong to tracking areas as indicated in the TAI List for Paging IE.

If the Paging DRX IE is included in the PAGING message, the NG-RAN node shall use it according to TS 38.304 [12] and TS 36.304 [29].

For each cell that belongs to any of the tracking areas indicated in the TAI List for Paging IE, the NG-RAN node shall generate one page on the radio interface.

If the Paging Priority IE is included in the PAGING message, the NG-RAN node may use it according to TS 23.501 [9].

If the UE Radio Capability for Paging IE is included in the PAGING message, the NG-RAN node may use it to apply specific paging schemes.

If the Assistance Data for Recommended Cells IE is included in the Assistance Data for Paging IE it may be used, together with the Paging Attempt Information IE if also present, according to TS 38.300 [8].

If the Next Paging Area Scope IE is included in the Paging Attempt Information IE it may be used for paging the UE according to TS 38.300 [8].

If the Paging Origin IE is included in the PAGING message, the NG-RAN node shall transfer it to the UE according to TS 38.331 [18] and TS 36.331 [21].

If the CAG List for Paging IE is included in the PAGING message, the NG-RAN node may use it for paging the UE according to TS 38.300 [8].

If the NID List for Paging IE is included in the PAGING message, the NG-RAN node may use it for paging the UE according to TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

#### 9.2.4.1	PAGING

This message is sent by the AMF and is used to page a UE in one or several tracking areas.

Direction: AMF  gNB

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.3.1.1 |  | YES | ignore |
| UE Paging Identity | M |  |  | 9.3.3.18 |  | YES | ignore |
| Paging DRX | O |  |  | 9.3.1.90 |  | YES | ignore |
| TAI List for Paging |  |  | 1 |  |  | YES | ignore |
| >TAI List for Paging Item |  |  | 1..<maxnoofTAIforPaging> |  |  | - |  |
| >>TAI | M |  |  | 9.3.3.11 |  | - |  |
| Paging Priority | O |  |  | 9.3.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  |  | 9.3.1.68 |  | YES | ignore |
| Paging Origin | O |  |  | 9.3.3.22 |  | YES | ignore |
| Assistance Data for Paging | O |  |  | 9.3.1.69 |  | YES | ignore |
| CAG List for Paging |  |  | 0..1 |  |  | YES | ignore |
| >CAG List for Paging Item |  |  | 1..<maxnoofCAGsPerUE> |  |  |  |  |
| >>CAG ID | M |  |  | 9.3.1.X |  |  |  |
| NID List for paging |  | 0..1 |  |  | YES | ignore |
| >NID List for Paging Item |  | 1..<maxnoofNIDsPerUE> |  |  |  |  |
| >> NID ID | M |  | 9.3.1.Y |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforPaging | Maximum no. of TAIs for paging. Value is 16. |
| maxnoofCAGsPerUE | Maximum no. of CAGs per UE per PLMN. Value is pending to RAN2. |
| maxnoofNIDsPerUE | Maximum no. of NIDs per UE per PLMN. Value is pending to RAN2. |

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.X	CAG ID

This information element indicates the identifier of the Closed Access Group.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CAG ID | M |  | FFS | Pending to RAN2. |

9.3.1.Y	NID

This information element  indicates the Network identifier (NID) of the SNPN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NID | M |  | FFS | Pending to RAN2. |

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>