3GPP TSG-RAN WG3 Meeting #105bis	R3-195620

# Chongqing, China, 14th - 18th Oct 2019

Agenda item:	10.2.1

Source: 	ZTE

Title: 	Mobility Settings Change procedure for MRO/MLB

Document for:	Discussion and Approval

Introduction

According to the analysis in R3-195619, the following proposals are proposed:

Proposal1: The Mobility Settings Change procedure needs to be introduced in NR for MRO and MLB.

Proposal2: The cell reselection settings should also be allowed to be adjusted via Mobility Settings Change procedure.

The corresponding stage2 TP for TS38.300 is provided as below:

*** First change, ommited text not changed ***

# 15.x Self-Optimization

# 15.x.y Adapting handover and/or reselection configuration

This function enables requesting of a change of handover and/or reselection parameters at target cell. The source cell estimates if it is needed to change mobility configuration in the source and/or target cell due to load balance or mobility performance. If the amendment is needed, the source cell initializes mobility negotiation procedure toward the target cell.

The source cell informs the target cell about the new mobility setting and provides cause for the change (e.g. load balancing related request). The proposed change is expressed by the means of the difference (delta) between the current and the new values of the handover trigger. The handover trigger is the cell specific offset that corresponds to the threshold at which a cell initialises the handover preparation procedure. Cell reselection configuration may be amended to reflect changes in the HO setting, e.g., the Qoffset can be used to be negotiated between cells to help to balance idle mode UE offload. The target cell responds to the information from the source cell. The allowed delta range for HO trigger parameter may be carried in the failure response message. The source cell should consider the responses before executing the planned change of its mobility setting.

All automatic changes on the HO and/or reselection parameters must be within the range allowed by OAM.