mo3GPP TSG-RAN WG3 Meeting #105bis	R3-195633

# Chongqing, P.R. China, October 14th – 18th 2019

Agenda Item:	13.2.1.3

Source:	Ericsson, KDDI

Title:	IP Address Assignment in IAB

Document for:	Agreement

# Introduction

IP address assignment for IAB nodes is still under discussion in RAN3. Although RAN3 normally does not specify IP address allocation methods, it seems that majority of companies in RAN3 think that some description may be necessary.

In several contributions to RAN3#105 meeting it was proposed that the IAB-donor CU should perform the IP address assignment to the IAB nodes and signal the assigned IP address via RRC. In our view, allocation of IAB IP address by the IAB-donor CU has several issues which have been overlooked by the proponents.

This contribution explains why IAB IP address allocation by the IAB-donor CU is inadequate and proposes a way forward.

# Discussion

Two options for IAB IP address assignment are currently being discussed in RAN3, with the main difference with respect to which entity allocates the IP addresses:

- IP address assignment by the IAB-donor CU: in this option, the IAB-donor CU-CP assigns the IP address to the IAB node, using the newly-established RRC connectivity of the IAB-MT.

- IP address assignment via (by) IAB-donor DU: in this option, the IAB-donor DU acts as a DHCP server or DHCP proxy.

In several contributions to RAN3#105 meeting it was argued that allocating the IP address by the IAB-donor CU-CP is a simple and efficient solution. In our view, this is not as simple as the proponents claim, as discussed below.

## Triggering of IP address assignment

As discussed in R3-193739, 3GPP gNB-DUs can support any number of IP addresses, e.g. multiple IP addresses for OAM, F1-C, F1-U etc. In that respect, there is no reason why the same principle should not apply for IAB-DUs as well. This is important because only the IAB node (as “normal” DUs) can know how many IP addresses it requires, and it is not the responsibility of the IAB-donor DU or IAB-donor CU to decide this.  So, regardless of whether the allocation of the IP addresses is done by the IAB-donor DU or IAB-donor CU, this should be upon request from the IAB node, or it should be set by the OAM system, which knows how many IP addresses are needed for the IAB node.

Observation 1: Currently, a 3GPP gNB-DU can support multiple IP addresses, which implies that the IAB-DU can also acquire/support multiple IP addresses.

Observation 2: Allocating multiple IP addresses to IAB-DU by the IAB-donor-CU will be difficult, since the IAB-donor CU needs to know when and how many IP addresses it should allocate to IAB-DU.

Observation 3: It is more plausible that the IAB-DU requests the IP addresses from the IAB-donor, compared to the IAB-donor deciding/triggering the IP address allocation for the IAB-DU.

Proposal 1: Regardless of which solution is used to allocate and signal the IP addresses to the IAB node, the triggering for the allocation should, as for normal DUs, come from the IAB-DU or the OAM system of the IAB-DU.

## Lack of transport network topology knowledge by the CU

As of today, the CU is not in charge of allocating IP addresses to DUs. Namely, all that a CU needs to do is to start an SCTP server and listen for SCTP setup requests from DUs coming from any IP address, from which the CU will learn the DUs IP address. Similar to legacy CUs, it is expected that the IAB-donor CU will be located in a central cloud environment and lack the knowledge about which IP addresses should be used in the distributed part of the network. This makes CU unsuited for allocating the IP addresses to IAB nodes.

Observation 4: The IAB-donor CU may not be in a good place to allocate IP addresses to IAB nodes, e.g. it could be located in a central cloud environment and lack the knowledge about which IP addresses should be used in the distributed part of the network.

The knowledge of IP transport network (i.e., L3) should not be confused with the IAB-donor CU’s responsibility for IAB network topology (i.e., L2). In other words, the fact that IAB-donor CU oversees L2 IAB network topology does not mean that the IAB-donor CU knows about the L3 transport network topology. Introducing this new responsibility on the CU could have a large impact on the operation of CUs, which contradicts one of the guiding principles of IAB standardization work – the reuse of existing functionalities.

Observation 5: Currently, it is not the responsibility of the IAB-donor CU to know the transport network topology. Introducing such functionality could have a large impact on the operation of CUs, which contradicts one of the guiding principles of IAB standardization work – the reuse of existing functionalities.

Should the CU be responsible for allocating the IP addresses to distributed DUs or IAB nodes located somewhere else in the IP network, a significant update to mechanism used to allocate IP addresses to the CU, such as DHCP, cloud orchestration would be required. Therefore, it seems reasonable that even if the “IP allocation signaling” involves the IAB-donor CU, the IP address(es) most likely come from somewhere else.

Observation 6: The mechanism to allocate the IP address to the CU e.g. DHCP or cloud orchestration, are most likely not suitable for allocating IP addresses to distributed DUs or IAB nodes located somewhere else in the IP network.

Proposal 2: Even if RAN3 adopts a solution where the IAB-donor CU is involved in the signalling to allocate the IP address(es) to the IAB node, the actual IP address(es) should come from some other node i.e. a node that knows the IP transport topology and knows which IP addresses are available where the IAB node is located.

For the existing CU-DU split, the CU does not have the functionality to allocate IP address to the DUs or to know anything about the transport topology. Given the arguments above and the desire to maximize the similarities between IAB-DU nodes and normal DUs, i.e., avoid IAB specific impacts to CUs, we should not change these principles for IAB nodes.

Observation 7: Making the CU responsible for handling IP address management of DUs for the sake of IAB would lead to a different CU-DU functional split for IAB nodes which is not desirable.

Proposal 3: The principle that the CU is not responsible for IP address management of the DUs should be kept also for IAB.

## Efficiency at topology change

One of the arguments for using IAB-donor CU and RRC to allocate the IP addresses to IAB nodes is that this would be a more efficient approach during topology change. However, in our view, this efficiency can be attained even without IAB-donor CU allocating the IP address. To get the best performance during topology change, the IAB-donor CU should ideally allocate the resources, set up routing, etc. in the target nodes (e.g. target IAB parent, target IAB-donor DU) prior to executing the IAB node handover.

Observation 8: To get the best performance at a topology change, the IAB-donor CU should ideally allocate resources, set up routing, etc. in the target nodes (e.g. target IAB parent, target IAB-donor DU) prior to executing the IAB node handover.

As such, it would be equally efficient to adopt a solution where the IAB-donor DU, who knows local IP topology, allocates the IP addresses and then signal this to the IAB-donor CU during handover preparation for the IAB-donor CU, which then signals this to the migrating IAB node.

Observation 9: Allocating IP addresses in the target IAB-donor DU during handover preparation for topology change would be equally efficient as allocating IP addresses at the IAB-donor CU.

# Way forward

Given that multiple companies support the allocation of IP addresses via the CU/RRC we propose a way forward that could be supported under the following assumptions:

- OAM-based solutions for IP address assignment are also supported.

- The triggering of the initial allocation of IP addresses is done from the IAB node to the IAB-donor CU, making it possible for IAB node to request IP addresses when needed.

- The IAB node should be allowed to request multiple IP addresses.

- The actual allocation of the IP addresses is done by the IAB-donor DU, meaning that the IAB-donor CU will, as part of F1 MT context setup/modification, request IP addresses. In this way, the mapping between BAP routing IDs and IP addresses can also be established.

- The re-allocation of IP address at mobility can be done at the IAB-donor DU, when the IAB-donor CU establishes or modifies the MT context in the IAB-donor DU.

Proposal 4: RAN3 to agree that, in addition to OAM-based method for IP address assignment for IAB nodes, it should also be possible to support the following mechanism:

- The triggering of the initial allocation of IP addresses is done from the IAB node to the IAB-donor CU, making it possible for IAB node to request IP addresses when needed.

- The IAB node should be allowed to request multiple IP addresses.

- The actual allocation of the IP addresses is done by the IAB-donor DU, meaning that the IAB-donor CU will, as part of F1 MT context setup/modification, request IP addresses. In this way the mapping between BAP routing IDs and IP addresses can also be established.

- The re-allocation of IP address at mobility can be done at the IAB-donor DU, when the IAB-donor CU establish or modifies the MT context in the IAB-donor DU.

The high-level description of the above method is given below:

- When needed, an IAB node triggers IP address allocation by sending an RRC message to the IAB-donor CU, indicating how many IP addresses are requested.

- IAB-donor CU receives the RRC message and forwards the IP address allocation request to IAB-donor DU inside an F1AP message. In addition to the indication of how many IP addresses are requested, the F1AP message also includes an equal number of BAP routing IDs, each one corresponding to exactly one IP address to be allocated.

- Based on the received request, IAB-donor DU allocates the required number of IP addresses and maps them to the corresponding BAP routing IDs received from the IAB-donor CU. How the IAB-donor DU allocates the IP addresses can be up to implementation. For example:

  - IAB-donor DU may act as a DHCP proxy and use DHCP (and virtual MAC addresses, if necessary) to obtain the IP addresses from a DHCP server, or

  - IAB-donor DU may manage a pool of IP addresses assigned by OAM, or

  - IAB-donor DU may be allowed to allocate IP addresses based on its own IP address prefix.

- The IAB-donor DU replies to IAB-donor CU, where the F1AP message contains the mapping between the BAP routing IDs and their respective newly-allocated IP addresses.

- The IAB-donor CU informs the IAB node about the newly-allocated BAP routing IDs and their respective IP addresses via RRC.

Proposal 5: RAN3 to agree the method for IAB IP address allocation described above and specify the corresponding F1AP signaling.

Proposal 6: RAN3 to send an LS to RAN2, requesting to accommodate the above mechanism in the RRC.

# Conclusion

In this contribution we discuss the issue of IP address management for IAB-nodes, and make the following observations:

Observation 1: Currently, a 3GPP gNB-DU can support multiple IP addresses, which implies that the IAB-DU can also acquire/support multiple IP addresses.

Observation 2: Allocating multiple IP addresses to IAB-DU by the IAB-donor-CU will be difficult, since the IAB-donor CU needs to know when and how many IP addresses it should allocate to IAB-DU.

Observation 3: It is more plausible that the IAB-DU requests the IP addresses from the IAB-donor, compared to the IAB-donor deciding/triggering the IP address allocation for the IAB-DU.

Observation 4: The IAB-donor CU may not be in a good place to allocate IP addresses to IAB nodes, e.g. it could be located in a central cloud environment and lack the knowledge about which IP addresses should be used in the distributed part of the network.

Observation 5: Currently, it is not the responsibility of the IAB-donor CU to know the transport network topology. Introducing such functionality could have a large impact on the operation of CUs, which contradicts one of the guiding principles of IAB standardization work – the reuse of existing functionalities.

Observation 6: The mechanism to allocate the IP address to the CU e.g. DHCP or cloud orchestration, are most likely not suitable for allocating IP addresses to distributed DUs or IAB nodes located somewhere else in the IP network.

Observation 7: Making the CU responsible for handling IP address management of DUs for the sake of IAB would lead to a different CU-DU functional split for IAB nodes which is not desirable.

Observation 8: To get the best performance at a topology change, the IAB-donor CU should ideally allocate resources, set up routing, etc. in the target nodes (e.g. target IAB parent, target IAB-donor DU) prior to executing the IAB node handover.

Observation 9: Allocating IP addresses in the target IAB-donor DU during handover preparation for topology change would be equally efficient as allocating IP addresses at the IAB-donor CU.

Based on the observations, we propose the following:

Proposal 1: Regardless of which solution is used to allocate and signal the IP addresses to the IAB node, the triggering for the allocation should, as for normal DUs, come from the IAB-DU or the OAM system of the IAB-DU.

Proposal 2: Even if RAN3 adopts a solution where the IAB-donor CU is involved in the signalling to allocate the IP address(es) to the IAB node, the actual IP address(es) should come from some other node i.e. a node that knows the IP transport topology and knows which IP addresses are available where the IAB node is located.

Proposal 3: The principle that the CU is not responsible for IP address management of the DUs should be kept also for IAB.

Proposal 4: RAN3 to agree that, in addition to OAM-based method for IP address assignment for IAB nodes, it should also be possible to support the following mechanism:

- The triggering of the initial allocation of IP addresses is done from the IAB node to the IAB-donor CU, making it possible for IAB node to request IP addresses when needed.

- The IAB node should be allowed to request multiple IP addresses.

- The actual allocation of the IP addresses is done by the IAB-donor DU, meaning that the IAB-donor CU will, as part of F1 MT context setup/modification, request IP addresses. In this way the mapping between BAP routing IDs and IP addresses can also be established.

- The re-allocation of IP address at mobility can be done at the IAB-donor DU, when the CU establish or modifies the MT context in the IAB-donor DU.

Proposal 5: RAN3 to agree the method for IAB IP address allocation described above and specify the corresponding F1AP signaling.

Proposal 6: RAN3 to send an LS to RAN2, requesting to accommodate the above mechanism in the RRC.