3GPP TSG-RAN WG3 Meeting #105bis	R3-195668

Chongqing, China, October 14th – 18th, 2019

Agenda item:		15.2.3

Source:	Intel Corporation

Title:	(TP for NR_Mob_enh-Core BL CR for TS 38.423): Data Forwarding and SN Status Transfer for 0ms Interruption

Document for:		Other

# 1		Introduction

This contribution proposes TP for TS 38.423 based on enhancements proposed for RUDI (Reduction in User Data Interruption) data forwarding and SN status continuation as discussed in [1].

# 2		Reference

[1] R3-195664, “Data Forwarding and SN Status Transfer for 0ms Interruption”, Intel Corporation

# 3		TP for BL CR for TS 38.423

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.2.2	SN Status Transfer

#### 8.2.2.1	General

The purpose of the SN Status Transfer procedure is to transfer the uplink PDCP SN and HFN receiver status and the downlink PDCP SN and HFN transmitter status either, from the source to the target NG-RAN node during an Xn handover, between the NG-RAN nodes involved in dual connectivity, or after retrieval of a UE context for RRC reestablishment, for each respective DRB of the source DRB configuration for which PDCP SN and HFN status preservation applies.

In case that the Xn handover is a non-split bearer handover, the SN Status Transfer procedure may also be used to transfer the uplink HFN receiver status, or the downlink PDCP SN and HFN transmitter status for a DRB associated with RLC-UM as in TS 38.300 [9].

Editor’s note: FFS whether RUDI HO is supported for RLC-UM. Pending RAN2.

If the SN Status Transfer procedure is applied in the course of dual connectivity or RRC connection re-establishment in the subsequent specification text

-	the behaviour of the NG-RAN node from which the DRB context is transferred, i.e. the NG-RAN node involved in dual connectivity or RRC connection re-establishment, from which data is forwarded, is specified by the behaviour of the "source NG-RAN node",

-	the behaviour of the NG-RAN node to which the DRB context is transferred, i.e., the NG-RAN node involved in dual connectivity or RRC connection re-establishment, to which data is forwarded, is specified by the behaviour of the "target NG-RAN node".

The procedure uses UE-associated signalling.

#### 8.2.2.2	Successful Operation

Figure 8.2.2.2-1: SN Status Transfer, successful operation

The source NG-RAN node initiates the procedure by stop assigning PDCP SNs to downlink SDUs and stop delivering UL SDUs towards the 5GC and sending the SN STATUS TRANSFER message to the target NG-RAN node at the time point when it considers the transmitter/receiver status to be frozen. The target NG-RAN node using full configuration for this handover as per TS 38.300 [9] shall ignore the information received in this message.

In case that the Xn handover is a non-split bearer handover, the source NG-RAN node may continue assigning PDCP SNs to downlink SDUs and delivering uplink SDUs toward the 5GC when initiating this procedure as in TS 38.300 [9].

For each DRB in the DRBs Subject to Status Transfer List IE, the source NG-RAN node shall include the DRB ID IE, the UL COUNT Value IE and the DL COUNT Value IE.

Editor’s note: FFS whether RUDI HO is supported for RLC-UM. Pending RAN2.

The source NG-RAN node may also include in the SN STATUS TRANSFER message the missing and the received uplink SDUs in the Receive Status of UL PDCP SDUs IE for each DRB for which the source NG-RAN node has accepted the request from the target NG-RAN node for uplink forwarding.

For each DRB in the DRBs Subject to Status Transfer List IE, the target NG-RAN node shall not deliver any uplink packet which has a PDCP-SN lower than the value contained within the UL Count Value IE.

For each DRB in the DRBs Subject to Status Transfer List IE, the target NG-RAN node shall use the value of the PDCP SN contained within the DL COUNT Value IE for the first downlink packet for which there is no PDCP-SN yet assigned. In case that the Xn handover is a non-split bearer handover, the target NG-RAN node shall use the value of the DL COUNT Value IE conveyed for the first time for a DRB, as the PDCP SN and HFN of the first PDCP SDU that the source NG-RAN nodes forwards to the target NG-RAN node.

If the Receive Status of UL PDCP SDUs IE is included for at least one DRB in the SN STATUS TRANSFER message, the target NG-RAN node may use it in a Status Report message sent to the UE over the radio interface.

#### 8.2.2.3	Unsuccessful Operation

Not applicable.

#### 8.2.2.4	Abnormal Conditions

If the target NG-RAN node receives this message for a UE for which no prepared handover exists at the target NG-RAN node, the target NG-RAN node shall ignore the message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.2.X	Handover Success

#### 8.2.X.1	General

The Handover Success procedure is used during a conditional handover or a non-split bearer handover, to enable a target NG-RAN node to inform the source NG-RAN node that the UE has successfully accessed the target NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.2.X.2	Successful Operation

Figure 8.2.X.2-1: Handover Success, successful operation

The target NG-RAN node initiates the procedure by sending the HANDOVER SUCCESS message to the source NG-RAN node.

Interactions with other procedures

If a CONDITIONAL HANDOVER CANCEL message was received for this UE prior the reception of the HANDOVER SUCCESS message, the source NG-RAN node shall consider that the UE successfully executed the handover. The source NG-RAN node shall ignore the CONDITIONAL HANDOVER CANCEL message and shall start data forwarding if late data forwarding was configured for this UE.

Editor’s note: FFS whether the target NG-RAN node can indicate the cell ID that the UE has successfully attached to.

Editor's note: The impact on the NG-RAN node's behaviour should be verified if the source NG-RAN node receives the HANDOVER SUCCESS message for a UE from the target NG-RAN node after receiving the CONDITIONAL HANDOVER CANCEL message for the same UE from the same target NG-RAN node.

#### 8.2.X.3	Unsuccessful Operation

Not applicable.

#### 8.2.X.4	Abnormal Conditions

If the HANDOVER SUCCESS message refers to a context that does not exist, the source NG-RAN node shall ignore the message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.1.14	DRBs Subject To Status Transfer List

This IE contains a list of DRBs containing information about PDCP PDU transfer status.

| IE/Group Name | Presence | Range |  | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DRBs Subject To Status Transfer Item |  | 1 .. <maxnoofDRBs> |  |  |  |  |  |
| >DRB ID | M |  |  | 9.2.3.33 |  | – |  |
| >CHOICE PDCP Status Transfer UL | M |  |  |  |  | – |  |
| >>12 bits |  |  |  |  |  |  |  |
| >>>Receive Status Of PDCP SDU | O |  |  | BIT STRING (1.. 2048) | The IE is used in case of 12-bit long PDCP-SN. The first bit indicates the status of the SDU after the First Missing UL PDCP SDU. The Nth bit indicates the status of the UL PDCP SDU in position (N + First Missing SDU Number) modulo (1 + the maximum value of the PDCP-SN). 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | – |  |
| >>>UL COUNT Value | M |  |  | COUNT Value for PDCP SN Length 12 9.2.3.36 | PDCP-SN and Hyper Frame Number of the first missing UL SDU in case of 12-bit long PDCP-SN | – |  |
| >>18 bits |  |  |  |  |  |  |  |
| >>>Receive Status Of PDCP SDU | O |  |  | BIT STRING (1.. 131072) | The IE is used in case of 18-bit long PDCP-SN. The first bit indicates the status of the SDU after the First Missing UL PDCP SDU. The Nth bit indicates the status of the UL PDCP SDU in position (N + First Missing SDU Number) modulo (1 + the maximum value of the PDCP-SN). 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | – |  |
| >>>UL COUNT Value | M |  |  | COUNT Value for PDCP SN Length 18 9.2.3.37 | PDCP-SN and Hyper Frame Number of the first missing UL SDU in case of 18-bit long PDCP-SN | – |  |
| >CHOICE PDCP Status Transfer DL | M |  |  |  |  | – |  |
| >>12 bits |  |  |  |  |  |  |  |
| >>>DL COUNT Value | M |  |  | COUNT Value for PDCP SN Length 12 9.2.3.36 | PDCP-SN and Hyper Frame Number that the target NG-RAN node (handover) or the NG-RAN node to which the DRB context is transferred (dual connectivity) should assign for the next DL SDU not having an SN yet in case of 12-bit long PDCP-SN. In case of non-split bearer handover, indicates PDCP SN and Hyper Frame Number of the first PDCP SDU that the source NG-RAN nodes forwards to the target NG-RAN node, if conveyed for the first time for a DRB. | – |  |
| >>18 bits |  |  |  |  |  |  |  |
| >>>DL COUNT Value | M |  |  | COUNT Value for PDCP SN Length 18 9.2.3.37 | PDCP-SN and Hyper Frame Number that the target NG-RAN node (handover) or the NG-RAN node to which the DRB context is transferred (dual connectivity) should assign for the next DL SDU not having an SN yet in case of 18-bit long PDCP-SN. In case of non-split bearer handover, indicates PDCP SN and Hyper Frame Number of the first PDCP SDU that the source NG-RAN nodes forwards to the target NG-RAN node, if conveyed for the first time for a DRB. | – |  |
| >Old QoS Flow List - UL End Marker expected | O |  |  | QoS Flow List 9.2.1.4a | This IE is included to be used for indicating that the source NG-RAN node has initiated QoS flow re-mapping and has not yet received SDAP end markers, as described in TS 38.300 [8]. It shall only contain UL QoS flow information for QoS flows for which no SDAP end marker has been yet received on the source side. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |