3GPP TSG-RAN WG3 #105bis								R3-195704

Chongqing, China, 14-18 October 2019

Agenda Item:	31.3.6

Source:	China Telecom

Title: 	Support of inter-system HO from SA to EN-DC in Rel-16

Document for:	Discussion

Introduction

The most operators will deploy EN-DC to support eMBB service in their first stage of 5G deployment, and after that state, part EN-DC base station may be upgraded to NR SA or NSA/SA dual mode via software update. So when NSA/SA dual mode UE moves out of NR SA cell coverage, direct configuration of EN-DC during inter-system handover can reduce the DC configuration latency and quickly recover user’s data rate after HO procedure. From the operator’s point of view, it would be beneficial to support SA to EN-DC in NR practical network. However, the NR SA to EN-DC HO is not supported in Rel-15 for time limitations [1]. The intention of this contribution is to discuss the scenario and gives some analysis on the RAN3 impact.

Discussion

Scenarios and Requirements

The main driver for employing EN-DC, is to utilize the extended bandwidth provided by NR spectrum, while keeping the control signalling on LTE. So EN-DC is a good solution to provide high date rate for 2C service in first stage of 5G deployment. In order to provide 2B service, such as uRLLC service, in certain areas, the operators will consider to upgrade part of en-gNBs to support EN-DC and SA alone access simultaneously. So there is a boundary between SA and NSA region for the NSA/SA dual mode UE. As per Rel-15 specification, when the NSA/SA dual mode UE moves out of NR SA cell coverage or triggers EPS fallback, it would be handover to LTE firstly and then perform subsequent SCG reconfigurations. This will result in more control plane messages and a larger service interruption than direct HO from NR SA to EN-DC. Specially for the scenario of EPS fallback, the eMBB service has to be also handover to a LTE cell with the voice service. We must say that this is not a perfect solution. In principle, the eMBB service shall be kept in the source NR cell during the EPS fallback procedure to achieve better experience.

In Rel-15, the HO from NR SA to EN-DC is not supported due to time limitations. Now the NR to LTE Handover has been fully supported in Rel-15. From the technical point of view, there are no obstacles for supporting NR SA to EN-DC HO in Rel-16. We also have a RAN2 contribution to suggest the support of NR SA to EN-DC in Rel-16 [2].

Proposal 1: The NR SA to EN-DC handover shall be supported in Rel-16.

Figure 1 UE moves out of SA coverage to EN-DC

RAN3 Impact Analysis

UE capability: The feature of NR SA to EN-DC HO may be not support for partial commercial NR UE. So this function may need to introduce a new UE capability indication for NR in RAN2. If RAN2 agree with the introduction of this new UE capability, it can be contained in a RRC Container during the HO procedure. Therefore, introduction of new UE capability is no impact to RAN3.

Assistance information to configure the source gNB as the target SN: Currently, the VoNR is not supported in most NR base station. For voice service, the NR SA cell shall trigger EPS fallback to handover this UE to the LTE cell. The LTE eNB need to know this UE is still in the NR coverage to configure the source gNB as the target SN because there may be no measurement report for the source gNB contained in HO request message. In order to resolve this issue, a possible solution is as follows:

- The source gNB add an Assistance information, e.g., EPS fallback indicator, in RRC container convey on HO required message. The target eNB can derive the source gNB as the SN based on the following information:

  - the coverage relationship between the target eNB and the source gNB: this is up to the NW implementation

  - Assistance information

Proposal 2: There is no RAN3 impact to introduce the SA to NSA directly HO in Rel-16

3	Proposal

The intention of this contribution is to discuss the scenario and requirement for the support of NR SA to EN-DC in Rel-16, and gives some analysis on the RAN3 impact.

Proposal 1: The NR SA to EN-DC handover shall be supported in Rel-16.

Proposal 2: There is no RAN3 impact to introduce the SA to NSA directly HO in Rel-16

4	Reference

- R2-1907085, ZTE Corporation, Sanechips, Inter-RAT HO support

- R2-1912447, China Telecom, Support of inter-RAT HO from SA to EN-DC in Rel-16