3GPP TSG-RAN3 Meeting #105bis	R3-195763

Chongqing, China, 14-18 October 2019

Title: 	Initial UE message for NPN

Source: 	Huawei

Agenda item:	16.2.2.1

Document for:	Discussion

1. Introduction

The non-public network (NPN) was discussed during RAN3#105 meeting. In [1], the agreements on stand-alone NPN (SNPN) and public network integrated NPN (PNI-NPN) were summarized while the following remains open.

#### 2/ Access control, Initial UE Message

#### 2.1/ PNI NPN

#### a/ need to add selected CAG ID to the Initial UE message?

#### (Likely but to be confirmed due to ongoing discussions on privacy in MSG5)

#### (If CAG ID not in msg5, pre-check? then maybe list of cell CAG IDs to be sent in initial UE message?)

#### b/ after verification by AMF, new cause in release message?

#### 2.2 SNPN

#### After verification in AMF, new cause in release message?

Further SA2 has sent a LS on sending CAG ID over NAS in [2] with the following question.

- SA3 asks SA2, RAN2 and RAN3 groups to provide feedback on sending CAG ID in the NAS layer rather than the AS layer

In this document we discuss issues on Initial UE Message for NPN.

2. Discussion

2.1 PNI-NPN

For the state transition from CM-IDLE to CM-CONNECTED, it is described in [2] as follows:

-	During transition from CM-IDLE to CM-CONNECTED, if the UE is accessing the 5GS via a CAG cell, the UE shall provide the selected CAG Identifier to NG-RAN and the NG-RAN shall provide the CAG Identifier to the AMF:

-	The AMF shall verify whether UE access is allowed by Mobility Restrictions:

-	If the CAG Identifier received from the NG-RAN is part of the UE's Allowed CAG list, then the AMF accepts the NAS request;

-	If the CAG Identifier received from the NG-RAN is not part of the UE's Allowed CAG list, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE removes that CAG Identifier, if it exists, from its Allowed CAG list, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure; and

-	If the UE is accessing the network via a non-CAG cell and the UE's subscription contains an indication that the UE is only allowed to access CAG cells, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE updates its local configuration, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure.

It can be observed that the AMF is responsible for access control and verify the UE access based on the CAG Identifier provided by the NG-RAN during the initial UE access.

The CAG Identifier should be reported to AMF for the UE verification.

Whether to add the CAG ID into Initial UE Message depends on how to report the UE selected CAG ID to the AMF. Regarding to this, there are two options:

- Option 1: UE reports the selected CAG ID to the AMF by the NAS signalling, e.g., the selected CAG ID is included in the NAS container in MSG5.

- Option 2: UE reports the selected CAG ID to the AMF via the NG-RAN, i.e., the UE first reports the selected CAG ID to the NG-RAN, and then the NG-RAN sends it to the AMF.

For Option 1, this is specified as one possible way by SA3 as stated in [2] as follows

One proposal under discussion in SA3 is to not have the CAG ID sent in AS signalling but include it in NAS signalling instead. This means that the CAG ID can be protected by existing NAS signalling security mechanisms for example.

Hence the advantage is that CAG ID can be protected by existing NAS signalling security mechanisms. But the NAS security in Msg5 cannot be activated during attach procedure. Also the discussion may be related to the one whether the NG-RAN should be aware of the CAG ID.

We understands that there is no showstopper for Option 1 from RAN3 perspective. Now that the CAG ID has been included in NAS signalling, there is no need to add CAG ID into Initial UE Message.

RAN3 understands that the CAG ID is not added into Initial UE Message.

As described in SA2 that:

-	If the CAG Identifier received from the NG-RAN is not part of the UE's Allowed CAG list, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE removes that CAG Identifier, if it exists, from its Allowed CAG list, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure;

This may happen in case that the allowed CAG list for UE is changed in the 5GC but not inform to the UE yet, then the UE may select an invalid CAG ID, which is included in the allowed CAG list stored in the UE and has been removed in the updated allowed CAG list in the 5GC, during the registration management procedure (e.g., for update the allowed CAG list). In this case, the AMF will initiate an ERROR INDICATION message to the NG-RAN, which includes an appropriate cause value, i.e., invalid CAG ID.

On the other hand, if a UE, which is only allowed to access CAG cells, requests to access a non-CAG cell, the AMF shall reject this request and initiate an ERROR INDICATION message to the NG-RAN with an appropriate cause value, i.e., only CAG cells is allowed.

New cause values which are equal to “invalid CAG ID” and “only CAG cell is allowed” should be introduced on NG interface.

2.2 SNPN

For SNPN, in RAN3#105, it was agreed that in [1],

Add NID into the Initial UE Message.

Similar to PNI-NPN, the AMF is responsible for access control and verify the UE access based on the subscription. The AMF will initiate an ERROR INDICATION message to the NG-RAN if the selected NID is not permitted, with an appropriate cause value, e.g., invalid NID.

New cause values which are equal to “invalid NID” should be introduced on NG interface.

3. Conclusion

In this paper, we have discussed whether CAG ID should be added into Initial UE Message, and have the following proposals,

- The CAG Identifier should be reported to AMF for the UE verification.

- RAN3 understands that the CAG ID is not added into Initial UE Message.

New cause values which are equal to “invalid CAG ID” and “only CAG cell is allowed” should be introduced on NG interface.

New cause values which are equal to “invalid NID” should be introduced on NG interface.

The corresponding CR is provided in [5], and the LS to SA3 is in [6].

4. Reference

- R3-194686, Way Forward with private networks, Nokia, Nokia Shanghai Bell

- S3-193142, LS on Sending CAG ID in NAS layer, Qualcomm

- 3GPP TS 23.501 V16.1.0 (2019-06)

- 3GPP TS 33.501 V15.5.0 (2019-06)

- R3-195764	Initial UE Message for NPN, Huawei

- R3-195765	[Draft] Reply LS on sending CAG ID in NAS layer, Huawei