3GPP TSG-RAN3 Meeting #105bis	R3-195774

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0171 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remaining issues on support of NPN over E1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was agreed to support non-public network (NPN) for RAN in [RP-191563]. In addition to the basic agreements in [R3-194686], the remaining open issues should be included over E1 interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add defination on PNI-NPN, - Add abbreviation on CAG, PNI-NPN, - Add PNI-NPN support information per PLMN of the gNB-CU-UP in GNB-CU-UP E1 SETUP REQUEST, GNB-CU-CP E1 SETUP RESPONSE, GNB-CU-UP CONFIGURATION UPDATE message, - Add NID IE in the BEARER CONTEXT SETUP REQUEST message, and add the related text descriptions, - Add text descriptions on NPN for gNB-CU-UP E1 Setup, gNB-CU-CP E1 Setup, gNB-CU-UP Configuration Update procedures, - Add Information Element Definitions on NID, CAG ID, - Add new Cause Values (i.e., CAG ID(s) not supported, NID(s) not supported) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NPN feature is not supported over E1 interface. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8.2.3, 8.2.4, 8.2.5, 9.2.1.4, 9.2.1.8, 9.2.1.10, 9.2.2.1, 9.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Elementary Procedure: E1AP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between gNB-CU-CP and gNB-CU-UP. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. The usage of several E1AP EPs together is specified in stage 2 specifications (e.g., TS 38.460 [3]).

An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success and/or failure).

-	Class 2: Elementary Procedures without response.

For Class 1 EPs, the types of responses can be as follows:

Successful:

-	A signalling message explicitly indicates that the elementary procedure successfully completed with the receipt of the response.

Unsuccessful:

-	A signalling message explicitly indicates that the EP failed.

-	On time supervision expiry (i.e., absence of expected response).

Successful and Unsuccessful:

-	One signalling message reports both successful and unsuccessful outcome for the different included requests. The response message used is the one defined for successful outcome.

Class 2 EPs are considered always successful.

gNB: as defined in TS 38.300 [4].

gNB-CU: as defined in TS 38.401 [2].

gNB-DU: as defined in TS 38.401 [2].

gNB-CU-CP: as defined in TS 38.401 [2].

gNB-CU-UP: as defined in TS 38.401 [2].

PDU Session Resource: as defined in TS 38.401 [2].

UE-associated signalling: When E1AP messages associated to one UE uses the UE-associated logical E1-connection for association of the message to the UE in gNB-CU-UP and gNB-CU-CP.

UE-associated logical E1-connection: The UE-associated logical E1-connection uses the identities GNB-CU-CP UE E1AP ID and GNB-CU-UP UE E1AP ID according to the definition in TS 38.401 [2]. For a received UE associated E1AP message the gNB-CU-CP identifies the associated UE based on the GNB-CU-CP UE E1AP ID IE and the gNB-CU-UP identifies the associated UE based on the GNB-CU-UP UE E1AP ID IE.

Public network integrated NPN: as defined in TS 23.501 [20].

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

CGI	Cell Global Identifier

CN	Core Network

CP	Control Plane

DL	Downlink

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

MCG	Master Cell Group

NSSAI	Network Slice Selection Assistance Information

RANAC	RAN Area Code

SCG	Secondary Cell Group

SDAP	Service Data Adaptation Protocol

S-NSSAI	Single Network Slice Selection Assistance Information

TNLA	Transport Network Layer Association

CAG                    Closed Access Group

PNI-NPN	Public network integrated NPN

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.2.3	gNB-CU-UP E1 Setup

8.2.3.1	General

The purpose of the gNB-CU-UP E1 Setup procedure is to exchange application level data needed for the gNB-CU-UP and the gNB-CU-CP to correctly interoperate on the E1 interface. If the gNB-CU-UP initiates the first TNL association, it shall also initiate the gNB-CU-UP E1 Setup procedure. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the E1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

8.2.3.2	Successful Operation

Figure 8.2.3.2-1: gNB-CU-UP E1 Setup procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending a GNB-CU-UP E1 SETUP REQUEST message including the appropriate data to the gNB-CU-CP. The gNB-CU-CP responds with a GNB-CU-UP E1 SETUP RESPONSE message including the appropriate data.

If the GNB-CU-UP E1 SETUP REQUEST message contains the gNB-CU-UP Name IE the gNB-CU-CP may use this IE as a human readable name of the gNB-CU-UP.

If the Slice Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

If the NR CGI Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

If the QoS Parameters Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the E1 interface is operational and other E1 messages can be exchanged.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall take this IE into account.

If the CAG Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall take this IE into account.

<Unchanged Text Omitted>

8.2.4	gNB-CU-CP E1 Setup

8.2.4.1	General

The purpose of the gNB-CU-CP E1 Setup procedure is to exchange application level data needed for the gNB-CU-CP and the gNB-CU-UP to correctly interoperate on the E1 interface. If the gNB-CU-CP initiates the first TNL association, it shall also initiate the gNB-CU-CP E1 Setup procedure.The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the E1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-CU-CP E1 Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending a GNB-CU-CP E1 SETUP REQUEST message including the appropriate data to the gNB-CU-UP. The gNB-CU-UP responds with a GNB-CU-CP E1 SETUP RESPONSE message including the appropriate data.

If the GNB-CU-CP E1 SETUP REQUEST message contains the gNB-CU-CP Name IE the gNB-CU-UP may use this IE as a human readable name of the gNB-CU-CP.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the E1 interface is operational and other E1 messages can be exchanged.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall take this IE into account.

If the CAG Support List IE is contained in the GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall take this IE into account.

<Unchanged Text Omitted>

8.2.5	gNB-CU-UP Configuration Update

8.2.5.1	General

The purpose of the gNB-CU-UP Configuration Update procedure is to update application level configuration data needed for the gNB-CU-UP and the gNB-CU-CP to interoperate correctly on the E1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU-UP Configuration Update procedure: Successful Operation.

<Unchanged Text Omitted>

If the GNB-CU-UP CONFIGURATION UPDATE message includes gNB-CU-UP TNLA To Remove List IE, and the Endpoint IP address IE and the Port Number IE for both TNL endpoints of the TNL association(s) are included in the gNB-CU-UP TNLA To Remove List IE, the gNB-CU-CP shall, if supported, consider that the TNL association(s) indicated by both received TNL endpoints will be removed by the gNB-CU-UP. If the Endpoint IP address IE, or the Endpoint IP address IE and the Port Number IE for one or both of the TNL endpoints is included in the gNB-CU-UP TNLA To Remove List IE in GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall, if supported, consider that the TNL association(s) indicated by the received endpoint IP address(es) will be removed by the gNB-CU-UP.

If the CAG Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall shall store the corresponding information and replace any existing information.

8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: gNB-CU-UP Configuration Update procedure: Unsuccessful Operation.

If the gNB-CU-CP cannot accept the update, it shall respond with a GNB-CU-UP CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU-UP CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU-CP shall wait at least for the indicated time before reinitiating the GNB-CU-UP CONFIGURATION UPDATE message towards the same gNB-CU-CP.

8.2.5.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.3	Bearer Context Management procedures

8.3.1	Bearer Context Setup

8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

<Unchanged Text Omitted>

If the DRB QoS IE is contained within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, take it into account as specified in TS 28.552 [22].

If the gNB-DU-ID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store the information received.

If the RAN UE ID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store the information received.

For each successfully established DRB, the gNB-CU-UP shall provide, in the respective UL UP Parameters IE of the BEARER CONTEXT SETUP RESPONSE, one UL UP Transport Layer Information Item per cell group entry contained in the respective Cell Group Information IE of the BEARER CONTEXT SETUP REQUEST message.

If the Serving NID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall combine the Serving NID IE with the Serving PLMN IE to identify the serving NPN, and take it into account.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2	Message Functional Definition and Content

9.2.1	Interface Management messages

<Unchanged Text Omitted>

9.2.1.4	GNB-CU-UP E1 SETUP REQUEST

This message is sent by the gNB-CU-UP to transfer information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| CN Support | M |  | ENUMERATED (EPC. 5GC, both, …) |  | YES | reject |
| Supported PLMNs |  | 1..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| >CAG Support List |  | 0..1 |  | Supported PNI-NPNs per PLMN. | - | - |
| >>CAG Support Item |  | 1..<maxnoofCAGs> |  |  | - | - |
| >>>CAG Id | M |  | 9.3.1.x |  | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofCAGs | Maximum no. of CAG Ids broadcasted by an NG-RAN. Value is 12. |

<Unchanged Text Omitted>

9.2.1.8	GNB-CU-CP E1 SETUP RESPONSE

This message is sent by the gNB-CU-UP to transfer information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| CN Support | M |  | ENUMERATED (EPC. 5GC, both, …) |  | YES | reject |
| Supported PLMNs |  | 1..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| >CAG Support List |  | 0..1 |  | Supported PNI-NPNs per PLMN. | - | - |
| >>CAG Support Item |  | 1..<maxnoofCAGs> |  |  | - | - |
| >>>CAG Id | M |  | 9.3.1.x |  | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofCAGs | Maximum no. of CAG Ids broadcasted by an NG-RAN. Value is 12. |

<Unchanged Text Omitted>

9.2.1.10	GNB-CU-UP CONFIGURATION UPDATE

This message is sent by the gNB-CU-UP to transfer updated information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | O |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| Supported PLMNs |  | 0..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| >CAG Support List |  | 0..1 |  | Supported PNI-NPNs per PLMN. | - | - |
| >>CAG Support Item |  | 1..<maxnoofCAGs> |  |  | - | - |
| >>>CAG Id | M |  | 9.3.1.x |  | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |
| gNB-CU-UP TNLA To Remove List |  | 0..1 |  |  | YES | reject |
| >gNB-CU-UP TNLA To Remove Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-UP. | - | - |
| >>TNLA Transport Layer Address gNB-CU-CP | O |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-UP and the gNB-CU-CP. Value is 32. |
| maxnoofCAGs | Maximum no. of CAG Ids broadcasted by an NG-RAN. Value is 12. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.2	Bearer Context Management messages

9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| Security Information | M |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.20 |  | YES | reject |
| UE DL Maximum Integrity Protected Data Rate | O |  | Bit Rate 9.3.1.20 | The Bit Rate is a portion of the UE’s Maximum Integrity Protected Data Rate, and is enforced by the gNB-CU-UP node. | YES | reject |
| Serving PLMN | M |  | PLMN Identity 9.3.1.7 |  | YES | ignore |
| Activity Notification Level | M |  | 9.3.1.67 |  | YES | reject |
| UE Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to UE. | - | - |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …) | Indicates the status of the Bearer Context | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List | M |  | DRB To Setup List E-UTRAN 9.3.3.1 |  | YES | reject |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List | M |  | 9.3.3.2 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE(8)) |  | YES | ignore |
| gNB-DU ID | O |  | 9.3.1.65 | Included whenever it is known by the gNB-CU-CP | YES | Ignore |
| Serving NID | O |  | 9.3.1.y |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3	Information Element Definitions

9.3.1	Radio Network Layer Related IEs

<Unchanged Text Omitted>

9.3.1.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the E1AP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, Unknown or already allocated gNB-CU-CP UE E1AP ID, Unknown or already allocated gNB-CU-UP UE E1AP ID, Unknown or inconsistent pair of UE E1AP ID, Interaction with other procedure, PDCP Count Wrap Around, Not supported QCI value, Not supported 5QI value, Encryption algorithms not supported, Integrity protection algorithms not supported, UP integrity protection not possible, UP confidentiality protection not possible, Multiple PDU Session ID Instances, Unknown PDU Session ID, Multiple QoS Flow ID Instances, Unknown QoS Flow ID, Multiple DRB ID Instances, Unknown DRB ID, Invalid QoS combination, Procedure cancelled, Normal release, No radio resources available, Action desirable for radio reasons, Resources not available for the slice, PDCP configuration not supported, …, UE DL maximum integrity protected data rate reason, UP integrity protection failure, Release due to Pre-Emption, PNI-NPN(s) not supported, SNPN(s) not supported) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified, Transport Resource Unavailable, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Not enough User Plane Processing Resources, Hardware Failure, O&M Intervention, Unspecified, …) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| Unknown or already allocated gNB-CU-CP UE E1AP ID | The action failed because the gNB-CU-CP UE E1AP ID is either unknown, or (for a first message received at the gNB-CU) is known and already allocated to an existing context. |
| Unknown or already allocated gNB-CU-UP UE E1AP ID | The action failed because the gNB-CU-UP UE E1AP ID is either unknown, or (for a first message received at the gNB-CU-UP) is known and already allocated to an existing context. |
| Unknown or inconsistent pair of UE E1AP ID | The action failed because both UE E1AP IDs are unknown, or are known but do not define a single UE context. |
| Interaction with other procedure | The action is due to an ongoing interaction with another procedure. |
| PDCP COUNT wrap around | PDCP COUNT approaches the maximum value. |
| Not supported QCI value | The action failed because the requested QCI is not supported. |
| Not supported 5QI value | The action failed because the requested 5QI is not supported. |
| Encryption algorithms not supported | The gNB-CU-UP is unable to support the selected encryption algorithm for the UE. |
| Integrity protection algorithms not supported | The gNB-CU-UP is unable to support the selected integrity protection algorithm for the UE. |
| UP integrity protection not possible | The PDU Session cannot be accepted according to the required user plane integrity protection policy. |
| UP confidentiality protection not possible | The PDU Session cannot be accepted according to the required user plane confidentiality protection policy |
| Multiple PDU Session ID Instances | The action failed because multiple instances of the same PDU Session had been provided. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown. |
| Multiple QoS Flow ID Instances | The action failed because multiple instances of the same QoS flow had been provided. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknow. |
| Multiple DRB ID Instances | The action failed because multiple instances of the same DRB had been provided. |
| Unknown DRB ID | The action failed because the DRB ID is unknow. |
| Invalid QoS combination | The action was failed because of invalid QoS combination |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. |
| Normal release | The action is due to a normal release of the UE (e.g. because of mobility) and does not indicate an error. |
| No radio resources available | The requested node doesn’t have sufficient radio resources available. |
| Action desirable for radio reasons | The reason for requesting the action is radio related. |
| Resources not available for the slice | The requested resources are not available for the slice. |
| PDCP configuration not supported, | The gNB-CU-UP is unable to support the selected PDCP configuration for the UE. |
| UE DL maximum integrity protected data rate reason | The request is not accepted in order to comply with the maximum downlink data rate for integrity protection supported by the UE. |
| UP integrity protection failure | The gNB-CU-UP detects an integrity protection failure in the UL PDU. |
| Release due to Pre-Emption | Release is initiated due to pre-emption. |
| PNI-NPN(s) not supported | The request is not accepted due to the PNI-NPN(s) not supported. |
| SNPN(s) not supported | The request is not accepted due to the SNPN(s) not supported. |

<Unchanged Text Omitted>

9.3.1.x	CAG ID

This IE uniquely identifies a CAG per PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CAG ID | M |  | FFS | Corresponds to the FFS defined in TS 38.331 [8]. |

9.3.1.y	NID

This IE uniquely identifies a SNPN per PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NID | M |  | FFS | Corresponds to the FFS defined in TS 38.331 [8]. |

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>