3GPP TSG-RAN WG3 Meeting #105-bis 	R3-195846

# Chongqing, P. R. China, 14. - 18.10.2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0087 | rev | 1 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enabling hold-and-go solution and reporting of the transmission attempt failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIoT-Core |  |  |  |  |  | Date: |  |  | 1.10.2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It order to enable more efficient duplication for URLLC, a mechanism enabling holding duplicated PDU at the assisting node is introduced. Then, a command to send it and a feedback about failure at the first transmission attempt are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A flag is added to indicate the PDU shall not be transmitted right away is added; an IE to indicate which PDUs shall now be transmitted is added together with the remaining time budget for them; an IE to inform the hosting node that the first attempt of transmission failed is added. Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change only affect one feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The enhancements to the data duplication will not be enabled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.1, 5.4.1.2, 5.4.2.1, 5.4.2.2, 5.5.2.1, 5.5.2.2, 5.5.3.A1 (new), 5.5.3.A2 (new), 5.5.3.A3 (new), 5.5.3.A4 (new), 5.5.3.A5 (new), 5.5.3.A6 (new), 5.5.3.A7 (new), 5.5.3.A8 (new), 5.5.3.A9 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1: Resubmission to RAN3 #105-bis |  |  |  |  |  |  |  |  |

*** First change, skipped text not changed ***

### 5.4.1	Transfer of Downlink User Data

#### 5.4.1.1	Successful operation

The purpose of the Transfer of Downlink User Data procedure is to provide NR-U specific sequence number information at the transfer of user data carrying a DL NR PDCP PDU from the node hosting the NR PDCP entity to the corresponding node.

An NR user plane protocol instance making use of the Transfer of Downlink User Data procedure is associated to a single data radio bearer only.

The node hosting the NR PDCP entity shall assign consecutive NR-U sequence numbers to each transferred NR-U packet. A retransmitted NR PDCP PDU shall be assigned a new NR-U sequence number.

The node hosting the NR PDCP entity indicates to the corresponding node whether this NR-U packet is a retransmission of NR PDCP PDU.

The node hosting the NR PDCP entity can indicate to the corresponding node to either discard all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs.

If the Assistance Information Report Polling Flag is equal to 1, the corresponding node shall, if supported, send the ASSISTANCE INFORMATION DATA to the node hosting the NR PDCP entity.

The corresponding node shall detect whether an NR-U packet was lost and memorise the respective sequence number after it has declared the respective NR-U packet as being "lost".

The corresponding node shall transfer the remaining NR PDCP PDUs towards the UE and memorise the highest NR PDCP PDU sequence number of the NR PDCP PDU that was successfully delivered (as defined in TS 36.322 [6] and TS 38.322 [7]) in sequence towards the UE (in case RLC AM is used) and the highest NR PDCP PDU sequence number of the NR PDCP PDU that was transmitted to the lower layers.

The corresponding node shall send the DL DATA DELIVERY STATUS if the Report Polling Flag is set to 1 or when the NR PDCP PDU with the indicated DL report NR PDCP PDU SN has been successfully delivered, unless a situation of overload at the corresponding node is encountered. The DL DATA DELIVERY STATUS sent as a response to a specific DL report NR PDCP PDU SN shall be sent only when all PDCP PDU SNs up to this DL report NR PDCP PDU have been successfully delivered in-sequence.

NOTE:	The Transfer of Downlink User Data procedure and the associated feedback of lost NR-U packets assist the node hosting the NR PDCP entity in avoiding NR PDCP HFN de-synchronisation. If a deployment decides to not use the Transfer of Downlink User Data procedure, NR PDCP HFN synchronization should be ensured by other means.

If the User data existence flag is set to 1, the corresponding node assumes that the node hosting the NR PDCP entity has some user data for the concerned data radio bearer. The corresponding node decides whether and when to use DRX for the UE (i.e. the corresponding node may indicate the UE to use DRX even if the flag is set to 1 and the received DL USER DATA frame contains no user data).

The node hosting PDCP may indicate to the corresponding node that the PDU shall not yet be sent to lower layers for transmission towards the UE by setting the hold flag to 1. If the PDU is to be hold, its transmission shall be postponed until its transmission is requested or it is to be discarded. If the transmit commands flag is set to 1, the Downlink User Data contains a list of hold PDUs to be sent to lower layers for transmission.

The node hosting PDCP may indicate a specific time budget for the PDU by setting the Time budget present flag to 1 and including the Time budget field.

Figure 5.4.1.1-1: Successful Transfer of Downlink User Data

#### 5.4.1.2	Unsuccessful operation

Void.

### 5.4.2	Downlink Data Delivery Status

#### 5.4.2.1	Successful operation

The purpose of the Downlink Data Delivery Status procedure is to provide feedback from the corresponding node to the node hosting the NR PDCP entity to allow the node hosting the NR PDCP entity to control the downlink user data flow via the corresponding node for the respective data radio bearer. The corresponding node may also transfer uplink user data for the concerned data radio bearer to the node hosting the NR PDCP entity together with a DL DATA DELIVERY STATUS frame within the same GTP-U PDU.

The Downlink Data Delivery Status procedure is also used to provide feedback from the corresponding node to the node hosting the NR PDCP entity to allow the node hosting the NR PDCP entity to control the successful delivery of DL control data to the corresponding node.

When the corresponding node decides to trigger the feedback for Downlink Data Delivery procedure it shall report as specified in section 5.2:

a)	in case of RLC AM, the highest NR PDCP PDU sequence number successfully delivered in sequence to the UE among those NR PDCP PDUs received from the node hosting the NR PDCP entity i.e. excludes those retransmission NR PDCP PDUs;

b)	the desired buffer size in bytes for the concerned data radio bearer;

c)	optionally, the desired data rate in bytes associated with a specific data radio bearer configured for the UE;

d)	the NR-U packets that were declared as being "lost" by the corresponding node and have not yet been reported to the node hosting the NR PDCP entity within the DL DATA DELIVERY STATUS frame;

e)	if retransmission NR PDCP PDUs have been delivered, the NR PDCP PDU sequence number associated with the highest NR-U sequence number among the retransmission NR PDCP PDUs successfully delivered to the UE in sequence of NR-U sequence number;

f)	if retransmission NR PDCP PDUs have been transmitted to the lower layers, the NR PDCP PDU sequence number associated with the highest NR-U sequence number among the retransmission NR PDCP PDUs transmitted to the lower layers in sequence of NR-U sequence number;

g)	the highest NR PDCP PDU sequence number transmitted to the lower layers among those NR PDCP PDUs received from the node hosting the NR PDCP entity i.e. excludes those retransmission NR PDCP PDUs.

NOTE:	If a deployment has decided not to use the Transfer of Downlink User Data procedure, d), e) and f)  above are not applicable.

As soon as the corresponding node detects the successful RACH access by the UE for the corresponding data radio bearer(s), the corresponding node shall send initial DL DATA DELIVERY STATUS frame to the node(s) hosting the NR PDCP entity(ies). The node hosting NR PDCP entity may start sending DL data before receiving the initial DL DATA DELIVERY STATUS frame. In case the DL DATA DELIVERY STATUS frame is sent before any NR PDCP PDU is transferred to lower layers, the information on the highest NR PDCP PDU sequence number successfully delivered in sequence to the UE and the highest NR PDCP PDU sequence number transmitted to the lower layers may not be provided.

The DL DATA DELIVERY STATUS frame shall also include a final frame indication when this frame is the last DL status report. When receiving such indication, the node hosting the NR PDCP entity considers that no more UL or DL data is expected to be transmitted between the corresponding node and the UE.

The DL DATA DELIVERY STATUS frame may also include an indication of detected radio link outage or radio link resume for the concerned data radio bearer. When receiving an indication of radio link outage detection, the node hosting the NR PDCP entity considers that traffic delivery over the data radio bearer configured for the UE is unavailable at the corresponding node both in UL and DL. When receiving an indication of radio link resume detection, the node hosting the NR PDCP entity considers that traffic delivery over the data radio bearer configured for the UE is available at the corresponding node both in UL and in DL. When receiving an indication of UL or DL radio link outage detection, the node hosting the NR PDCP entity considers that traffic delivery over the data radio bearer configured for the UE is unavailable at the corresponding node for UL or DL, depending on the indicated outage. When receiving an indication of UL or DL radio link resume detection, the node hosting the NR PDCP entity considers that traffic delivery over the data radio bearer configured for the UE is available at the corresponding node in UL or in DL, depending on the indicated resume.

The node hosting the NR PDCP entity, when receiving the DL DATA DELIVERY STATUS frame:

-	regards the desired buffer size under b) and the data rate under c) above as the amount of data to be sent from the hosting node:

-	If the value of the desired buffer size is 0, the hosting node shall stop sending any data per bearer.

-	If the value of the desired buffer size in b) above is greater than 0, the hosting node may send up to this amount of data per bearer starting from the last "Highest successfully delivered NR PDCP Sequence Number" for RLC AM, or the hosting node may send up to this amount of data per bearer starting from the last "Highest transmitted NR PDCP Sequence Number" for RLC UM.

-	The value of the desired data rate in c) above is the amount of data desired to be received in a specific amount of time. The amount of time is 1 sec.

-	The information of the buffer size in b) above and of the data rate in c) above is valid until the next DL DATA DELIVERY STATUS frame is received.

-	is allowed to remove the buffered NR PDCP PDUs of a RLC AM bearer, according to the feedback of successfully delivered NR PDCP PDUs;

-	decides upon the actions necessary to take for NR PDCP PDUs reported other than transmitted and/or successfully delivered.

In case of RLC AM, after the highest NR PDCP PDU sequence number successfully delivered in sequence is reported to the node hosting the NR PDCP entity, the corresponding node removes the respective NR PDCP PDUs. For RLC UM, the corresponding node may remove the respective NR PDCP PDUs after transmitting to lower layers.

The corresponding node may indicate to the node hosting PDCP that first attempt of transmission of certain NR PDCP PDUs failed by setting the Transmission failure report flag to 1 and including a list of the failed sequence numbers.

Figure 5.4.2.1-1: Successful Downlink Data Delivery Status

#### 5.4.2.2	Unsuccessful operation

Void.

*** Next change, skipped text not changed ***

#### 5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined to allow the corresponding node to detect lost NR-U packets and is associated with the transfer of a Downlink NR PDCP PDU.

The following shows the respective DL USER DATA frame.

| Bits |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 |  | 4 |  | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  |  |  | Hold | DL Discard Blocks | DL Flush | Report polling | 1 |
| Spare |  | Transmit commands |  |  | Time budget present | Report Delivered | User data existence flag | Assistance Info. Report Polling Flag | Retransmission flag | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL report NR PDCP PDU SN |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Time budget |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Number of transmit commands |  |  |  |  |  |  |  |  |  | 0 or 1 |
| NR PDCP PDU SN to be sent (first) |  |  |  |  |  |  |  |  |  | 0 or 3 |
| ... |  |  |  |  |  |  |  |  |  |  |
| NR PDCP PDU SN to be sent (last) |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

#### 5.5.2.2	DL DATA DELIVERY STATUS (PDU Type 1)

This frame format is defined to transfer feedback to allow the receiving node (i.e. the node that hosts the NR PDCP entity) to control the downlink user data flow via the sending node (i.e. the corresponding node).

The following shows the respective DL DATA DELIVERY STATUS frame. The Figure shows an example of how a frame is structured when all optional IEs (i.e. those whose presence is indicated by an associated flag) are present.

Absence of such an IE changes the position of all subsequent IEs on octet level.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 |  | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  |  | Highest Transmitted NR PDCP SN Ind | Highest Delivered NR PDCP SN Ind | Final Frame Ind. | Lost Packet Report | 1 |
| Spare |  | Transmission failure report |  |  | Data rate Ind. | Retransmitted NR PDCP SN Ind | Delivered Retransmitted NR PDCP SN Ind | Cause Report | 1 |
| Desired buffer size for the data radio bearer |  |  |  |  |  |  |  |  | 4 |
| Desired Data Rate |  |  |  |  |  |  |  |  | 0 or 4 |
| Number of lost NR-U Sequence Number ranges reported |  |  |  |  |  |  |  |  | 0 or 1 |
| Start of lost NR-U Sequence Number range |  |  |  |  |  |  |  |  | 0 or (6* Number of reported lost NR-U SN ranges) |
| End of lost NR-U Sequence Number range |  |  |  |  |  |  |  |  |  |
| Highest successfully delivered NR PDCP Sequence Number |  |  |  |  |  |  |  |  | 0 or 3 |
| Highest transmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  | 0 or 3 |
| Cause Value |  |  |  |  |  |  |  |  | 0 or 1 |
| Successfully delivered retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  | 0 or 3 |
| Retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  | 0 or 3 |
| Number of failed NR PDCP PDUs |  |  |  |  |  |  |  |  | 0 or 1 |
| Failed first NR PDCP PDU SN transmission (first) |  |  |  |  |  |  |  |  | 0 or 3 |
| ... |  |  |  |  |  |  |  |  |  |
| Failed first NR PDCP PDU SN transmission (last) |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: DL DATA DELIVERY STATUS (PDU Type 1) Format

*** Next change, skipped text not changed ***

#### 5.5.3.A1	Hold

Description: This parameter indicates the included NR PDCP PDU shall be hold before sending to lower layers for transmission.

Value range: {0= PDU shall be sent as soon as possible, 1= PDU shall be hold}.

Field length: 1 bit.

#### 5.5.3.A2	Time budget present

Description: This parameter indicates the presence of the Time budget information for the hold PDU (to be ignored if the transmit command is not present).

Value range: {0= Time budget not present, 1= Time budget present}.

Field length: 1 bit.

#### 5.5.3.A3	Time budget

Description: This parameter indicates the maximum time the hold PDU is valid (to be ignored if the transmit command is not present).

Value range: {0..28-1} × 0,1 ms.

Field length: 1 octet.

#### 5.5.3.A4	Transmit commands

Description: This parameter indicates the presence of the list of Transmit commands.

Value range: {0= Transmit commands not present, 1= Transmit commands present}.

Field length: 1 bit.

#### 5.5.3.A5	Number of transmit commands

Description: This parameter indicates the number of the NR PDCP PDUs to be transmitted.

Value range: {1..244}.

Field length: 1 octet.

#### 5.5.3.A6	NR PDCP PDU SN to be sent

Description: This parameter indicates the SN of a downlink NR PDCP PDU to be sent to lower layers for transmission (if requested to be hold before).

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.A7	Transmission failure report

Description: This parameter indicates the presence of the list of the failed NR PDCP PDUs.

Value range: {0= Failed NR PDCP PDUs not present, 1= Failed NR PDCP PDUs present}.

Field length: 1 bit.

#### 5.5.3.A8	Number of failed NR PDCP PDUs

Description: This parameter indicates the number of the failed NR PDCP PDUs.

Value range: {1..244}.

Field length: 1 octet.

#### 5.5.3.A9	Failed NR PDCP PDU SN transmission

Description: This parameter indicates the SN of a downlink NR PDCP PDU that failed at the first transmission attempt.

Value range: {0..218-1}.

Field length: 3 octets.

*** Remaining text not changed ***