# 3GPP TSG-RAN WG3 #105bis	Tdoc R3-195867

Chongqing, China, 14th – 18th October

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0478 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for setting up IPSec on the fly in F1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | IPSec tunnel establishment is not supported currently in the F1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new IP-Sec Transport Layer Address to UE CONTEXT SETUP REQUEST and UE CONTEXT SETUP RESPONSE messages. Impact assessment towards the previous version of the specification (same release): This CR impacts all architectures This CR is backward compatible |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case of split architecture, the gNB-CU and the gNB-DU cannot exhange the IPSec address to be used for IPSec establishment |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1, 9.2.2.1, 9.2.2.2, 9.3.2.x (new), 9.4.4, 9.4.5 and 9.4.7. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, and DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure.

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the SpCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly.

If the SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the SCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the DRX Cycle IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall use the provided value from the gNB-CU.

If the UL Configuration IE in DRB to Be Setup Item IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the SRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB.

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB.

If two UL UP TNL Information IEs are included in UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT SETUP RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activing/deactiving CA based PDCP duplication for the DRB.

If DC Based Duplication Configured IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. If DC Based Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activing/deactiving DC based PDCP duplication for this DRB.

If UL PDCP SN length IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT SETUP REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is available at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT SETUP REQUEST. The gNB-DU may use it for RRM purposes.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT SETUP RESPONSE message, the result for all the requested DRBs and SRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to Setup List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

For EN-DC operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15].

For NG-RAN operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the DRB Information IE.

For DC operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE at the gNB acting as secondary node. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the HandoverPreparationInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU of the gNB acting as master node shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. The gNB-CU of the gNB acting as master node shall only initiate the UE Context Setup procedure for handover or secondary node addition when at least one DRB is setup for the UE. If the received CU to DU RRC Information IE does not include source cell group configuration, the gNB-DU shall generate the cell group configuration using full configuration. Otherwise, delta configuration is allowed.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message.

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT SETUP REQUEST message shall be ignored. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the Masked IMEISV IE is contained in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be established with an appropriate cause value for each SCell failed to setup.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT SETUP REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT SETUP RESPONSE message and set to "Not-supported", the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

If the CellGroupConfig IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

If the Full Configuration IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

If the C-RNTI IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

The UE Context Setup Procedure is not used to configure SRB0.

If the UE CONTEXT SETUP REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE via SRB1.

If the Notification Control IE is included in the DRB to Be Setup List IE and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store the received UL PDU Session Aggregate Maximum Bit Rate and use it when enforcing uplink traffic policing for non-GBR Bearers for the concerned UE as specified in TS 23.501 [21].

The gNB-DU shall store the received gNB-DU UE Aggregate Maximum Bit Rate Uplink and use it for non-GBR Bearers for the concerned UE.

If the UE CONTEXT SETUP REQUEST message contains the QoS Flow Mapping Indication IE, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the UE CONTEXT SETUP REQUEST message contains the New gNB-CU UE F1AP ID IE, the gNB-DU shall, if supported, replace the value received in the gNB-CU UE F1AP ID IE by the value of the New gNB-CU UE F1AP ID and use it for further signalling.

If the RAN UE ID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store and replace any previous information received.

If the TNL Configuration Info IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall take this IE into account for IPSec establishment.

If the TNL Configuration Info IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall take this IE into account for IPSec establishment.

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: UE Context Setup Request procedure: unsuccessful Operation

If the gNB-DU is not able to establish an F1 UE context, or cannot even establish one bearer it shall consider the procedure as failed and reply with the UE CONTEXT SETUP FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT SETUP REQUEST message, it shall reply with the UE CONTEXT SETUP FAILURE message with an appropriate cause value. Further, if the Candidate SpCell List IE is included in the UE CONTEXT SETUP REQUEST message and the gNB-DU is not able to accept the SpCell ID IE, the gNB-DU shall, if supported, include the Potential SpCell List IE in the UE CONTEXT SETUP FAILURE message and the gNB-CU should take this into account for selection of an opportune SpCell. The gNB-DU shall include the cells in the Potential SpCell List IE in a priority order, where the first cell in the list is the one most desired and the last one is the one least desired (e.g., based on load conditions). If the Potential SpCell List IE is present but no Potential SpCell Item IE is present, the gNB-CU should assume that none of the cells in the Candidate SpCell List IE are acceptable for the gNB-DU.

#### 8.3.1.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value. If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT SETUP REQUEST message and is set to the value “delay critical” but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< 2nd Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. | - |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>> UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >> RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >> UL Configuration | O |  | UL Configuraiton 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | - |  |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC basedUL PDCP duplication | YES | reject |
| >>DL PDCP SN length | M |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.55 |  | YES | ignore |
| Serving PLMN | O |  | PLMN ID 9.3.1.14 | Indicates the PLMN serving the UE. | YES | ignore |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | C-ifDRBSetup |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |
| TNL Configuration Info | O |  | 9.3.2.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |

<<<<<<<<<<<<<<<<<<<< 3rd Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DU To CU RRC Information | M |  | 9.3.1.26 |  | YES | reject |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item Iist |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>> DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| SRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Failed to Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >DRB Failed to Setup Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SCell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell Failed to Setup Item |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Inactivity Monitoring Response | O |  | ENUMERATED (not-supported, ...) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| SRB Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| TNL Configuration Info | O |  | 9.3.2.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |

<<<<<<<<<<<<<<<<<<<< 4th Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.2.x	TNL Configuration Info

This IE is used for signalling IP addresses of IPSEc endpoints used for establishment of IPSec tunnels.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended Transport Layer Addresses |  | 0..<maxnoofExtTLAs> |  |  |
| >IP-Sec Transport Layer Address | O |  | Transport Layer Address 9.3.2.3 | Transport Layer Addresses for IP-Sec endpoint. |

| Range bound | Explanation |
| --- | --- |
| maxnoofExtTLAs | Maximum no. of Extended Transport Layer Addresses in the message. Value is 16. |

<<<<<<<<<<<<<<<<<<<< 5th Change >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Candidate-SpCell-Item,

Cause,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CellULConfigured,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBID,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DRXConfigurationIndicator,

DUtoCURRCInformation,

EUTRANQoS,

ExecuteDuplication,

FullConfiguration,

GNB-CU-UE-F1AP-ID,

GNB-DU-UE-F1AP-ID,

GNB-DU-ID,

GNB-DU-Served-Cells-Item,

GNB-DU-System-Information,

GNB-CU-Name,

GNB-DU-Name,

InactivityMonitoringRequest,

InactivityMonitoringResponse,

NotificationControl,

NRCGI,

NRPCI,

UEContextNotRetrievable,

Potential-SpCell-Item,

RAT-FrequencyPriorityInformation,

ResourceCoordinationTransferContainer,

RRCContainer,

RRCReconfigurationCompleteIndicator,

SCellIndex,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

SCell-FailedtoSetup-Item,

SCell-FailedtoSetupMod-Item,

ServCellIndex,

Served-Cell-Information,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

ServingCellMO,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

SRBs-Modified-Item,

SRBs-Setup-Item,

SRBs-SetupMod-Item,

TimeToWait,

TransactionID,

TransmissionActionIndicator,

UE-associatedLogicalF1-ConnectionItem,

DUtoCURRCContainer,

PagingCell-Item,

SItype-List,

UEIdentityIndexValue,

GNB-CU-TNL-Association-Setup-Item,

GNB-CU-TNL-Association-Failed-To-Setup-Item,

GNB-CU-TNL-Association-To-Add-Item,

GNB-CU-TNL-Association-To-Remove-Item,

GNB-CU-TNL-Association-To-Update-Item,

MaskedIMEISV,

PagingDRX,

PagingPriority,

PagingIdentity,

Cells-to-be-Barred-Item,

PWSSystemInformation,

Broadcast-To-Be-Cancelled-Item,

Cells-Broadcast-Cancelled-Item,

NR-CGI-List-For-Restart-Item,

PWS-Failed-NR-CGI-Item,

RepetitionPeriod,

NumberofBroadcastRequest,

Cells-To-Be-Broadcast-Item,

Cells-Broadcast-Completed-Item,

Cancel-all-Warning-Messages-Indicator,

EUTRA-NR-CellResourceCoordinationReq-Container,

EUTRA-NR-CellResourceCoordinationReqAck-Container,

RequestType,

PLMN-Identity,

RLCFailureIndication,

UplinkTxDirectCurrentListInformation,

SULAccessIndication,

Protected-EUTRA-Resources-Item,

GNB-DUConfigurationQuery,

BitRate,

RRC-Version,

GNBDUOverloadInformation,

RRCDeliveryStatusRequest,

NeedforGap,

RRCDeliveryStatus,

ResourceCoordinationTransferInformation,

Dedicated-SIDelivery-NeededUE-Item,

Associated-SCell-Item,

IgnoreResourceCoordinationContainer,

PagingOrigin,

UAC-Assistance-Info,

RANUEID,

GNB-DU-TNL-Association-To-Remove-Item,

TNLConfigurationInfo

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-Candidate-SpCell-Item,

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-Status-Item,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-ConfirmedUEID,

id-CriticalityDiagnostics,

id-C-RNTI,

id-CUtoDURRCInformation,

id-DRB-Activity-Item,

id-DRB-Activity-List,

id-DRBs-FailedToBeModified-Item,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-Item,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-Item,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-Item,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-Item,

id-DRBs-Modified-List,

id-DRB-Notify-Item,

id-DRB-Notify-List,

id-DRBs-Required-ToBeModified-Item,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-Item,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-Item,

id-DRBs-Setup-List,

id-DRBs-SetupMod-Item,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-Item,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-Item,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-Item,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-Item,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-ExecuteDuplication,

id-FullConfiguration,

id-gNB-CU-UE-F1AP-ID,

id-gNB-DU-UE-F1AP-ID,

id-gNB-DU-ID,

id-GNB-DU-Served-Cells-Item,

id-gNB-DU-Served-Cells-List,

id-gNB-CU-Name,

id-gNB-DU-Name,

id-InactivityMonitoringRequest,

id-InactivityMonitoringResponse,

id-new-gNB-CU-UE-F1AP-ID,

id-new-gNB-DU-UE-F1AP-ID,

id-oldgNB-DU-UE-F1AP-ID,

id-PLMNAssistanceInfoForNetShar,

id-Potential-SpCell-Item,

id-Potential-SpCell-List,

id-RAT-FrequencyPriorityInformation,

id-RedirectedRRCmessage,

id-ResetType,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-RRCReconfigurationCompleteIndicator,

id-SCell-FailedtoSetup-List,

id-SCell-FailedtoSetup-Item,

id-SCell-FailedtoSetupMod-List,

id-SCell-FailedtoSetupMod-Item,

id-SCell-ToBeRemoved-Item,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-Item,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-Item,

id-SCell-ToBeSetupMod-List,

id-SelectedPLMNID,

id-Served-Cells-To-Add-Item,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-Item,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-Item,

id-Served-Cells-To-Modify-List,

id-ServCellIndex,

id-ServingCellMO,

id-SpCell-ID,

id-SpCellULConfigured,

id-SRBID,

id-SRBs-FailedToBeSetup-Item,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-Item,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-Item,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-ToBeReleased-Item,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-Item,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-Item,

id-SRBs-ToBeSetupMod-List,

id-SRBs-Modified-Item,

id-SRBs-Modified-List,

id-SRBs-Setup-Item,

id-SRBs-Setup-List,

id-SRBs-SetupMod-Item,

id-SRBs-SetupMod-List,

id-TimeToWait,

id-TransactionID,

id-TransmissionActionIndicator,

id-UEContextNotRetrievable,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

id-DUtoCURRCContainer,

id-NRCGI,

id-PagingCell-Item,

id-PagingCell-List,

id-PagingDRX,

id-PagingPriority,

id-SItype-List,

id-UEIdentityIndexValue,

id-GNB-CU-TNL-Association-Setup-List,

id-GNB-CU-TNL-Association-Setup-Item,

id-GNB-CU-TNL-Association-Failed-To-Setup-List,

id-GNB-CU-TNL-Association-Failed-To-Setup-Item,

id-GNB-CU-TNL-Association-To-Add-Item,

id-GNB-CU-TNL-Association-To-Add-List,

id-GNB-CU-TNL-Association-To-Remove-Item,

id-GNB-CU-TNL-Association-To-Remove-List,

id-GNB-CU-TNL-Association-To-Update-Item,

id-GNB-CU-TNL-Association-To-Update-List,

id-MaskedIMEISV,

id-PagingIdentity,

id-Cells-to-be-Barred-List,

id-Cells-to-be-Barred-Item,

id-PWSSystemInformation,

id-RepetitionPeriod,

id-NumberofBroadcastRequest,

id-Cells-To-Be-Broadcast-List,

id-Cells-To-Be-Broadcast-Item,

id-Cells-Broadcast-Completed-List,

id-Cells-Broadcast-Completed-Item,

id-Broadcast-To-Be-Cancelled-List,

id-Broadcast-To-Be-Cancelled-Item,

id-Cells-Broadcast-Cancelled-List,

id-Cells-Broadcast-Cancelled-Item,

id-NR-CGI-List-For-Restart-List,

id-NR-CGI-List-For-Restart-Item,

id-PWS-Failed-NR-CGI-List,

id-PWS-Failed-NR-CGI-Item,

id-EUTRA-NR-CellResourceCoordinationReq-Container,

id-EUTRA-NR-CellResourceCoordinationReqAck-Container,

id-Protected-EUTRA-Resources-List,

id-RequestType,

id-ServingPLMN,

id-DRXConfigurationIndicator,

id-RLCFailureIndication,

id-UplinkTxDirectCurrentListInformation,

id-SULAccessIndication,

id-Protected-EUTRA-Resources-Item,

id-GNB-DUConfigurationQuery,

id-GNB-DU-UE-AMBR-UL,

id-GNB-CU-RRC-Version,

id-GNB-DU-RRC-Version,

id-GNBDUOverloadInformation,

id-NeedforGap,

id-RRCDeliveryStatusRequest,

id-RRCDeliveryStatus,

id-Dedicated-SIDelivery-NeededUE-List,

id-Dedicated-SIDelivery-NeededUE-Item,

id-ResourceCoordinationTransferInformation,

id-Associated-SCell-List,

id-Associated-SCell-Item,

id-IgnoreResourceCoordinationContainer,

id-UAC-Assistance-Info,

id-RANUEID,

id-PagingOrigin,

id-GNB-DU-TNL-Association-To-Remove-Item,

id-GNB-DU-TNL-Association-To-Remove-List,

id-TNLConfigurationInfo,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs

<<<<<<<<<<<<<<<<<<<< 6th Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- UE CONTEXT SETUP REQUEST

--

-- **************************************************************

UEContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupRequestIEs} },

...

}

UEContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional 	}|

{ ID id-SpCell-ID								CRITICALITY reject	TYPE NRCGI												PRESENCE mandatory	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE mandatory	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE mandatory}|

{ ID id-Candidate-SpCell-List					CRITICALITY ignore	TYPE Candidate-SpCell-List							PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-SCell-ToBeSetup-List					CRITICALITY ignore	TYPE SCell-ToBeSetup-List								PRESENCE optional	}|

{ ID id-SRBs-ToBeSetup-List						CRITICALITY reject	TYPE SRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-DRBs-ToBeSetup-List						CRITICALITY reject	TYPE DRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY ignore	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV										PRESENCE optional	}|

{ ID id-ServingPLMN								CRITICALITY ignore	TYPE PLMN-Identity										PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE conditional }|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest					PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-new-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional }|

{ ID id-RANUEID									CRITICALITY ignore	TYPE RANUEID											PRESENCE optional	}|

{ ID id-TNLConfigurationInfo					CRITICALITY ignore	TYPE TNLConfigurationInfo			   				PRESENCE optional	},

...

}

Candidate-SpCell-List::= SEQUENCE (SIZE(1..maxnoofCandidateSpCells)) OF ProtocolIE-SingleContainer { { Candidate-SpCell-ItemIEs} }

SCell-ToBeSetup-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetup-ItemIEs} }

SRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetup-ItemIEs} }

DRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetup-ItemIEs} }

Candidate-SpCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Candidate-SpCell-Item					CRITICALITY ignore	TYPE Candidate-SpCell-Item						PRESENCE mandatory	},

...

}

SCell-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetup-Item						CRITICALITY ignore	TYPE SCell-ToBeSetup-Item					PRESENCE mandatory	},

...

}

SRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetup-Item		CRITICALITY reject		TYPE SRBs-ToBeSetup-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetup-Item					CRITICALITY reject	TYPE DRBs-ToBeSetup-Item					PRESENCE mandatory},

...

}

<<<<<<<<<<<<<<<<<<<< 7th Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- UE CONTEXT SETUP RESPONSE

--

-- **************************************************************

UEContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupResponseIEs} },

...

}

UEContextSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID								PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID								PRESENCE mandatory	}|

{ ID id-DUtoCURRCInformation					CRITICALITY reject	TYPE DUtoCURRCInformation							PRESENCE mandatory }|

{ ID id-C-RNTI									CRITICALITY ignore	TYPE C-RNTI											PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer	PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration								PRESENCE optional	}|

{ ID id-DRBs-Setup-List							CRITICALITY ignore	TYPE DRBs-Setup-List								PRESENCE optional	}|

{ ID id-SRBs-FailedToBeSetup-List				CRITICALITY ignore	TYPE SRBs-FailedToBeSetup-List					PRESENCE optional	}|

{ ID id-DRBs-FailedToBeSetup-List				CRITICALITY ignore	TYPE DRBs-FailedToBeSetup-List					PRESENCE optional	}|

{ ID id-SCell-FailedtoSetup-List				CRITICALITY ignore	TYPE SCell-FailedtoSetup-List					PRESENCE optional	}|

{ ID id-InactivityMonitoringResponse			CRITICALITY reject	TYPE InactivityMonitoringResponse				PRESENCE optional	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	}|

{ ID id-SRBs-Setup-List							CRITICALITY ignore	TYPE SRBs-Setup-List								PRESENCE optional	}|

{ ID id-TNLConfigurationInfo					CRITICALITY ignore	TYPE TNLConfigurationInfo			   			PRESENCE optional	},

...

}

DRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-Setup-ItemIEs} }

SRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-FailedToBeSetup-ItemIEs} }

DRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-FailedToBeSetup-ItemIEs} }

SCell-FailedtoSetup-List ::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-FailedtoSetup-ItemIEs} }

SRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-Setup-ItemIEs} }

DRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Setup-Item						CRITICALITY ignore	TYPE DRBs-Setup-Item						PRESENCE mandatory},

...

}

SRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-Setup-Item						CRITICALITY ignore	TYPE SRBs-Setup-Item						PRESENCE mandatory},

...

}

SRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-FailedToBeSetup-Item		CRITICALITY ignore		TYPE SRBs-FailedToBeSetup-Item		PRESENCE mandatory},

...

}

DRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-FailedToBeSetup-Item		CRITICALITY ignore	TYPE DRBs-FailedToBeSetup-Item			PRESENCE mandatory},

...

}

SCell-FailedtoSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-FailedtoSetup-Item			CRITICALITY ignore	TYPE SCell-FailedtoSetup-Item			PRESENCE mandatory},

...

}

<<<<<<<<<<<<<<<<<<<< 8th Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Cell-Direction,

id-Cell-Type,

id-CellGroupConfig,

id-AvailablePLMNList,

id-PDUSessionID,

id-ULPDUSessionAggregateMaximumBitRate,

id-DC-Based-Duplication-Configured,

id-DC-Based-Duplication-Activation,

id-Duplication-Activation,

id-DLPDCPSNLength,

id-ULPDCPSNLength,

id-RLC-Status,

id-MeasurementTimingConfiguration,

id-DRB-Information,

id-QoSFlowMappingIndication,

id-ServingCellMO,

id-RLCMode,

id-ExtendedServedPLMNs-List,

id-ExtendedAvailablePLMN-List,

id-DRX-LongCycleStartOffset,

id-SelectedBandCombinationIndex,

id-SelectedFeatureSetEntryIndex,

id-Ph-InfoSCG,

id-latest-RRC-Version-Enhanced,

id-RequestedBandCombinationIndex,

id-RequestedFeatureSetEntryIndex,

id-RequestedP-MaxFR2,

id-DRX-Config,

id-UEAssistanceInformation,

id-PDCCH-BlindDetectionSCG,

id-Requested-PDCCH-BlindDetectionSCG,

id-BPLMN-ID-Info-List,

id-NotificationInformation,

id-TNLAssociationTransportLayerAddressgNBDU,

id-portNumber,

id-AdditionalSIBMessageList,

id-IgnorePRACHConfiguration,

id-CG-Config,

id-Ph-InfoMCG,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNRminus1,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

maxnoofAdditionalSIBs,

maxnoofUACPLMNs,

maxnoofUACperPLMN,

maxnoofExtTLAs

<<<<<<<<<<<<<<<<<<<< 9th Change >>>>>>>>>>>>>>>>>>>>

ExtendedServedPLMNs-List ::= SEQUENCE (SIZE(1.. maxnoofExtendedBPLMNs)) OF ExtendedServedPLMNs-Item

ExtendedServedPLMNs-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

tAISliceSupportList 		SliceSupportList	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ExtendedServedPLMNs-ItemExtIEs} } OPTIONAL,

...

}

ExtendedServedPLMNs-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ExtTLAs ::= SEQUENCE (SIZE(1..maxnoofExtTLAs)) OF ExtTLA-Item

ExtTLA-Item ::= SEQUENCE {

iPsecTLA					TransportLayerAddress		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExtTLA-Item-ExtIEs} } OPTIONAL,

...

}

ExtTLA-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< 10th Change >>>>>>>>>>>>>>>>>>>>

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TNLConfigurationInfo ::= SEQUENCE {

ExtendedTransportLayerAddresses			ExtTLAs										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TNLConfigurationInfo-ExtIEs} }	OPTIONAL,

...

}

TNLConfigurationInfo-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< 11th Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

id-F1Setup									ProcedureCode ::= 1

id-ErrorIndication							ProcedureCode ::= 2

id-gNBDUConfigurationUpdate					ProcedureCode ::= 3

id-gNBCUConfigurationUpdate					ProcedureCode ::= 4

id-UEContextSetup							ProcedureCode ::= 5

id-UEContextRelease							ProcedureCode ::= 6

id-UEContextModification					ProcedureCode ::= 7

id-UEContextModificationRequired			ProcedureCode ::= 8

id-UEMobilityCommand						ProcedureCode ::= 9

id-UEContextReleaseRequest					ProcedureCode ::= 10

id-InitialULRRCMessageTransfer				ProcedureCode ::= 11

id-DLRRCMessageTransfer						ProcedureCode ::= 12

id-ULRRCMessageTransfer						ProcedureCode ::= 13

id-privateMessage							ProcedureCode ::= 14

id-UEInactivityNotification					ProcedureCode ::= 15

id-GNBDUResourceCoordination				ProcedureCode ::= 16

id-SystemInformationDeliveryCommand			ProcedureCode ::= 17

id-Paging									ProcedureCode ::= 18

id-Notify									ProcedureCode ::= 19

id-WriteReplaceWarning						ProcedureCode ::= 20

id-PWSCancel								ProcedureCode ::= 21

id-PWSRestartIndication						ProcedureCode ::= 22

id-PWSFailureIndication						ProcedureCode ::= 23

id-GNBDUStatusIndication 					ProcedureCode ::= 24

id-RRCDeliveryReport		 				ProcedureCode ::= 25

id-F1Removal								ProcedureCode ::= 26

id-NetworkAccessRateReduction				ProcedureCode ::= 27

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

maxCellingNBDU							INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofNrCellBands						INTEGER ::= 32

maxnoofSIBTypes							INTEGER ::= 32

maxnoofSITypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxnoofBPLMNsNRminus1					INTEGER ::= 11

maxnoofUACPLMNs							INTEGER ::= 12

maxnoofUACperPLMN						INTEGER ::= 64

maxnoofAdditionalSIBs					INTEGER ::= 63

maxnoofExtTLAs							INTEGER ::= 16

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List				ProtocolIE-ID ::= 1

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 2

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 3

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 4

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 5

id-Cells-to-be-Deactivated-List-Item				ProtocolIE-ID ::= 6

id-CriticalityDiagnostics							ProtocolIE-ID ::= 7

id-CUtoDURRCInformation								ProtocolIE-ID ::= 9

id-DRBs-FailedToBeModified-Item						ProtocolIE-ID ::= 12

id-DRBs-FailedToBeModified-List						ProtocolIE-ID ::= 13

id-DRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 14

id-DRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 15

id-DRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 16

id-DRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 17

id-DRBs-ModifiedConf-Item							ProtocolIE-ID ::= 18

id-DRBs-ModifiedConf-List							ProtocolIE-ID ::= 19

id-DRBs-Modified-Item								ProtocolIE-ID ::= 20

id-DRBs-Modified-List								ProtocolIE-ID ::= 21

id-DRBs-Required-ToBeModified-Item					ProtocolIE-ID ::= 22

id-DRBs-Required-ToBeModified-List					ProtocolIE-ID ::= 23

id-DRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 24

id-DRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 25

id-DRBs-Setup-Item									ProtocolIE-ID ::= 26

id-DRBs-Setup-List									ProtocolIE-ID ::= 27

id-DRBs-SetupMod-Item								ProtocolIE-ID ::= 28

id-DRBs-SetupMod-List								ProtocolIE-ID ::= 29

id-DRBs-ToBeModified-Item							ProtocolIE-ID ::= 30

id-DRBs-ToBeModified-List							ProtocolIE-ID ::= 31

id-DRBs-ToBeReleased-Item							ProtocolIE-ID ::= 32

id-DRBs-ToBeReleased-List							ProtocolIE-ID ::= 33

id-DRBs-ToBeSetup-Item								ProtocolIE-ID ::= 34

id-DRBs-ToBeSetup-List								ProtocolIE-ID ::= 35

id-DRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 36

id-DRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 37

id-DRXCycle											ProtocolIE-ID ::= 38

id-DUtoCURRCInformation								ProtocolIE-ID ::= 39

id-gNB-CU-UE-F1AP-ID								ProtocolIE-ID ::= 40

id-gNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 41

id-gNB-DU-ID										ProtocolIE-ID ::= 42

id-GNB-DU-Served-Cells-Item							ProtocolIE-ID ::= 43

id-gNB-DU-Served-Cells-List							ProtocolIE-ID ::= 44

id-gNB-DU-Name										ProtocolIE-ID ::= 45

id-NRCellID											ProtocolIE-ID ::= 46

id-oldgNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 47

id-ResetType										ProtocolIE-ID ::= 48

id-ResourceCoordinationTransferContainer			ProtocolIE-ID ::= 49

id-RRCContainer										ProtocolIE-ID ::= 50

id-SCell-ToBeRemoved-Item							ProtocolIE-ID ::= 51

id-SCell-ToBeRemoved-List							ProtocolIE-ID ::= 52

id-SCell-ToBeSetup-Item								ProtocolIE-ID ::= 53

id-SCell-ToBeSetup-List								ProtocolIE-ID ::= 54

id-SCell-ToBeSetupMod-Item							ProtocolIE-ID ::= 55

id-SCell-ToBeSetupMod-List							ProtocolIE-ID ::= 56

id-Served-Cells-To-Add-Item							ProtocolIE-ID ::= 57

id-Served-Cells-To-Add-List							ProtocolIE-ID ::= 58

id-Served-Cells-To-Delete-Item						ProtocolIE-ID ::= 59

id-Served-Cells-To-Delete-List						ProtocolIE-ID ::= 60

id-Served-Cells-To-Modify-Item						ProtocolIE-ID ::= 61

id-Served-Cells-To-Modify-List						ProtocolIE-ID ::= 62

id-SpCell-ID										ProtocolIE-ID ::= 63

id-SRBID											ProtocolIE-ID ::= 64

id-SRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 65

id-SRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 66

id-SRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 67

id-SRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 68

id-SRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 69

id-SRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 70

id-SRBs-ToBeReleased-Item							ProtocolIE-ID ::= 71

id-SRBs-ToBeReleased-List							ProtocolIE-ID ::= 72

id-SRBs-ToBeSetup-Item								ProtocolIE-ID ::= 73

id-SRBs-ToBeSetup-List								ProtocolIE-ID ::= 74

id-SRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 75

id-SRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 76

id-TimeToWait										ProtocolIE-ID ::= 77

id-TransactionID									ProtocolIE-ID ::= 78

id-TransmissionActionIndicator						ProtocolIE-ID ::= 79

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 80

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 81

id-gNB-CU-Name										ProtocolIE-ID ::= 82

id-SCell-FailedtoSetup-List							ProtocolIE-ID ::= 83

id-SCell-FailedtoSetup-Item							ProtocolIE-ID ::= 84

id-SCell-FailedtoSetupMod-List						ProtocolIE-ID ::= 85

id-SCell-FailedtoSetupMod-Item						ProtocolIE-ID ::= 86

id-RRCReconfigurationCompleteIndicator 				ProtocolIE-ID ::= 87

id-Cells-Status-Item								ProtocolIE-ID ::= 88

id-Cells-Status-List								ProtocolIE-ID ::= 89

id-Candidate-SpCell-List							ProtocolIE-ID ::= 90

id-Candidate-SpCell-Item							ProtocolIE-ID ::= 91

id-Potential-SpCell-List							ProtocolIE-ID ::= 92

id-Potential-SpCell-Item							ProtocolIE-ID ::= 93

id-FullConfiguration								ProtocolIE-ID ::= 94

id-C-RNTI											ProtocolIE-ID ::= 95

id-SpCellULConfigured								ProtocolIE-ID ::= 96

id-InactivityMonitoringRequest						ProtocolIE-ID ::= 97

id-InactivityMonitoringResponse						ProtocolIE-ID ::= 98

id-DRB-Activity-Item								ProtocolIE-ID ::= 99

id-DRB-Activity-List								ProtocolIE-ID ::= 100

id-EUTRA-NR-CellResourceCoordinationReq-Container 		ProtocolIE-ID ::= 101

id-EUTRA-NR-CellResourceCoordinationReqAck-Container 	ProtocolIE-ID ::= 102

id-Protected-EUTRA-Resources-List					ProtocolIE-ID ::= 105

id-RequestType 										ProtocolIE-ID ::= 106

id-ServCellIndex									ProtocolIE-ID ::= 107

id-RAT-FrequencyPriorityInformation					ProtocolIE-ID ::= 108

id-ExecuteDuplication								ProtocolIE-ID ::= 109

id-NRCGI											ProtocolIE-ID ::= 111

id-PagingCell-Item									ProtocolIE-ID ::= 112

id-PagingCell-List									ProtocolIE-ID ::= 113

id-PagingDRX										ProtocolIE-ID ::= 114

id-PagingPriority 									ProtocolIE-ID ::= 115

id-SItype-List										ProtocolIE-ID ::= 116

id-UEIdentityIndexValue								ProtocolIE-ID ::= 117

id-gNB-CUSystemInformation							ProtocolIE-ID ::= 118

id-HandoverPreparationInformation					ProtocolIE-ID ::= 119

id-GNB-CU-TNL-Association-To-Add-Item				ProtocolIE-ID ::= 120

id-GNB-CU-TNL-Association-To-Add-List				ProtocolIE-ID ::= 121

id-GNB-CU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 122

id-GNB-CU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 123

id-GNB-CU-TNL-Association-To-Update-Item			ProtocolIE-ID ::= 124

id-GNB-CU-TNL-Association-To-Update-List			ProtocolIE-ID ::= 125

id-MaskedIMEISV										ProtocolIE-ID ::= 126

id-PagingIdentity									ProtocolIE-ID ::= 127

id-DUtoCURRCContainer								ProtocolIE-ID ::= 128

id-Cells-to-be-Barred-List							ProtocolIE-ID ::= 129

id-Cells-to-be-Barred-Item							ProtocolIE-ID ::= 130

id-TAISliceSupportList								ProtocolIE-ID ::= 131

id-GNB-CU-TNL-Association-Setup-List				ProtocolIE-ID ::= 132

id-GNB-CU-TNL-Association-Setup-Item				ProtocolIE-ID ::= 133

id-GNB-CU-TNL-Association-Failed-To-Setup-List		ProtocolIE-ID ::= 134

id-GNB-CU-TNL-Association-Failed-To-Setup-Item		ProtocolIE-ID ::= 135

id-DRB-Notify-Item									ProtocolIE-ID ::= 136

id-DRB-Notify-List									ProtocolIE-ID ::= 137

id-NotficationControl								ProtocolIE-ID ::= 138

id-RANAC											ProtocolIE-ID ::= 139

id-PWSSystemInformation								ProtocolIE-ID ::= 140

id-RepetitionPeriod									ProtocolIE-ID ::= 141

id-NumberofBroadcastRequest							ProtocolIE-ID ::= 142

id-Cells-To-Be-Broadcast-List						ProtocolIE-ID ::= 144

id-Cells-To-Be-Broadcast-Item						ProtocolIE-ID ::= 145

id-Cells-Broadcast-Completed-List 					ProtocolIE-ID ::= 146

id-Cells-Broadcast-Completed-Item 					ProtocolIE-ID ::= 147

id-Broadcast-To-Be-Cancelled-List 					ProtocolIE-ID ::= 148

id-Broadcast-To-Be-Cancelled-Item 					ProtocolIE-ID ::= 149

id-Cells-Broadcast-Cancelled-List 					ProtocolIE-ID ::= 150

id-Cells-Broadcast-Cancelled-Item 					ProtocolIE-ID ::= 151

id-NR-CGI-List-For-Restart-List 					ProtocolIE-ID ::= 152

id-NR-CGI-List-For-Restart-Item 					ProtocolIE-ID ::= 153

id-PWS-Failed-NR-CGI-List 							ProtocolIE-ID ::= 154

id-PWS-Failed-NR-CGI-Item 							ProtocolIE-ID ::= 155

id-ConfirmedUEID									ProtocolIE-ID ::= 156

id-Cancel-all-Warning-Messages-Indicator			ProtocolIE-ID ::= 157

id-GNB-DU-UE-AMBR-UL								ProtocolIE-ID ::= 158

id-DRXConfigurationIndicator						ProtocolIE-ID ::= 159

id-RLC-Status										ProtocolIE-ID ::= 160

id-DLPDCPSNLength									ProtocolIE-ID ::= 161

id-GNB-DUConfigurationQuery							ProtocolIE-ID ::= 162

id-MeasurementTimingConfiguration					ProtocolIE-ID ::= 163

id-DRB-Information									ProtocolIE-ID ::= 164

id-ServingPLMN										ProtocolIE-ID ::= 165

id-Protected-EUTRA-Resources-Item					ProtocolIE-ID ::= 168

id-GNB-CU-RRC-Version								ProtocolIE-ID ::= 170

id-GNB-DU-RRC-Version								ProtocolIE-ID ::= 171

id-GNBDUOverloadInformation							ProtocolIE-ID ::= 172

id-CellGroupConfig									ProtocolIE-ID ::= 173

id-RLCFailureIndication								ProtocolIE-ID ::= 174

id-UplinkTxDirectCurrentListInformation				ProtocolIE-ID ::= 175

id-DC-Based-Duplication-Configured					ProtocolIE-ID ::= 176

id-DC-Based-Duplication-Activation					ProtocolIE-ID ::= 177

id-SULAccessIndication								ProtocolIE-ID ::= 178

id-AvailablePLMNList								ProtocolIE-ID ::= 179

id-PDUSessionID										ProtocolIE-ID ::= 180

id-ULPDUSessionAggregateMaximumBitRate				ProtocolIE-ID ::= 181

id-ServingCellMO									ProtocolIE-ID ::= 182

id-QoSFlowMappingIndication							ProtocolIE-ID ::= 183

id-RRCDeliveryStatusRequest							ProtocolIE-ID ::= 184

id-RRCDeliveryStatus								ProtocolIE-ID ::= 185

id-BearerTypeChange									ProtocolIE-ID ::= 186

id-RLCMode											ProtocolIE-ID ::= 187

id-Duplication-Activation							ProtocolIE-ID ::= 188

id-Dedicated-SIDelivery-NeededUE-List				ProtocolIE-ID ::= 189

id-Dedicated-SIDelivery-NeededUE-Item				ProtocolIE-ID ::= 190

id-DRX-LongCycleStartOffset							ProtocolIE-ID ::= 191

id-ULPDCPSNLength									ProtocolIE-ID ::= 192

id-SelectedBandCombinationIndex						ProtocolIE-ID ::= 193

id-SelectedFeatureSetEntryIndex						ProtocolIE-ID ::= 194

id-ResourceCoordinationTransferInformation			ProtocolIE-ID ::= 195

id-ExtendedServedPLMNs-List							ProtocolIE-ID ::= 196

id-ExtendedAvailablePLMN-List						ProtocolIE-ID ::= 197

id-Associated-SCell-List							ProtocolIE-ID ::= 198

id-latest-RRC-Version-Enhanced						ProtocolIE-ID ::= 199

id-Associated-SCell-Item							ProtocolIE-ID ::= 200

id-Cell-Direction									ProtocolIE-ID ::= 201

id-SRBs-Setup-List									ProtocolIE-ID ::= 202

id-SRBs-Setup-Item									ProtocolIE-ID ::= 203

id-SRBs-SetupMod-List								ProtocolIE-ID ::= 204

id-SRBs-SetupMod-Item								ProtocolIE-ID ::= 205

id-SRBs-Modified-List								ProtocolIE-ID ::= 206

id-SRBs-Modified-Item								ProtocolIE-ID ::= 207

id-Ph-InfoSCG										ProtocolIE-ID ::= 208

id-RequestedBandCombinationIndex					ProtocolIE-ID ::= 209

id-RequestedFeatureSetEntryIndex					ProtocolIE-ID ::= 210

id-RequestedP-MaxFR2								ProtocolIE-ID ::= 211

id-DRX-Config										ProtocolIE-ID ::= 212

id-IgnoreResourceCoordinationContainer				ProtocolIE-ID ::= 213

id-UEAssistanceInformation							ProtocolIE-ID ::= 214

id-NeedforGap										ProtocolIE-ID ::= 215

id-PagingOrigin										ProtocolIE-ID ::= 216

id-new-gNB-CU-UE-F1AP-ID							ProtocolIE-ID ::= 217

id-RedirectedRRCmessage								ProtocolIE-ID ::= 218

id-new-gNB-DU-UE-F1AP-ID							ProtocolIE-ID ::= 219

id-NotificationInformation							ProtocolIE-ID ::= 220

id-PLMNAssistanceInfoForNetShar						ProtocolIE-ID ::= 221

id-UEContextNotRetrievable							ProtocolIE-ID ::= 222

id-BPLMN-ID-Info-List								ProtocolIE-ID ::= 223

id-SelectedPLMNID									ProtocolIE-ID ::= 224

id-UAC-Assistance-Info								ProtocolIE-ID ::= 225

id-RANUEID											ProtocolIE-ID ::= 226

id-GNB-DU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 227

id-GNB-DU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 228

id-TNLAssociationTransportLayerAddressgNBDU			ProtocolIE-ID ::= 229

id-portNumber										ProtocolIE-ID ::= 230

id-AdditionalSIBMessageList							ProtocolIE-ID ::= 231

id-Cell-Type										ProtocolIE-ID ::= 232

id-IgnorePRACHConfiguration							ProtocolIE-ID ::= 233

id-CG-Config										ProtocolIE-ID ::= 234

id-PDCCH-BlindDetectionSCG							ProtocolIE-ID ::= 235

id-Requested-PDCCH-BlindDetectionSCG					ProtocolIE-ID ::= 236

id-Ph-InfoMCG										ProtocolIE-ID ::= 237

id-TNLConfigurationInfo								ProtocolIE-ID ::= xxx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>