3GPP TSG-RAN3 Meeting #105bis	R3-195897

Chongqing, China, 14-18 October 2019

Title: 	Discussion on LPPa Reporting of NR Cells as Inter-RAT Measurements for E-CID

Source: 	Huawei

Agenda item:	9.3.4

Document for:	Discussion

# Introduction

During last RAN3#105, a proposal to specify LPPa enhancement to support E-CID for EN-DC [1] based on Inter-RAT Measurements for E-CID [2] was discussed without conclusion.

This contribution clarifies some observations from the previous discussion [1], provides a short summary of what is possible to achieve with the regards of LPPa on Radio Access Network and Radio Access measurements and then attempts a way forward for TEI-16.

# Background

Last meeting the following observations and proposal were establish [1]:

- Observation 1: EN-DC is out of scope in current NR positioning support WID

- Observation 2: Before introducing 5GC, there would be no way to achieve NR positioning

- Observation 3: As NR positioning accuracy would be higher than the one of LTE, there would be demand to introduce NR positioning (even if LTE positioning already works.)

- Observation 4: For E-CID, from both spec. and implementation pov, E-CID has very limited impact (i.e. extend to report NR cells as inter-RAT measurement)because (1)eNB already supports to measure neighbour NR cell quality by Inter-RAT measurement and (2) eNB already supports to terminate LPPa

- Proposal 1: RAN3 to specify LPPa enhancement to support E-CID for EN-DC

All thoughts the observations 2-3-4 are not totally correct because the rel-15 has some NR positioning methods available, the accuracy of position is pending to the method use and not the Radio Access and as we will see later in this contribution E-CID involving EN-DC might be really complex e.g. further involve RAN2…

However, RAN3 acknowledged, the observation 1 and propose to specify the LPPa enhancement to support E-CID for EN-DC in TEI-16.

# Clarification LPPa E-CID

Today LPPa E-CID is base on multiple types of measurement request [3]:

- Measurement Quantities (same network cells)

- Inter-RAT Measurement Quantities (other Networks)

- WLAN Measurement Quantities (other systems)

This is reflect in the E-CID MEASUREMENT INITIATION REQUEST message:

Direction: E-SMLC  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED(OnDemand, Periodic,…) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED(120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) |  | YES | reject |
| Measurement Quantities |  | 1 .. <maxnoMeas> |  |  | EACH | reject |
| >Measurement Quantities Item | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Timing Advance Type 1, Timing Advance Type 2, RSRP, RSRQ,…) |  | - | - |
| Inter-RAT Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >Inter-RAT Measurement Quantities Item | M |  | ENUMERATED(GERAN, UTRAN,…) |  |  |  |
| WLAN Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >WLAN Measurement Quantities Item | M |  | ENUMERATED (WLAN, ...) |  | - |  |

This message is sent by eNB to indicate that the requested E-CID measurement is successfully initiated.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| eNB UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| E-CID Measurement Result | O |  | 9.2.5 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |
| Inter-RAT Measurement Result | O |  | 9.2.13 |  | YES | ignore |
| WLAN Measurement Result | O |  | 9.2.15 |  | YES | ignore |

The purpose of the Inter-RAT Measurement Result information element is to provide the Inter-RAT measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Other-RAT Measured Results |  | 1.. <maxnoMeas> |  |  |
| >CHOICE Other-RAT Measured Results Value | M |  |  |  |
| >>Result GERAN | M | 1..<maxGERANMeas> |  |  |
| >>>ARFCN of BCCH | M |  | INTEGER (0..1023, ...) |  |
| >>>Physical CellId GERAN | M |  | INTEGER (0..63, ...) |  |
| >>>RSSI | M |  | INTEGER (0..63, ...) |  |
| >>Result UTRAN |  | 1..<maxUTRANMeas> |  |  |
| >>>UARFCN | M |  | INTEGER (0..16383, ...) |  |
| >>>CHOICE Physical CellId UTRA | M |  |  |  |
| >>>>Physical CellId UTRA FDD | M |  | INTEGER (0..511, ...) |  |
| >>>>Physical CellId UTRA TDD | M |  | INTEGER (0..127, ...) |  |
| >>>UTRA RSCP | O |  | INTEGER (-5..91, ...) |  |
| >>>UTRA EcNo | O |  | INTEGER (0..49, ...) | This IE applies to FDD only. |

In particular, we would like to highlight, sub clause 8.2.1.2 that:

If the Report Characteristics IE is set to "OnDemand", the eNB shall return the result of the measurement in the E-CID MEASUREMENT INITIATION RESPONSE message including, if available, the E-UTRAN Access Point Position IE in the E-CID Measurement Result IE, and the E-SMLC shall consider that the E-CID measurements for the UE has been terminated by the eNB. If available, the eNB shall include the Cell Portion ID IE in the E-CID MEASUREMENT INITIATION RESPONSE message. Upon reception of the Cell Portion ID IE, the E-SMLC may use the value as the cell portion for the measurement. If the Report Characteristics IE is set to "OnDemand" and the Inter-RAT Measurement Quantities IE is included in the E-CID MEASUREMENT INITIATION REQUEST message, the eNB shall, if supported, provide the corresponding measurements, if available in the eNB, in the Inter-RAT Measurement Result IE in E-CID MEASUREMENT INITIATION RESPONSE message. If the Report Characteristics IE is set to "OnDemand" and the WLAN Measurement Quantities IE is included in the E-CID MEASUREMENT INITIATION REQUEST message, the eNB shall, if supported, provide the corresponding measurements, if available in the eNB, in the WLAN Measurement Result IE in E-CID MEASUREMENT INITIATION RESPONSE message.

At this stage we would like to clarify from our understanding that the Inter-RAT Measurement Result IE is used as response of Inter-RAT Measurement Quantities IE which correspond to other Radio Access Network e.g. UTRAN or GERAN. The Inter-RAT Measurement Result IE is use for the RAN radio measurement and not to other Radio Access in same RAN e.g. measurement of NR from en-gNB in E-UTRAN.

The cells measurements corresponding to E-UTRAN (Radio Access Network) where the LPPa operates are report into the E-CID Measurement Result IE:

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | ECGI 9.2.6 | E-UTRAN Cell Identifier of the serving cell |
| Serving Cell TAC | M |  | OCTET STRING(2) | Tracking Area Code of the serving cell |
| E-UTRAN Access Point Position | O |  | 9.2.8 | The configured estimated geographical position of the antenna of the cell. |
| Measured Results |  | 0 .. <maxnoMeas> |  |  |
| >CHOICE Measured Results Value | M |  |  |  |
| >>Value Angle of Arrival | M |  | INTEGER (0..719) | According to mapping in TS 36.133 [8] |
| >>Value Timing Advance Type 1 | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [8] |
| >>Value Timing Advance Type 2 | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [8] |
| >>Result RSRP |  | 1 .. <maxCellReport> |  |  |
| >>>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the reported cell |
| >>>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>>ECGI | O |  | ECGI 9.2.6 | E-UTRAN Cell Global Identifier of the reported cell |
| >>>Value RSRP | M |  | INTEGER(0..97, …) |  |
| >>Result RSRQ |  | 1 . <maxCellReport> |  |  |
| >>>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the reported cell |
| >>>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>>ECGI | O |  | ECGI 9.2.6 | E-UTRAN Cell Global Identifier of the reported cell |
| >>>Value RSRQ | M |  | INTEGER(0..34, …) |  |

By principle the NNR cells of en-gNB as part of the E-UTRAN and should be reported in the E-CID Measurement Result IE.

# Full picture on NR cell measurement reports for E-CID (LPPa)

This section gives an overview of the NR cell measurements that a UE might report to eNB for E-CID.

Note: This contribution does not discuss the requirements and capability of the E-SMLC to have all information related to gNB (NG-RAN) and en-gNB (E-UTRAN) positions in order to determine a location based on NR cell measurement reports …

The UE can report NR cell (radio) measurement to the eNB from a gNB (NG-RAN) or an en-gNB (E-UTRAN). In the absolute, E-CID might use the measurements report from both to provide better accuracy. In this case the gNB cell measurements should be “Inter-RAT Measurement Result” and the en-gNB cell measurements should be “E-CID Measurement Result”.

The introduction of the NR, gNB cell Inter-RAT Measurement Result in LPPa is the pending CR of [2] introduction of E-UTRA, eNB cell Inter-RAT Measurement Result in NRPPa. This can be consider separately, independently of EN-DC E-CID aspects (see proposal in R3-195899 [4]) .

To be complete, we would like also mention a 3rd possible NR measurement report, might be available in case of EN-DC, which could be performed by the en-gNB e.g. in case of SNB relocation. Those measurements are transfer to from the SNB to the UE via a transparent container and it is currently not visible to the eNB. This could also improve the positioning … From our understanding, RAN2 had recently restricted the configuration of these measurements into the en-gNB.

In EN-DC the reporting of en-gNB (SNB) cell measurements configured in the en-gNB should probably affect RAN2 and X2, we suggest to not considering this 3rd possible NR measurement report for E-CID.

We propose then to focus for EN-DC as requested on the introduction the en-gNB NR cell measurements report configure by the eNB in E-CID Measurement Result IE for TEI-16.

# Way forward and conclusion

The enhancement of E-CID position of LPPa could be perform by measurement of gNB cell (from NG-RAN Standalone) and en-gNB cells (from E-UTRAN EN-DC) configured in the eNB.

It seems difficult to consider the possibility to include during en-gNB cell measurement (from E-UTRAN) configured in the en-gNB during TEI-16, due to the possible impact on RAN2.

As requested in [1] we propose then to focus on EN-DC by the introduction the en-gNB NR cell measurements report to the eNB via the E-CID Measurement Result IE as TEI-16. The Draft CR R3-195898 propose a version which deservers further checking, from our understanding the NR-PRSP is still under discussion in RAN1 and some IE deserve further stage 3 checking, without compromise the principle of the proposal.

.

# Reference

[1] R3-193381, LPPa enhancement for EN-DC support, NTT DOCOMO, INC.,Vodafone, Deutsche Telekom

[2] R3-182768, Inter-RAT Measurements for E-CID, Ericsson

[3] TS 36.455, LTE Positioning Protocol A (LPPa)

[4] R3-195899, Introduction of Inter-RAT Measurements for E-CID, Huawei