3GPP TSG-RAN WG3 #105bis	R3-195922

Chongqing, China, 14-18 October 2019

Agenda Item:	30.2

Source:	Ericsson

Title:	Support for NavIC Navigation Satellite System for LTE

Document for:	Discussion and Approval

# 1	Introduction

A new WI on Support for NavIC Navigation Satellite System for LTE was approved at RAN#85 (RP-192350).

The objective of this work item is to include support for NavIC L5 band to LTE specifications by defining a new GNSS ID and introducing some necessary assistance data information (e.g. Clock Model for NavIC, Navigation Model for NavIC, and Almanac for NavIC) based on the existing GNSS framework.

Target date is RAN#87 (March 2020)

More information about NavIC can be found in R2-1908706 (and R2-1908707 draft 36.355 LPP CR).

# 2	Discussion

The approved WID does not indicate any update to RRC (only LPP), but the current frame work also allow assistance data for a UE to be signaled via LPPa and RRC.

The impact on LPPa would at least be to add a new code point for NavIC to the GNSS ID IE in the Assistance Information Meta Data IE in the Assistance Information IE that can be included in the ASSISTANCE INFORMATION CONTROL message from the E-SMLC to the eNB.

It is currently unknown if RAN2 see a need to update any positioning SIB’s or add new positioning SIB’s. Assuming no new positioning SIB’s are need then the only impact to LPPa will then be to add a code point for NavIC.

# 3	Conclusions and Proposals

Proposal 1: It is proposed to agree the 36.455 CR in R3-195923.