3GPP TSG-RAN3 Meeting #105bis	R3-195948

Chongqing, China, 14th October – 18th October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0225 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LTE-M Indication for 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5 |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | LTE-M Indication needs to be sent to CN before the UE context and the user plane is set up. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add LTE-M Indication in INITIAL UE MESSAGE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LTE-M Indication arrives to CN too late |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: rebaseling with latest version of the TS |  |  |  |  |  |  |  |  |

### 8.6.1	Initial UE Message

#### 8.6.1.1	General

The Initial UE Message procedure is used when the NG-RAN node has received from the radio interface the first uplink NAS message transmitted on an RRC connection to be forwarded to an AMF.

#### 8.6.1.2	Successful Operation

Figure 8.6.1.2-1: Initial UE message

The NG-RAN node initiates the procedure by sending an INITIAL UE MESSAGE message to the AMF. The NG-RAN node shall allocate a unique RAN UE NGAP ID to be used for the UE and the NG-RAN node shall include this identity in the INITIAL UE MESSAGE message.

The NAS-PDU IE contains a UE – AMF message that is transferred without interpretation in the NG-RAN node.

In case of network sharing, the selected PLMN is indicated by the PLMN Identity IE within the TAI IE included in the INITIAL UE MESSAGE message.

When the NG-RAN node has received from the radio interface the 5G-S-TMSI IE, it shall include it in the INITIAL UE MESSAGE message.

If the AMF Set ID IE is included in the INITIAL UE MESSAGE message this indicates that the message is a rerouted message and the AMF shall, if supported, use the IE as described in TS 23.502 [10].

If the UE Context Request IE is included in the INITIAL UE MESSAGE message the AMF shall trigger an Initial Context Setup procedure towards the NG-RAN node.

If the Allowed NSSAI IE is included in the INITIAL UE MESSAGE message the AMF shall use the IE as defined in TS 23.502 [10].

If the LTE-M indication IE is included in the INITIAL UE MESSAGE message the AMF shall, if supported, use it according to TS 23.501 [10].

#### 8.6.1.3	Abnormal Conditions

If the 5G-S-TMSI is not received by the AMF in the INITIAL UE MESSAGE message whereas expected, the AMF shall consider the procedure as failed.

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.5.1	INITIAL UE MESSAGE

This message is sent by the NG-RAN node to transfer the initial layer 3 message to the AMF over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | reject |
| RRC Establishment Cause | M |  | 9.3.1.111 |  | YES | ignore |
| 5G-S-TMSI | O |  | 9.3.3.20 |  | YES | reject |
| AMF Set ID | O |  | 9.3.3.12 |  | YES | ignore |
| UE Context Request | O |  | ENUMERATED (requested, ...) | Indicates that a UE context including security information needs to be setup at the NG-RAN. | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 |  | YES | reject |
| LTE-M Indication | O |  | 9.3.1.xx |  | YES | ignore |

#### 9.3.1.xx	LTE-M Indication

This element is provided by the NG-RAN node to inform that the UE indicates category M1 or M2 in its UE Radio Capability.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTE-M Indication | M |  | ENUMERATED (LTE-M, ...) |  |

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-TNLAssociationSetupList,

AMF-TNLAssociationToAddList,

AMF-TNLAssociationToRemoveList,

AMF-TNLAssociationToUpdateList,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

CancelAllWarningMessages,

Cause,

CellIDListForRestart,

CNAssistedRANTuning,

ConcurrentWarningMessageInd,

CoreNetworkAssistanceInformationForInactive,

CPTransportLayerInformation,

CriticalityDiagnostics,

DataCodingScheme,

DirectForwardingPathAvailability,

EmergencyAreaIDListForRestart,

EmergencyFallbackIndicator,

EN-DCSONConfigurationTransfer,

EUTRA-CGI,

FiveG-S-TMSI,

GlobalRANNodeID,

GUAMI,

HandoverFlag,

HandoverType,

IMSVoiceSupportIndicator,

IndexToRFSP,

InfoOnRecommendedCellsAndRANNodesForPaging,

LocationReportingRequestType,

LTE-M-Indication,

MaskedIMEISV,

MessageIdentifier,

MobilityRestrictionList,

NAS-PDU,

NASSecurityParametersFromNGRAN,

NewSecurityContextInd,

NGRAN-CGI,

NGRANTraceID,

------------------------------------------

Skip to next change

------------------------------------------

id-AMFSetID,

id-AMF-TNLAssociationFailedToSetupList,

id-AMF-TNLAssociationSetupList,

id-AMF-TNLAssociationToAddList,

id-AMF-TNLAssociationToRemoveList,

id-AMF-TNLAssociationToUpdateList,

id-AMFTrafficLoadReductionIndication,

id-AMF-UE-NGAP-ID,

id-AssistanceDataForPaging,

id-BroadcastCancelledAreaList,

id-BroadcastCompletedAreaList,

id-CancelAllWarningMessages,

id-Cause,

id-CellIDListForRestart,

id-CNAssistedRANTuning,

id-ConcurrentWarningMessageInd,

id-CoreNetworkAssistanceInformationForInactive,

id-CriticalityDiagnostics,

id-DataCodingScheme,

id-DefaultPagingDRX,

id-DirectForwardingPathAvailability,

id-EmergencyAreaIDListForRestart,

id-EmergencyFallbackIndicator,

id-ENDC-SONConfigurationTransferDL,

id-ENDC-SONConfigurationTransferUL,

id-EUTRA-CGI,

id-FiveG-S-TMSI,

id-GlobalRANNodeID,

id-GUAMI,

id-HandoverFlag,

id-HandoverType,

id-IMSVoiceSupportIndicator,

id-IndexToRFSP,

id-InfoOnRecommendedCellsAndRANNodesForPaging,

id-LocationReportingRequestType,

id-LTE-M-Indication,

id-MaskedIMEISV,

id-MessageIdentifier,

id-MobilityRestrictionList,

id-NAS-PDU,

------------------------------------------

Skip to next change

------------------------------------------

-- **************************************************************

--

#### -- NAS TRANSPORT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- INITIAL UE MESSAGE

--

-- **************************************************************

InitialUEMessage ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InitialUEMessage-IEs} },

...

}

InitialUEMessage-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-NAS-PDU						CRITICALITY reject	TYPE NAS-PDU					PRESENCE mandatory	}|

{ ID id-UserLocationInformation		CRITICALITY reject	TYPE UserLocationInformation	PRESENCE mandatory	}|

{ ID id-RRCEstablishmentCause		CRITICALITY ignore	TYPE RRCEstablishmentCause		PRESENCE mandatory	}|

{ ID id-FiveG-S-TMSI				CRITICALITY reject	TYPE FiveG-S-TMSI				PRESENCE optional	}|

{ ID id-AMFSetID					CRITICALITY ignore	TYPE AMFSetID					PRESENCE optional	}|

{ ID id-UEContextRequest			CRITICALITY ignore	TYPE UEContextRequest			PRESENCE optional	}|

{ ID id-AllowedNSSAI				CRITICALITY reject	TYPE AllowedNSSAI				PRESENCE optional	}|

{ ID id-LTE-M-Indication			CRITICALITY ignore	TYPE LTE-M-Indication			PRESENCE optional},

...

}

------------------------------------------

Skip to next change

------------------------------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-Cause,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-DataForwardingNotPossible,

id-DL-NGU-UP-TNLInformation,

id-LastEUTRAN-PLMNIdentity,

id-MaximumIntegrityProtectedDataRate-DL,

id-NetworkInstance,

id-OldAssociatedQosFlowList-ULendmarkerexpected,

id-PDUSessionAggregateMaximumBitRate,

id-PDUSessionResourceFailedToSetupListCxtFail,

id-PDUSessionResourceReleaseResponseTransfer,

id-PDUSessionType,

id-PSCellInformation,

id-QosFlowAddOrModifyRequestList,

id-QosFlowSetupRequestList,

id-QosFlowToReleaseList,

id-SecondaryRATUsageInformation,

id-SecurityIndication,

id-SecurityResult,

id-S-NSSAI,

id-UL-NGU-UP-TNLInformation,

id-UL-NGU-UP-TNLModifyList,

id-ULForwarding,

id-ULForwardingUP-TNLInformation,

maxnoofAllowedAreas,

maxnoofAllowedS-NSSAIs,

maxnoofBPLMNs,

maxnoofCellIDforWarning,

-------------------------

Skip to the next change

-------------------------

#### -- L

LastVisitedCellInformation ::= CHOICE {

nGRANCell		LastVisitedNGRANCellInformation,

eUTRANCell		LastVisitedEUTRANCellInformation,

uTRANCell		LastVisitedUTRANCellInformation,

gERANCell		LastVisitedGERANCellInformation,

choice-Extensions		ProtocolIE-SingleContainer { {LastVisitedCellInformation-ExtIEs} }

}

LastVisitedCellInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

LastVisitedCellItem ::= SEQUENCE {

lastVisitedCellInformation		LastVisitedCellInformation,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedCellItem-ExtIEs} }	OPTIONAL,

...

}

LastVisitedCellItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedEUTRANCellInformation ::= OCTET STRING

LastVisitedGERANCellInformation ::= OCTET STRING

LastVisitedNGRANCellInformation::= SEQUENCE {

globalCellID								NGRAN-CGI,

cellType									CellType,

timeUEStayedInCell							TimeUEStayedInCell,

timeUEStayedInCellEnhancedGranularity		TimeUEStayedInCellEnhancedGranularity 			OPTIONAL,

hOCauseValue								Cause		 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedNGRANCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedNGRANCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedUTRANCellInformation ::= OCTET STRING

LocationReportingReferenceID ::= INTEGER (1..64, ...)

LocationReportingRequestType ::= SEQUENCE {

eventType										EventType,

reportArea										ReportArea,

areaOfInterestList								AreaOfInterestList								OPTIONAL,

locationReportingReferenceIDToBeCancelled		LocationReportingReferenceID					OPTIONAL,

-- The above IE shall be present if the event type is set to “stop reporting UE presence in the area of interest”

iE-Extensions		ProtocolExtensionContainer { {LocationReportingRequestType-ExtIEs} }		OPTIONAL,

...

}

LocationReportingRequestType-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LTE-M-Indication ::= ENUMERATED {lte-m, ... }

-------------------------

Skip to the next change

-------------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

-------------------------

Skip to the next change

-------------------------

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofAllowedAreas					INTEGER ::= 16

maxnoofAllowedS-NSSAIs				INTEGER ::= 8

maxnoofAlternativeQoSProfiles		INTEGER ::= 8

maxnoofBPLMNs						INTEGER ::= 12

maxnoofCellIDforWarning				INTEGER ::= 65535

maxnoofCellinAoI					INTEGER ::= 256

maxnoofCellinEAI					INTEGER ::= 65535

maxnoofCellinTAI					INTEGER ::= 65535

maxnoofCellsingNB					INTEGER ::= 16384

maxnoofCellsinngeNB					INTEGER ::= 256

maxnoofCellsinUEHistoryInfo			INTEGER ::= 16

-------------------------

Skip to the next change

-------------------------

id-SecurityResult										ProtocolIE-ID ::= 156

id-ENDC-SONConfigurationTransferDL						ProtocolIE-ID ::= 157

id-ENDC-SONConfigurationTransferUL						ProtocolIE-ID ::= 158

id-OldAssociatedQosFlowList-ULendmarkerexpected			ProtocolIE-ID ::= 159

id-CNTypeRestrictionsForEquivalent						ProtocolIE-ID ::= 160

id-CNTypeRestrictionsForServing							ProtocolIE-ID ::= 161

id-NewGUAMI												ProtocolIE-ID ::= 162

id-ULForwarding											ProtocolIE-ID ::= 163

id-ULForwardingUP-TNLInformation						ProtocolIE-ID ::= 164

id-CNAssistedRANTuning									ProtocolIE-ID ::= 165

id-CommonNetworkInstance								ProtocolIE-ID ::= 166

id-LTE-M-Indication										ProtocolIE-ID ::= 1xx

END

-- ASN1STOP