3GPP TSG-RAN WG3 Meeting #105bis	R3-195980

Chongqing, China, 14 – 18 October, 2019

Agenda item:	10.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	[TP for BL CR for TS 38.473] Signalling of overload information per slice

Document for:	Text Proposal

# 1	Introduction

This TP enhances overload information for F1 per slice as proposed in [1]. The TP is based on TS 38.473 and is proposed to be integrated in the F1AP BL CR [2].

# References

[1]	R3-195979, Load information on E1 and F1 interfaces, Nokia, Nokia Shanghai Bell

[2]	R3-194773, BL CR to TS 38.473

# Annex	- TP for BL CR for TS 38.473

Beginning of Text Proposal to TS 38.473

### 8.2.7	gNB-DU Status Indication

#### 8.2.7.1	General

The purpose of the gNB-DU Status Indication procedure is informing the gNB-CU that the gNB-DU is overloaded so that overload reduction actions can be applied. The procedure uses non-UE associated signalling.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: gNB-DU Status Indication procedure

If the gNB-DU Overload Information IE in the GNB-DU STATUS INDICATION message indicates that the gNB-DU is overloaded, the gNB-CU shall apply overload reduction actions until informed, with a new GNB-DU STATUS INDICATION message, that the overload situation has ceased.

If the Cell Overload Information List IE is included in the GNB-DU STATUS INDICATION message, it indicates whether a cell is overloaded or not overloaded.

The detailed overload reduction policy is up to gNB-CU implementation.

#### 8.2.7.3	Abnormal Conditions

Void.

Next Text Proposal to TS 38.473

#### 9.2.1.15	GNB-DU STATUS INDICATION

This message is sent by the gNB-DU to indicate to the gNB-CU its status of overload.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-DU Overload Information | M |  | ENUMERATED (overloaded, not-overloaded) |  | YES | reject |
| Cell Overload Information List |  | 0..1 |  |  | YES | ignore |
| >Cell Overload Information Item |  | 1 .. <maxCellingNB-DU> |  |  | EACH | ignore |
| >>NR Cell ID | M |  | NCGI 9.3.1.12 |  | – | – |
| >>Slice To Report |  | 1 |  |  | YES | ignore |
| >>>Slice To Report Item |  | 1 .. < maxnoofSliceItems> |  |  | EACH | ignore |
| >>>>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  |  |  |
| >>>>Slice Overload Information | M |  | ENUMERATED (overloaded, not-overloaded) |  | – | – |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by an gNB-DU. Value is 512. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

End of Text Proposal to TS 38.473