3GPP TSG-RAN WG3 Meeting #105bis	R3-195982

Chongqing, China, 14 – 18 October, 2019

Agenda item:	10.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	[TP for BL CR for TS 38.463] Addition of load information on E1

Document for:	Text Proposal

# 1	Introduction

This TP introduces load information for E1 as proposed in [1]. The TP is based on [2].

# References

[1]	R3-195979, Load information on E1 and F1 interfaces, Nokia, Nokia Shanghai Bell

[2]	R3-194772, BL CR to TS 38.463

# Annex	- TP for BL CR for TS 38.463

Beginning of Text Proposal to TS 38.463

#### 9.2.1.X1	RESOURCE STATUS REQUEST

Editor’s note: The content of this section is FFS

This message is sent by an gNB-CU-CP to gNB-CU-UP to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStoporRemoveorAdd |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | Ignore |
| Registration Request | M |  | ENUMERATED(start, stop, partial stop, add, …,) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-CU-UP is requested to report. First Bit = HW Load, Second Bit = TNL Load. Other bits shall be ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity | O |  | ENUMERATED (1000ms, 2000ms, 5000ms, 10000ms, 20000ms, 30000ms, 40000ms, 50000ms, 60000ms, 70000ms, 80000ms, 90000ms, 100000ms, 110000ms, 12000ms, …) | Periodicity that can be used for reporting. | YES | ignore |
| Partial Success Indicator | O |  | ENUMERATED(partial success allowed, ...) | Included if partial success is allowed | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporPartialStoporAdd | This IE shall be present if the Registration Request IE is set to the value “stop”, “partial stop” or “add”. |

#### 9.2.1.X2	RESOURCE STATUS RESPONSE

Editor’s note: The content of this section is FFS

This message is sent by the gNB-CU-UP to indicate that the requested measurement, for all or for a subset of the measurement objects included in the measurement is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | Ignore |
| Measurement Initiation Result |  | 0..1 |  | List of all slices in which measurement objects were requested, included when indicating partial success. | YES | ignore |
| >Measurement Initiation Result Item |  | 1 .. < maxnoofSliceItems> |  |  | EACH | ignore |
| >>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  |  |  |
| >>Measurement Failure Cause List |  | 0..1 |  | Indicates that gNB-CU-UP could not initiate the measurement for at least one of the requested measurement objects | – | – |
| >>>Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>>>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object that failed to be initiated in the gNB-CU-UP. First Bit = HW Load, Second Bit = TNL Load. Other bits shall be ignored by the gNB-CU-CP. | – | – |
| >>>>Cause | M |  | 9.3.1.2 | Failure cause for measurement objects for which the measurement cannot be initiated | – | – |

| Range bound | Explanation |
| --- | --- |
| maxFailedMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 32. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

#### 9.2.1.X3	RESOURCE STATUS FAILURE

Editor’s note: The content of this section is FFS

This message is sent by the gNB-CU-UP to indicate that for none of the requested measurement objects the measurement can be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStoporRemoveorAdd |  |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Cause | M |  |  | 9.3.1.2 | Ignored by the receiver when the Complete Failure Cause Information IE is included | YES | ignore |
| Criticality Diagnostics |  | O |  | 9.3.1.3 |  | YES | Ignore |
| Complete Failure Cause Information |  |  | 0..1 |  | Complete list of failure causes for all requested slices | YES | ignore |
| >Complete Failure Cause Information Item |  |  | 1 .. < maxnoofSliceItems> |  |  | EACH | ignore |
| >>S-NSSAI |  | M |  | S-NSSAI 9.3.1.38 |  |  |  |
| >>>Measurement Failure Cause List |  |  | 1 |  |  | – | – |
| >>>>Measurement Failure Cause Item |  |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>>>>Measurement Failed Report Characteristics |  | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object that failed to be initiated in the gNB-DU. First Bit: HW Load, Second Bit = TNL Load. Other bits shall be ignored by the gNB-CU. | – | – |
| >>>>>Cause |  | M |  | 9.2.6 | Failure cause for measurements that cannot be initiated | – | – |

| Range bound | Explanation |
| --- | --- |
| maxFailedMeasObjects | Max number of measurement objects that can fail per measurement. Value is 32. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

#### 9.2.1.X4	RESOURCE STATUS UPDATE

Editor’s note: The content of this section is FFS

This message is sent by gNB-CU-UP to gNB-CU-CP to report the results of the requested measurements.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.53 |  | YES | Reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | reject |
| Slice Measurement Result List |  | 1 |  |  | YES | ignore |
| >Slice Measurement Result List Item |  | 1 .. < maxnoofSliceItems> |  |  | EACH | ignore |
| >>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  |  |  |
| >>HW Load per Slice | O |  | INTEGER(0..100, …) | Indicates the amount of HW resources utilized for the slice | YES | ignore |
| >>DL TNL Load per Slice | O |  | INTEGER(0..100, …) | Indicates the amount of downlink transport network level resources utilized for the slice | YES | ignore |
| >>UL TNL Load per Slice | O |  | INTEGER(0..100, …) | Indicates the amount of uplink transport network level resources utilized for the slice | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

Next Text Proposal to TS 38.463

#### 9.3.1.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the E1AP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, Unknown or already allocated gNB-CU-CP UE E1AP ID, Unknown or already allocated gNB-CU-UP UE E1AP ID, Unknown or inconsistent pair of UE E1AP ID, Interaction with other procedure, PDCP Count Wrap Around, Not supported QCI value, Not supported 5QI value, Encryption algorithms not supported, Integrity protection algorithms not supported, UP integrity protection not possible, UP confidentiality protection not possible, Multiple PDU Session ID Instances, Unknown PDU Session ID, Multiple QoS Flow ID Instances, Unknown QoS Flow ID, Multiple DRB ID Instances, Unknown DRB ID, Invalid QoS combination, Procedure cancelled, Normal release, No radio resources available, Action desirable for radio reasons, Resources not available for the slice, PDCP configuration not supported, …, UE DL maximum integrity protected data rate reason, UP integrity protection failure, Release due to Pre-Emption, ReportCharacteristicsEmpty, NoReportPeriodicity, ExistingMeasurementID, Unknown Measurement ID, Measurement Temporarily not Available) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified, Transport Resource Unavailable, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Not enough User Plane Processing Resources, Hardware Failure, O&M Intervention, Unspecified, …) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| Unknown or already allocated gNB-CU-CP UE E1AP ID | The action failed because the gNB-CU-CP UE E1AP ID is either unknown, or (for a first message received at the gNB-CU) is known and already allocated to an existing context. |
| Unknown or already allocated gNB-CU-UP UE E1AP ID | The action failed because the gNB-CU-UP UE E1AP ID is either unknown, or (for a first message received at the gNB-CU-UP) is known and already allocated to an existing context. |
| Unknown or inconsistent pair of UE E1AP ID | The action failed because both UE E1AP IDs are unknown, or are known but do not define a single UE context. |
| Interaction with other procedure | The action is due to an ongoing interaction with another procedure. |
| PDCP COUNT wrap around | PDCP COUNT approaches the maximum value. |
| Not supported QCI value | The action failed because the requested QCI is not supported. |
| Not supported 5QI value | The action failed because the requested 5QI is not supported. |
| Encryption algorithms not supported | The gNB-CU-UP is unable to support the selected encryption algorithm for the UE. |
| Integrity protection algorithms not supported | The gNB-CU-UP is unable to support the selected integrity protection algorithm for the UE. |
| UP integrity protection not possible | The PDU Session cannot be accepted according to the required user plane integrity protection policy. |
| UP confidentiality protection not possible | The PDU Session cannot be accepted according to the required user plane confidentiality protection policy |
| Multiple PDU Session ID Instances | The action failed because multiple instances of the same PDU Session had been provided. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown. |
| Multiple QoS Flow ID Instances | The action failed because multiple instances of the same QoS flow had been provided. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknow. |
| Multiple DRB ID Instances | The action failed because multiple instances of the same DRB had been provided. |
| Unknown DRB ID | The action failed because the DRB ID is unknow. |
| Invalid QoS combination | The action was failed because of invalid QoS combination |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. |
| Normal release | The action is due to a normal release of the UE (e.g. because of mobility) and does not indicate an error. |
| No radio resources available | The requested node doesn’t have sufficient radio resources available. |
| Action desirable for radio reasons | The reason for requesting the action is radio related. |
| Resources not available for the slice | The requested resources are not available for the slice. |
| PDCP configuration not supported, | The gNB-CU-UP is unable to support the selected PDCP configuration for the UE. |
| UE DL maximum integrity protected data rate reason | The request is not accepted in order to comply with the maximum downlink data rate for integrity protection supported by the UE. |
| UP integrity protection failure | The gNB-CU-UP detects an integrity protection failure in the UL PDU. |
| Release due to Pre-Emption | Release is initiated due to pre-emption. |
| ReportCharacteristicsEmpty | The action failed because there is no characteristic reported. |
| NoReportPeriodicity | The action failed because the periodicity is not defined. |
| ExistingMeasurementID | The action failed because measurement-ID is already used. |
| Unknown Measurement ID | The action failed because some Measurement-ID is unknown. |
| Measurement Temporarily not Available | The gNB can temporarily not provide the requested measurement object. |

| Transport Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |
| Transport Resource Unavailable | The required transport resources are not available. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated "reject". |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated "ignore and notify". |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | Control processing overload. |
| Not Enough User Plane Processing Resources Available | No enough resources are available related to user plane processing. |
| Hardware Failure | Action related to hardware failure. |
| O&M Intervention | The action is due to O&M intervention. |
| Unspecified Failure | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer, NAS or Protocol. |

End of Text Proposal to TS 38.463