3GPP TSG-RAN WG3 Meeting #105bis	R3-195987

Chongqing, China, 14 – 18 October, 2019

Agenda item:	8.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Answers to SA5 on OAM requirements for RIM

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#104, RAN3 sent a LS [1] to SA5 requesting OAM support for RIM. At RAN3#105, RAN3 received a reply LS [2] from SA5 with questions to RAN3. In this paper we propose replies to SA5’s questions.

# 2	Discussion

Question 1 and 2:

- In 3-split gNB architecture, does OAM send set IDs to gNB-CU-CP?

- In 3-split gNB architecture, does OAM send mapping between set ID and backhaul addresses of gNBs  to gNB-CU-CP?

Suggested reply: As described in CR to TS 38.300, the coordination communication in RIM frameworks can be wireless- or backhaul-based. For wireless-based coordination the communication is purely wireless, and for 2- and 3-split gNB architecture the gNB-CU (gNB-CU-CP) is therefore not involved. For backhaul based coordination the gNB-CU will be involved when an aggressor cell receives RIM-RS from the victim as follows (see CR to TS 38.401):

- The aggressor gNB-CU receives the F1AP DU-CU RADIO INFORMATION TRANSFER, containing the victim set id. The gNB-CU acts as a coordinator on behalf of its affiliated gNB-DUs, where the gNB-CU merges the outgoing RIM information received from its gNB-DUs in the aggressor set and forwards the merged information to all the gNBs in the victim set. The gNB-CU maps the victim set id onto a backhaul address the victim set and forwards the information via the core network. This backhaul address consists of the gNB ID + TAI of a gNB within the victim set. The aggressor gNB-CU therefore needs configured information about the aggressor set (aggressor set id and corresponding cell ids) for the purpose of intra-set coordination.  The aggressor gNB-CU also needs either configured information or a look-up functionality in order to retrieve the backhaul address (gNB ID + TAI) of the victim set.

- When the victim gNB-CU receives the message transmitted by the aggressor gNB-CU via the core network (containing the victim set id), it will forward the message to gNB-DUs controlling the cells of the victim set. The victim gNB-CU therefore needs configured information about the victim sets under its coverage (victim set id and corresponding cell ids).

Question 3:

- Does bullet 2 (OAM should be able to adjust the gNB grouping and the assigned set IDs based on the remote interference detection indication from victim gNBs and interfered set IDs reported from aggressor gNB) imply OAM system should have feature and intelligence to detect inference and adjust the gNB group?

Suggested reply: The initial set configuration is done by OAM. Any update or adjustment of the sets may be left to implementation.

# 3	Conclusion

We have suggested replies to SA5’s questions and submitted a corresponding draft reply LS to SA5 cc RAN1 in [3].

# References

[1]		R3-193245 - S5-195046, LS on OAM requirements for RIM, RAN3

[2]		S5-195662 - R3-194732, LS reply from 3GPP SA5 to RAN1 and RAN3 on OAM requirements for RIM, SA5

[3]	R3-195988, [DRAFT] Reply LS on clarification of OAM requirements for RIM, Nokia, Nokia Shanghai Bell