3GPP TSG-RAN WG3 #103	R3-196017

Chongqing , CN, 14th October – 18th October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| n |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Support for information on spatial load distribution |  |  |  |  |  |  |  |  |  |
| Source to WG: | Ericsson, NTT DoCoMo, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A description of load information that takes into account the spatial distribution of load has been deemed as necessary in RAN3. A stage 2 description of the load information per SSB area is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To describe how load information that take into account spatial distribution of load are exchanged |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to represent load in a spatial way and consequently it is not possible to make the load balancing function work correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------Start of the change---------------------------------------------------

## 15.X	Self-optimisation

### 15.X.1	Support for Mobility Load Balancing

Editor’s note: This section captures the stage 2 descriptions for Mobility Load Balancing

#### 15.X.1.1	General

Editor’s note: the content of this section is FFS.

The objective of mobility load balancing is to distribute cell load evenly among cells and across areas of cells covered by different SSBs or to transfer part of the traffic from congested cell or from congested areas of cells covered by different SSBs, or to offload users from one cell or carrier or RAT or areas of cells covered by different SSBs to achieve network energy saving, This can be done by means of optimization of cell reselection/handover parameters and handover actions. The automation of such optimisation can provide high quality user experience, while simultaneously improving the system capacity and also to minimize human intervention in the network management and optimization tasks.

Both intra-RAT and intra-system inter-RAT load balancing scenarios should be supported.

In general, support for mobility load balancing consists of one or more of following functions:

-	 Load reporting;

- Load balancing action based on handovers;

-	 Adapting handover and/or reselection configuration.

#### 15.X.1.2 Load reporting

Editor’s note: the content of this section is FFS.

The load reporting function is executed by exchanging load information over the Xn/X2/F1/E1 interfaces.

Editor’s note: load related information and its definition are FFS.

The following load related information should be supported which consists of,

-	Radio resource usage (per cell and per SSB area: DL/UL/SUL GBR PRB usage, DL/UL/SUL non-GBR PRB usage, DL/UL/SUL total PRB usage);

-	TNL load indicator (UL/DL /SUL TNL available capacity);

-	Cell Capacity Class value (per cell and per SSB area UL/DL relative capacity indicator);

- Capacity value (per cell and per SSB area UL/DL available capacity for load balancing as percentage of total cell capacity).

To achieve load reporting function, Resource Status Reporting Initiation & Resource Status Reporting procedures are used.

--------------------------------------------------End of the change-----------------------------------------------------